| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 Jan 2020 | Document: JVET-Q0229_v2 |
| --- | --- |

| Title: | CE5: report of CE5-1.4 and CE5-3.2 on Joint chroma cross-component adaptive loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. He, F. Le Léannec, E. Francois | Tel: Email: | yong.he@interdigital.com fabrice.leleannec@interdigital.com edouard.francois@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Cross-Component Adaptive Loop Filter (CCALF) (CE5-1.1 anchor method) applies a linear diamond shaped filter on luma samples and uses the output signal to refine each chroma component. To reduce the complexity, joint chroma CCALF (JC-CCALF) (CE5-1.4) uses only one set of CCALF filter coefficients to generate one filtered output as the refinement signal, which is then added directly to the Cb component, and be properly weighted and then added to the Cr component. It reduces the complexity of CCALF roughly by half. Simulation result shows that compared to VTM-7.0, CE5-1.4 JC-CCALF yields: −1.08% / −1.66% / −2.15% / −2.12% BD-YUV variations for AI/RA/LDB/LDP, while the original CE5-1.1 CCALF yields: −1.40% / −1.88% / −2.15% / −2.11%. Compared to CCALF, JC-CCALF yields: +0.32% / +0.22% /  0.00% / −0.01% BD-YUV change. CE5-3.2 combines CE5-1.4 with filtering multiplication removal of CE5-2.1, yielding: −1.03% / −1.64% / −2.17% BD-YUV variations over VTM-7.0 for AI/RA/LDB.   Compared to CE5 anchor, CE5-1.4 JC-CCALF yields: 0.37% / 0.27% / 0.02% / 0.00% BD-YUV variations for AI/RA/LDB/LDP. Compared to CE5 anchor, CE5-3.2 JC-CCALF yields: 0.43% / 0.30% / 0.00% BD-YUV variations for AI/RA/LDB.

# Introduction

## Normative change

Cross-Component Adaptive Loop Filter (CCALF) (CE5-1.1 anchor method) makes use of luma sample values to refine each chroma component. For each chroma component sample of a CTU, one out of four 8-tap CCALF filters is selected and applied to a diamond region centered at the collocated position in luma component, the filter output as a refinement is added to chroma sample value [1]. This CCALF filtering process is conducted for Cb and Cr, respectively.

To reduce complexity of CCALF, joint chroma CCALF (JC-CCALF) (CE5-1.4) uses only one selected CCALF filter to generate one CCALF filtered output as the common chroma refinement signal that is applied to both Cb and Cr jointly. Particularly, the output refinement signal is applied to Cb directly, and it is properly weighted when applied to the Cr component [2]. In this way, the complexity of CCALF is reduced roughly by half. The proposed method is illustrated in Figure 1.

SAO Luma

ALF Luma

SAO Cb

SAO Cr

Joint chroma

CC ALF

ALF Cb

Y

Cb

Cr

w

ALF Cr

Figure 1. The proposed joint chroma CCALF method workflow

CE5-1.4 JC-CCALF is based on CCALF CE5-1.1 anchor method, where four 8-tap diamond shaped filters may be selected and used. At slice level, a decision is made on whether JC-CCALF is going to be applied for Cb and Cr, respectively. If it is applied to either of the two color components, a weight value will be coded (into a sign flag and a weight index). The weight index (denoted as weight_index) is coded into 3 bits and specifies the magnitude of the JC-CCALF weight JcCcWeight. It cannot be equal to 0. The magnitude of JcCcWeight is set to JcCcAlfWLut[ weight_index ] with { JcCcAlfWLut[ k ], k = 1..7 } = { 1, 2, 3, 4, 16, 8, 6 }, and then divided by 4 when actually applied. Weight values are selected via fast search at the encoder side when training for the ALF filter coefficients.

At CTU level, two separate flags indicate the application of JC-CCALF on the Cb and Cr component, respectively. If it is applied for either of the components, a filter index will be coded. As for the iterative training process of the JC-CCALF filter coefficients at the encoder, only the CTUs that the exact same one filter out of 4 is used for both Cb and Cr components from the previous iteration, will be included in the filter training of the current iteration.

## Encoder change

An encoder side improvement related to the CCALF filter training process has also been tested on top of the design explained above. This is used to to decide whether a CTU should be included when gathering the statistics in the training for a certain CCALF filter (i.e. one of the 4 CCALF filters). The encoder change consists in the following. For gathering the statistics for a filter, instead of requiring that both Cb and Cr had used that filter in the previous training iteration, either Cb or Cr is required using that filter in the previous training iteration.

This translate by the following change in the file EncAdaptiveLoopFilter.cpp (line 5156):

if ((m_trainingCovControl[0][ctuRsAddr] == filterIdc) && (m_trainingCovControl[1][ctuRsAddr] == filterIdc))

changed to:

if ((m_trainingCovControl[0][ctuRsAddr] == filterIdc) || (m_trainingCovControl[1][ctuRsAddr] == filterIdc))

This change has been tested on top of CE5-3.2 that combines CE5-1.4 with CE5-2.1.

# Simulation results

CTC tests are performed based on VTM-7.0 as the anchor [3]. CE5-1.4 JC-CCALF is implemented based on CE5-1.1 CCALF anchor method. In below, BD-YUV = (8*BD-Y + BD-Cb + BD-Cr)/10 is used.

## CE5-1.4 JC-CCALF without encoder improvement

The results vs VTM7.0 are reported in file JVET-P-CE5-1.4_jcccalf.xlsm.

Table 1. Performance of CE5-1.4 JC-CCALF vs VTM7.0

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.13% | -3.77% | -4.15% | -0.69% | 99% | 99% |
| Class A2 | 0.08% | -5.11% | -10.36% | -1.48% | 100% | 99% |
| Class B | 0.10% | -9.94% | -6.86% | -1.60% | 99% | 99% |
| Class C | 0.11% | -3.04% | -2.89% | -0.51% | 99% | 98% |
| Class E | 0.17% | -8.98% | -2.14% | -0.98% | 100% | 99% |
| Overall | 0.11% | -6.41% | -5.32% | -1.08% | 99% | 99% |
| Class D | 0.07% | -2.27% | -1.68% | -0.34% | 100% | 100% |
| Class F | 0.08% | -4.79% | -4.94% | -0.91% | 100% | 98% |
|  |  |  | Random access |  |  |  |
|  |  |  | Over VTM-7.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.14% | -5.17% | -5.02% | -0.91% | 99% | 100% |
| Class A2 | 0.14% | -9.12% | -14.45% | -2.25% | 99% | 101% |
| Class B | 0.19% | -15.52% | -11.90% | -2.59% | 99% | 99% |
| Class C | 0.11% | -3.74% | -3.33% | -0.62% | 99% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.15% | -9.03% | -8.75% | -1.66% | 99% | 99% |
| Class D | 0.08% | -3.37% | -2.65% | -0.54% | 100% | 99% |
| Class F | 0.09% | -3.16% | -3.21% | -0.56% | 100% | 99% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-7.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.14% | -18.74% | -16.73% | -3.44% | 99% | 99% |
| Class C | 0.09% | -5.28% | -4.05% | -0.86% | 99% | 100% |
| Class E | 0.11% | -13.05% | -5.02% | -1.72% | 100% | 100% |
| Overall | 0.11% | -12.83% | -9.58% | -2.15% | 100% | 100% |
| Class D | 0.10% | -5.05% | -5.06% | -0.93% | 99% | 96% |
| Class F | 0.14% | -7.47% | -6.29% | -1.26% | 99% | 100% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-7.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.19% | -18.48% | -16.34% | -3.33% | 99% | 97% |
| Class C | 0.12% | -4.95% | -4.11% | -0.81% | 98% | 92% |
| Class E | 0.02% | -13.85% | -4.77% | -1.85% | 98% | 95% |
| Overall | 0.13% | -12.82% | -9.37% | -2.12% | 98% | 95% |
| Class D | 0.11% | -5.37% | -3.86% | -0.84% | 98% | 93% |
| Class F | 0.14% | -7.81% | -6.41% | -1.31% | 98% | 95% |

The results vs CE5 anchor are reported in file JVET-P-CE5-1.4_jcccalf_vs_CE5anchor.xlsm.

Table 2. Performance of CE5-1.4 JC-CCALF vs CE5 anchor

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | -0.04% | 1.27% | 1.24% | 0.22% | 99% | 99% |
| Class A2 | -0.03% | 3.23% | 2.18% | 0.52% | 100% | 99% |
| Class B | -0.05% | 3.86% | 4.46% | 0.79% | 100% | 99% |
| Class C | -0.08% | 0.92% | 0.74% | 0.10% | 100% | 98% |
| Class E | -0.07% | 0.72% | 0.34% | 0.05% | 100% | 98% |
| Overall | -0.06% | 2.15% | 2.03% | 0.37% | 100% | 99% |
| Class D | -0.03% | 1.27% | 0.06% | 0.11% | 100% | 98% |
| Class F | -0.08% | 1.17% | 1.20% | 0.17% | 100% | 99% |
|  |  |  | Random access |  |  |  |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.02% | 0.57% | 1.14% | 0.19% | 100% | 100% |
| Class A2 | 0.02% | 2.20% | 1.88% | 0.42% | 100% | 100% |
| Class B | 0.01% | 2.23% | 2.19% | 0.45% | 100% | 99% |
| Class C | -0.02% | 0.16% | 0.01% | 0.00% | 99% | 98% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 1.34% | 1.34% | 0.27% | 100% | 99% |
| Class D | -0.02% | 0.57% | 0.29% | 0.07% | 100% | 98% |
| Class F | -0.07% | 0.81% | 0.68% | 0.10% | 99% | 98% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.03% | 0.73% | 0.82% | 0.13% | 100% | 99% |
| Class C | -0.01% | -0.01% | -0.27% | -0.03% | 100% | 102% |
| Class E | 0.01% | -0.22% | -1.06% | -0.12% | 99% | 101% |
| Overall | -0.01% | 0.25% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.48% | -2.11% | -0.25% | 99% | 99% |
| Class F | 0.05% | 0.56% | 0.21% | 0.12% | 99% | 100% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | 0.75% | 0.65% | 0.15% | 99% | 96% |
| Class C | 0.00% | -0.23% | -0.22% | -0.04% | 98% | 94% |
| Class E | -0.05% | -0.45% | -0.91% | -0.17% | 98% | 95% |
| Overall | -0.01% | 0.12% | -0.03% | 0.00% | 98% | 95% |
| Class D | 0.07% | -1.60% | -2.00% | -0.30% | 99% | 94% |
| Class F | 0.00% | -0.42% | -0.31% | -0.08% | 97% | 95% |

## CE5-3.2: combine CE5-1.4 with CE5-2.1 without encoder improvement

The results are reported in file JVET-P-CE5-3.2.xlsm.

CE5-2.1 removes the multiplications from the filtering process in CCALF via restricting the filter coefficients to be only selected 2N values. CE5-3.2 is a combined test with CE5-1.4 and CE5-2.1.

Table 3. Performance of CE5-3.2 without encoder improvement JC-CCALF vs VTM7.0

|  |  |  | Random access |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.13% | -4.86% | -4.50% | -0.83% | 100% | 100% |
| Class A2 | 0.16% | -7.88% | -13.37% | -2.00% | 100% | 101% |
| Class B | 0.22% | -15.02% | -11.45% | -2.47% | 100% | 99% |
| Class C | 0.10% | -4.01% | -3.31% | -0.65% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.16% | -8.63% | -8.27% | -1.56% | 100% | 100% |
| Class D | 0.04% | -4.17% | -2.79% | -0.66% | 100% | 100% |
| Class F | 0.11% | -2.92% | -3.06% | -0.51% | 100% | 100% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-7.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.16% | -18.38% | -16.05% | -3.32% | 100% | 101% |
| Class C | 0.11% | -5.26% | -4.33% | -0.87% | 100% | 99% |
| Class E | 0.09% | -13.18% | -5.35% | -1.78% | 100% | 100% |
| Overall | 0.12% | -12.71% | -9.47% | -2.12% | 100% | 100% |
| Class D | 0.09% | -5.71% | -4.43% | -0.94% | 100% | 98% |
| Class F | 0.17% | -7.89% | -6.17% | -1.27% | 100% | 100% |

## CE5-3.2: combine CE5-1.4 with CE5-2.1 with encoder improvement

The results vs VTM7.0 are reported in file JVET-P-CE5-3.2_encImprovt.xlsm.

Table 4. Performance of CE5-3.2 with encoder improvement JC-CCALF vs VTM7.0

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM7.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.12% | -3.45% | -3.74% | -0.63% | 99% | 98% |
| Class A2 | 0.08% | -4.82% | -9.74% | -1.40% | 98% | 98% |
| Class B | 0.08% | -9.36% | -6.73% | -1.54% | 99% | 98% |
| Class C | 0.09% | -2.99% | -2.91% | -0.52% | 99% | 96% |
| Class E | 0.13% | -8.23% | -2.03% | -0.92% | 99% | 96% |
| Overall | 0.10% | -6.02% | -5.10% | -1.03% | 99% | 97% |
| Class D | 0.07% | -2.58% | -1.87% | -0.39% | 99% | 97% |
| Class F | 0.06% | -4.53% | -4.72% | -0.87% | 100% | 96% |
|  |  |  | Random access |  |  |  |
|  |  |  | Over VTM7.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.15% | -5.10% | -4.78% | -0.87% | 98% | 99% |
| Class A2 | 0.13% | -9.13% | -14.09% | -2.22% | 99% | 100% |
| Class B | 0.20% | -15.18% | -11.87% | -2.54% | 99% | 96% |
| Class C | 0.11% | -3.94% | -3.34% | -0.64% | 99% | 96% |
| Class E |  |  |  |  |  |  |
| Overall | 0.15% | -8.96% | -8.62% | -1.64% | 99% | 97% |
| Class D | 0.06% | -4.03% | -2.95% | -0.65% | 99% | 96% |
| Class F | 0.10% | -3.07% | -3.08% | -0.54% | 100% | 97% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM7.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.15% | -18.81% | -16.63% | -3.42% | 99% | 98% |
| Class C | 0.11% | -5.52% | -4.24% | -0.89% | 99% | 95% |
| Class E | 0.06% | -12.97% | -5.31% | -1.78% | 99% | 96% |
| Overall | 0.12% | -12.92% | -9.67% | -2.17% | 99% | 97% |
| Class D | 0.03% | -5.95% | -4.99% | -1.07% | 99% | 94% |
| Class F | 0.15% | -7.90% | -6.29% | -1.30% | 99% | 98% |

The results vs CE5 anchor are reported in file JVET-P-CE5-3.2_encImprovt_vs_CE5anchor.xlsm.

Table 5. Performance of CE5-3.2 with encoder improvement JC-CCALF vs CE5 anchor

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | -0.05% | 1.61% | 1.68% | 0.29% | 99% | 99% |
| Class A2 | -0.03% | 3.55% | 2.91% | 0.62% | 99% | 98% |
| Class B | -0.07% | 4.53% | 4.61% | 0.86% | 99% | 97% |
| Class C | -0.10% | 0.97% | 0.73% | 0.09% | 100% | 96% |
| Class E | -0.11% | 1.54% | 0.44% | 0.11% | 99% | 96% |
| Overall | -0.07% | 2.59% | 2.28% | 0.43% | 99% | 97% |
| Class D | -0.03% | 0.94% | -0.13% | 0.06% | 100% | 95% |
| Class F | -0.10% | 1.47% | 1.44% | 0.21% | 99% | 96% |
|  |  |  | Random access |  |  |  |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.02% | 0.66% | 1.40% | 0.23% | 99% | 99% |
| Class A2 | 0.02% | 2.18% | 2.30% | 0.46% | 99% | 99% |
| Class B | 0.03% | 2.67% | 2.21% | 0.51% | 99% | 96% |
| Class C | -0.02% | -0.05% | 0.00% | -0.02% | 99% | 96% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 1.44% | 1.48% | 0.30% | 99% | 97% |
| Class D | -0.04% | -0.16% | -0.03% | -0.05% | 99% | 95% |
| Class F | -0.06% | 0.92% | 0.82% | 0.12% | 100% | 96% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | 0.67% | 0.99% | 0.16% | 100% | 98% |
| Class C | 0.01% | -0.26% | -0.46% | -0.06% | 99% | 97% |
| Class E | -0.03% | -0.14% | -1.36% | -0.18% | 99% | 97% |
| Overall | -0.01% | 0.16% | -0.08% | 0.00% | 99% | 97% |
| Class D | -0.04% | -1.45% | -2.03% | -0.38% | 99% | 96% |
| Class F | 0.06% | 0.12% | 0.20% | 0.08% | 99% | 98% |

# Conclusion

To reduce complexity of CCALF, joint chroma CCALF conducts ALF filtering only once and applies the output refinement signal for both chroma components, i.e. directly applied to Cb, and weighted and applied to Cr, respectively. In this way, the computation complexity of CCALF is roughly reduced by half. On VTM-7.0, CE5-1.4 JC-CCALF yields: −1.08%/−1.66%/−2.15%/−2.12% BD-YUV variations for AI/RA/LDB/LDP. Compared to CE5-1.1 CCALF, JC-CCALF yields: +0.32%/+0.22%/0%/−0.01% BD-YUV change. Further combining with CE5-2.1 filter coefficient multiplication removal (CE5-3.2) yields: −1.03% / −1.64% / −2.17% BD-YUV variations over VTM-7.0 for AI/RA/LDB.

Compared to CE5 anchor, CE5-1.4 JC-CCALF yields: 0.37% / 0.27% / 0.02% / 0.00% BD-YUV variations for AI/RA/LDB/LDP. Compared to CE5 anchor, CE5-3.2 JC-CCALF yields: 0.43% / 0.30% / 0.00% BD-YUV variations for AI/RA/LDB.

# References

- K. Misra, F. Bossen, A. Segall, N. Hu, J. Dong, V. Seregin, M. Karczewicz, P. Onno, C. Gisquet, G. Laroche, J. Li, C.S. Lim, C.-W. Kuo, J. Nam, J. Choi, J. Lim, S. Kim, O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei, “CE5-related: On the design of CC-ALF”, JVET-P1008, 16th meeting, Oct 2019, Geneva, CH.

- H. Yang, Y. He, H. Li, “CE5-related: Joint chroma cross-component adaptive loop filtering”, JVET-P0372, 16th meeting, Oct 2019, Geneva, CH.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring,"JVET common test conditions and software reference configurations for SDR video", JVET-O2010, Gothenburg, Sweden, July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).