| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0233-v1 |
| --- | --- |

| Title: | AHG17 Signaling subpicture without slice for merged picture and other use cases |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hirabayashi Mitsuhiro Mitsuru Katsumata Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-7393 Mitsuhiro.Hirabayashi@sony.com Mitsuru.Katsumata@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to enable subpictures without slices for removing the restriction of resolution of subpictures. The following changes to the VVC specification.

- For enabling subpicture without slice, non-slice subpicture is introducing.

  - Add the definition of non-slice subpicture and change the definition that divide picture into subpicture.

  - Add the definition of non-sample slice for consistency among picture, subpicture and, non-slice subpicture definition.

- Introducing the non_slice_subpic_flag that specifies the subpicture that has no slice in SPS.

Introduction

One of the merge use cases is network bandwidth adaption streaming.  For example, in 360 cubemap streaming, there are high and low resolution encoded bitstream on server that each subpicture represents a cubemap face for a 360 cubemap projection. When the bandwidth is limited, client get and merge the subpicture that is/are high resolution that included in the viewport and is/are low resolution that not include in the viewport (Fig. 1).

Fig. 1 Merge example of 360 cubemap streaming

However, in the case of a picture and subpicture with the resolution in high resolution picture and low resolution picture shown in the Fig. 2, there is a region without slice in the merged picture.

Fig. 2 Merged picture with the region without slice

In the current VVC specification draft in JVET-P2001-vE, it is necessary to make a rectangle picture when merging subpictures.  Therefore, merged picture in the Fig. 2 is not conformant picture for current VVC specification.  In order to make merged picture in the Fig. 2 conformant, content author need to support the appropriate resolution which can be merged, or client need to fill the region without slice by encoded sample.

The flexibility of encoding for content author and the merging for the client should be required for client -driven adaptive streaming services.

This contribution proposes to enable subpicture without slice.

- This contribution proposes to enable subpicture without slice. It is proposed to define the enable subpicture without slice.

- It is proposed to signal subpicture that has no slice.

Proposal

For removing the restrictions of resolution of subpicture, the following changes to the VVC specification.

- For enabling subpicture without slice, non-slice subpicture is introducing.

  - Add the definition of non-slice subpicture and change the definition that divide picture into subpicture.

  - Add the definition of non-sample slice for consistency among picture, subpicture and, non-slice subpicture definition.

- Introducing the non_slice_subpic_flag that specifies the subpicture that has no slice in SPS.

Proposal Text

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-P2001-vE)

Below are the proposed definition in 3. Definitions and in 6.3.3	Spatial or component-wise partitionings on top of JVET-P2001-vE:

3.	Definitions

3.x	non-sample slice: the slice region that have no samples.

3.x	non-slice subpicture: An rectangular region of one non-sample slice within a picture..

6.3.3	Spatial or component-wise partitionings

The following divisions of processing elements of this Specification form spatial or component-wise partitioning:

–	The division of each picture into components

–	The division of each component into CTBs

–	The division of each picture into subpictures or non-slice subpictures

–	The division of each picture into tile columns

–	The division of each picture into tile rows

–	The division of each tile column into tiles

–	The division of each tile row into tiles

–	The division of each tile into CTUs

–	The division of each picture into slices or non-sample slices

–	The division of each subpicture into slices

Below are the proposed syntax and semantics changes in the SPS on top of JVET-P2001-vE:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| non_slice_subpic_flag [ i ] | u(1) |
| } |  |
| } |  |
| … |  |

non_slice_subpic_flag [ i ] equal to 1 specifies that subpicture is non-slice subpictue. non_slice_subpic_flag [ i ] equal to 0 specifies that subpicture is not non-slice subpicture.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).