| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0233-v3 (v3:editorial and small update) |
| --- | --- |

| Title: | AHG12 Signaling subpicture without slice for merged picture and other use cases |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mitsuhiro Hirabayashi Mitsuru Katsumata Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-7393 Mitsuhiro.Hirabayashi@sony.com Mitsuru.Katsumata@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to enable subpictures without slice for simple and flexible subpicture merging. The following changes to the VVC specification.

For enabling subpicture without slice,

- Introducing non-slice subpicture definition as follows.

  - Add the definition of non-slice subpicture and change the definition that divide picture into subpicture.

  - In addition, add the definition of non-sample slice for consistency among picture, subpicture and, non-slice subpicture definition.

- Introducing the non_slice_subpic_present_flag and non_slice_subpic_flag in SPS, specifying the subpicture that has no slice.

Introduction

One of the merge use cases is network bandwidth adaption streaming.  For example, in 360 cubemap streaming, there are high, middle and low resolution encoded bitstream on server that each subpicture represents a cubemap face for a 360 cubemap projection. When the bandwidth is limited, client get and merge the high resolution subpictures included in the viewport and the low resolution subpictures not include in the viewport (Fig. 1).

Fig. 1 Merging example of 360 cubemap streaming

However, when the bandwidth is more limited than Fig. 1, client get and merge the middle resolution subpictures included in the viewport and the low resolution subpictures not include in the viewport as shown in the Fig. 2, the regions without slice occur in the merged picture.

Fig. 2 Merged picture with the region without slice

In the current VVC specification draft in JVET-P2001-vE, merged picture as shown in the Fig. 2 is not allowed. Because it is necessary to make a rectangle merged picture.

In order to make a rectangle merged picture, content author need to support the appropriate resolution which can be merged as shown in the Fig. 1, or client need to fill the region without slice in the Fig. 2 with encoded useless sample.

Hence, the flexibility of encoding variations by the content author, which is not only for subpicture merging, and/or the simple merging process by the client would be useful for client -driven adaptive streaming use case.

This contribution proposes to enable subpicture without slice for the above flexibility and simplification.

Proposal

For enabling subpicture without slice,

- Introducing non-slice subpicture definition as follows.

  - Add the definition of non-slice subpicture and change the definition that divide picture into subpicture.

  - In addition, add the definition of non-sample slice for consistency among picture, subpicture and, non-slice subpicture definition.

- Introducing the non_slice_subpic_present_flag and non_slice_subpic_flag in SPS, specifying the subpicture that has no slice

This subpicture level signaling enables and flexible combination of subpictures and the simple subpicture merging process. Hence, regions without slice might occur in the merged picture.

Merging process related to this contribution is as follows

- Get the appropriate combination of subpictures depending on the network bandwidth

- When a region without slice would occur in the merged picture, Set subpicture mapping for those.

- Set the non_slice_subpic_present_flag and non_slice_subpic_flag in SPS for those.

Note that the bitstream in the merged picture shall be conformed to the VVC specification such as tile boundary and other information might be needed to select subpictures to be merged form different streams. This is not the scope of this contribution, though.

Proposal Text

The proposed syntax and semantics are marked in yellow with removed text in red strike through on top of the current VVC draft (JVET-P2001-vE)

Below are the proposed definition in 3. Definitions and in 6.3.3	Spatial or component-wise partitionings on top of JVET-P2001-vE:

3.	Definitions

3.x	non-sample slice: the slice region that have no samples.

3.x	non-slice subpicture: An rectangular region of one non-sample slice within a picture..

6.3.3	Spatial or component-wise partitionings

The following divisions of processing elements of this Specification form spatial or component-wise partitioning:

–	The division of each picture into components

–	The division of each component into CTBs

–	The division of each picture into subpictures or non-slice subpictures

–	The division of each picture into tile columns

–	The division of each picture into tile rows

–	The division of each tile column into tiles

–	The division of each tile row into tiles

–	The division of each tile into CTUs

–	The division of each picture into slices or non-sample slices

–	The division of each subpicture into slices

Below are the proposed syntax and semantics changes in the SPS on top of JVET-P2001-vE:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| non_slice_subpic_present_flag | u(1) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| if(non_slice_subpic_present_flag) |  |
| non_slice_subpic_flag [ i ] | u(1) |
| } |  |
| } |  |
| … |  |

non_slice_subpic_present_flag equal to 1 specifies that non-slice subpicture flag is present in the SPS. non_slice_subpic_present_flag equal to 0 specifies that non-slice subpicture flag is not present in the SPS.

non_slice_subpic_flag [ i ] equal to 1 specifies that subpicture is non-slice subpictue. non_slice_subpic_flag [ i ] equal to 0 specifies that subpicture is not non-slice subpicture.  When not present, the value of non_slice_subpic_flag [ i ] is inferred to be equal to 0.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).