| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0238 |
| --- | --- |

| Title: | AHG8/AHG9: On reference picture wraparound |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to move the reference wraparound signalling from SPS to PPS. Consequently, the contribution proposes enabling both reference wraparound and reference picture resampling for the same reference picture in the same layer as the current picture. The contribution also proposes to keep the capability of VVC Draft 7 to apply the reference wrapround feature for spatial scalability.

# Introduction

VVC Draft 7 specifies reference picture wraparound in SPS, which has the following consequences:

- Since each layer may refer to a different SPS, reference wraparound can be properly specified for spatial scalability.

- Adaptive resolution changing within a single layer and with reference wraparound is practically disabled.

The proponent asserts that the reference wraparound feature is important to enable proper resampling of ERP pictures for adaptive resolution changing or spatial scalability. In order to enable the capability to apply both reference wraparound and reference picture resampling for the same reference picture for adaptive resolution changing within a layer, the contribution proposes to move the reference wraparound signalling from SPS to PPS. The proposed change keeps the capability to apply the reference wrapround feature for spatial scalability.

# Proposal

The proposed specification text is provided as change marks in the accompanying document. Key changes in syntax and semantics are copied below.

## Proposed syntax changes

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| … |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| scaling_window_flag | u(1) |
| if( scaling_window_flag ) { |  |
| scaling_win_left_offset | ue(v) |
| scaling_win_right_offset | ue(v) |
| scaling_win_top_offset | ue(v) |
| scaling_win_bottom_offset | ue(v) |
| } |  |
| pps_ref_wraparound_enabled_flag | u(1) |
| if( pps_ref_wraparound_enabled_flag ) |  |
| pps_ref_wraparound_offset_minus1 | ue(v) |
| … |  |

## Proposed semantics

pps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. pps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is less than or equal to ( pic_width_in_luma_samples / MinCbSizeY – 1 ), the value of pps_ref_wraparound_enabled_flag shall be equal to 0.

pps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of pps_ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_in_luma_samples / MinCbSizeY ) − 1, inclusive.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).