| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0241-v3 |
| --- | --- |

| Title: | On QP adjustment in adaptive color transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehong Jung Dongcheol Kim Geonjung Ko Ju-Hyung (John) Son Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution presents bug fixes on QP derivation when ACT (Adaptive Colour Transform) is applied. In the current VVC design, QP is adjusted by negative integer offset to compensate dynamic range change of residual signal. It is reported that there are two bugs on QP derivation when ACT is applied.

- First, QP offsets in the current draft text are not aligned with JVET-P0517, the adopted version of ACT and VTM-7.1 software.

- Second, after adjusting QP by the offset, the modified QP can be a negative value. Also, when transform skip mode is applied, the modified QP can exceed the minimum allowed QP for transform skip mode, specified as QpPrimeTsMin.

For the first issue, it is proposed to align the draft text with VTM-7.1.

For the second issue, it is proposed to clip the modified QP to QpPrimeTsMin when transform skip mode is applied and to 0 when transform is applied.

It is reported that BD-rate changes of {G/Y, B/U, R/V} over VTM-7.1 are as follows.

- RGB sequences: {-0.26%, -0.18%, -0.17%} for AI, {-0.11%, -0.05%, -0.02%} for RA, and {-0.04%, -0.03%, -0.03%} for LDB

- YUV sequences: {0.00%, 0.00%, 0.00%} for AI, {0.00%, 0.00%, 0.00%} for RA, and {-0.02%, 0.00%, 0.00%} for LDB

- Average: {-0.13%, -0.09%, -0.08%} for AI, {-0.05%	, -0.03%, -0.01%} for RA, and {-0.03%, -0.02%, -0.01%} for LDB

Problem statement

In the 16th JVET meeting, ACT was adopted to the VVC in order to efficiently reduce the redundancy between color components [1]. Same as in the HEVC, the following forward and inverse YCgCo transform matrices are applied.

| Forward transform YCgCo=2112-1-10-22GBR/4 | Inverse transform GBR=1101-1-11-11YCgCo | (1) |
| --- | --- | --- |

As shown in the above matrices, the ACT transform matrices are not normalized. To compensate the change on dynamic range of residual signals before and after color transform, the QP adjustments of (-5, -5, -3) are applied to the residuals of Y, Cg and Co components, respectively.

Regarding the QP adjustment, there are two problems in the current design. Details are described in the following subsections.

Mismatch on QP offset

According to JVET-P0517 [2], the adopted version of ACT, QP offsets of (-5, -5, -5, -3) are applied to the residuals of Y, CbCr, Cb and Cr, respectively. Meanwhile, in the current VVC draft [3], QP offset of -5 is applied to all components. It is noted that the offsets in VTM-7.1 match those in JVET-P0517. Table 1 shows the summarized QP offset in JVET-P0517, VTM-7.1 and the draft text.

Table 1 – Mismatch on QP offset values between VTM-7.1 and VVC Draft 7

| Component | QP offset |  |
| --- | --- | --- |
|  | JVET-P0517, VTM-7.1 | VVC Draft 7 |
| Y | -5 | -5 |
| CbCr | -5 | -5 |
| Cb | -5 | -5 |
| Cr | -3 | -5 |

Negative QP value after QP adjustment

In clause 8.7.3 of VVC draft, the quantization parameter qP is modified as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 ) 	(2)

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, qP ) − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 ) 	 (3)

Based on (2) and (3), the modified qP can be a negative integer value. Because the quantization step size is defined on positive integer range of qP, qP should be clipped to non-negative integer value.

Furthermore, as shown in (3), when transform skip mode is applied, the modified qP can exceed the minimum allowed quantization parameter for transform skip mode specified as QpPrimeTsMin.

Proposed method

For the mismatch on QP offset issue, it is proposed to fix the QP offset in the current draft text as (-5, -5, -5, -3) for Y, CbCr, Cb and Cr, respectively.

For the negative QP value after QP adjustment issue, it is proposed to clip the adjusted QP as follows:

- When transform skip mode is not applied, the QP is clipped to 0.

- When transform skip mode is applied, the QP is clipped to the minimum allowed QP for transform skip mode, QpPrimeTsMin.

The proposed draft text changes over JVET-Q0041-v2_DraftText are presented. The changes for the first issue and the second issue are highlighted in yellow and green, respectively.

8.7.3 Scaling process for transform coefficients Inputs to this process are: - a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture, - a variable nTbW specifying the transform block width, - a variable nTbH specifying the transform block height, - a variable predMode specifying the prediction mode of the coding unit, - a variable cIdx specifying the colour component of the current block. Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ]. The quantization parameter qP and the variable QpActOffset is are derived as follows: - If cIdx is equal to 0, the following applies: qP = Qp′Y (1129) QpActOffset = -5 (xxxx) - Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies: qP = Qp′CbCr (1130) QpActOffset = -5 (xxxx) - Otherwise, if cIdx is equal to 1, the following applies: qP = Qp′Cb (1131) QpActOffset = -5 (xxxx) - Otherwise (cIdx is equal to 2), the following applies: qP = Qp′Cr (1132) QpActOffset = -3 (xxxx) The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows: - If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies: qP = Max( 0, qP −+ ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5QpActOffset : 0 ) ) (1133) rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 ) = = 1 ) ? 1 : 0 (1134) bdShift = BitDepth + rectNonTsFlag + (1135) ( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag - Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies: qP = Max( QpPrimeTsMin, qP ) − ( + cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5QpActOffset : 0 ) (1136) rectNonTsFlag = 0 (1137) bdShift = 10 (1138)

Experimental results

The proposed method was implemented on top of VTM-7.1 [4], and tested under the CE2 test condition [5]. In the experiments, IBC, HashME, PLT, BDPCM (both luma and chroma) and ColorTransform are enabled on low QPs, (2, 7, 12, 17) with the first 100 frames for both anchor and test. Table 2 shows the summarized test results for dual tree off case.

Table 2 – Test results for dual tree off

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.4% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.2% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | -0.2% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

Conclusion

In this contribution, it is proposed to fix the bugs on QP derivation when ACT is applied. It is recommended to adopt the proposed method into the next version of draft text and VTM.

References

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 7 (VTM 7),” JVET-P2002, 16th Meeting, Geneva, CH, 1–11 Oct. 2019

- X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, “Support of adaptive color transform for 444 video coding in VVC,” JVET-P0517, 16th Meeting, Geneva, CH, 1–11 Oct. 2019

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, 16th Meeting, Geneva, CH, 1–11 Oct. 2019

- VTM-7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-7.1

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding,” JVET-P2022, 16th Meeting, Geneva, CH, 1–11 Oct. 2019

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).