| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0249 |
| --- | --- |

| Title: | Clipping flag clean-up for chroma adaptive loop filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VVC draft 7, one adaptation parameter set can signal up to 8 chroma filters. One clipping flag is signalled for each chroma filter to control whether clipping is applied for this filter or not. In this contribution, it is proposed to apply one clipping flag to all chroma filters in one adaptation parameter set. Under common test conditions, compared to VTM-7.0 anchor, similar overall coding efficiency and running time are observed.

# Introduction

In VVC draft 7, one adaptation parameter set (APS) can signal up to 25 luma adaptive loop filters (ALF) and 8 chroma filters. A clipping flag (alf_luma_clip_flag) is signalled per APS. If alf_luma_clip_flag is true, clipping is applied to all luma filters in this APS. Otherwise, no clipping is applied to any luma filter. On the other hand, a clipping flag (alf_chroma_clip_flag[ altIdx ]) is signalled for each chroma filter per APS. If alf_chroma_clip_flag[ altIdx ] is true, clipping is applied to the chroma filter with filter index altIdx, otherwise, no clipping is applied to that chroma filter. In this contribution, it is proposed to signal one clipping flag for all chroma filters per APS to have similar design used for luma clipping.

# Working draft change

In function alf_data(), the following change is made

| if( alf_chroma_filter_signal_flag ) { |  |
| --- | --- |
| alf_chroma_clip_flag | u(1) |
| alf_chroma_num_alt_filters_minus1 | ue(v) |
| for( altIdx = 0; altIdx <= alf_chroma_num_alt_filters_minus1; altIdx++ ) { |  |
| alf_chroma_clip_flag[ altIdx ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ altIdx ][ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ altIdx ][ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag[ altIdx ] ) { |  |
| for( j = 0; j < 6; j++ ) |  |
| alf_chroma_clip_idx[ altIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |

In adaptive loop filter data semantics, the following change is made

alf_chroma_clip_flag[ altIdx ] equal to 0 specifies that linear adaptive loop filtering is applied on chroma components when using the chroma filter with index altIdx; alf_chroma_clip_flag[ altIdx ] equal to 1 specifies that non-linear adaptive loop filtering is applied on chroma components when using the chroma filter with index altIdx. When not present, alf_chroma_clip_flag[ altIdx ] is inferred to be equal to 0.

# Experimental results

The proposed method is implemented on top of VTM-7.0 and tested under JVET common test conditions [1].

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 99% | 99% |
| Class A2 | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class C | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class E | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Class F | 0.00% | -0.01% | -0.01% | 100% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.01% | 100% | 100% |
| Class A2 | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.07% | 0.07% | 100% | 100% |
| Class C | 0.01% | -0.01% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.04% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class F | -0.01% | 0.03% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.36% | -0.25% | 100% | 102% |
| Class C | -0.02% | -0.16% | -0.13% | 101% | 100% |
| Class E | -0.05% | 0.42% | -0.15% | 100% | 100% |
| Overall | -0.02% | -0.10% | -0.19% | 100% | 101% |
| Class D | -0.03% | -0.16% | 0.01% | 100% | 101% |
| Class F | 0.06% | -0.56% | 0.22% | 100% | 102% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.26% | -0.03% | 100% | 102% |
| Class C | 0.00% | -0.07% | -0.16% | 100% | 99% |
| Class E | -0.02% | -0.01% | -0.05% | 100% | 100% |
| Overall | 0.00% | 0.08% | -0.08% | 100% | 101% |
| Class D | -0.04% | -0.03% | -0.03% | 100% | 99% |
| Class F | 0.02% | 0.31% | 0.18% | 100% | 100% |

# Reference

[1] JVET-N1010, “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).