| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0252 |
| --- | --- |

| Title: | CE5-related: Line buffer problem of CCALF for 4:4:4 and 4:2:2 sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. To get an offset for a chroma sample, the filter is applied to luma samples (after applying luma sample adaptive offset) where the centre of the filter is the co-located luma sample of the current chroma sample.

In VVC draft 7, when applying loop filters, virtual boundary (VB) is defined as 4 lines of luma samples and 2 lines of chroma samples above a coding tree unit. When applying adaptive loop filter to a sample, the samples on the other side of a VB cannot be utilized.

However, when CC-ALF is applied to 4:4:4 and 4:2:2 video sequences, it is possible that a chroma sample and its co-located luma sample are located at different sides of an ALF VB. Under this circumstance, more lines of chroma samples must be stored while waiting for the co-located luma sample to be available.

In this contribution, two methods are proposed to solve the problem. Compared to CE5 anchor, similar coding efficiency and running time are observed.

# Introduction

In JVET-P1008, a cross component adaptive loop filter (CC-ALF) is tested to refine chroma components by using luma samples. To get an offset for a chroma sample, after deblocking filter (DBF) and sample adaptive offset (SAO), a 3x4 filter shown in Figure 2 is applied to luma samples. The centre of the filter (f2 in Figure 2) is the co-located luma sample of the current chroma sample.

Figure 1. Placement of CC-ALF with respect to other in-loop filters (JVET-P0080)

|  | f0 |  |
| --- | --- | --- |
| f1 | f2 | f3 |
| f4 | f5 | f6 |
|  | f7 |  |

Figure 2. 3x4 filter shape in CE5 anchor

In VVC draft 7, when applying loop filters, virtual boundary (VB) is defined as 4 lines of luma samples and 2 lines of chroma samples above a coding tree unit (CTU) as Figure 3. In Figure 3, when reconstructing a CTU, in-loop filters cannot be applied to the blue bottom samples until the pixels in below CTU become available. To reduce the line buffer requirement, when applying adaptive loop filter (ALF) and CC-ALF to a pixel, the samples on the other side of ALF VB cannot be utilized.

Figure 3. Virtual boundaries in VVC

Figure 4. Co-located luma samples mapping for chroma samples. Left: 4:2:0, right: 4:4:4 and 4:2:2

For 4:4:4 and 4:2:2 video sequences, a chroma sample and its co-located luma sample may be located at different sides of an ALF VB since there is no chroma sub-sampling in the vertical direction for those formats. For example, in Figure 4, chroma sample A is above the VB. Before applying CC-ALF and ALF to A, DBF and SAO have been applied to A. However, DBF and SAO have not been applied to its co-located luma pixels A’. Therefore, A’ cannot be used when applying CC-ALF to A, unless A is waiting until DBF and SAO are done for A’, which will require more line buffers by store As in memory.

# Proposed solution

To solve the above problem and avoid introducing extra chroma line buffers, two methods are proposed in this contribution.

## Method 1

Figure 5. Clip the position of the co-located luma sample within the VB

In the method 1, the position of the co-located luma sample is clipped such that the nearest luma sample within the same ALF VB is used as the co-located luma sample.

Spec change on top of CE5 anchor:

  - The luma location (xL, yL) corresponding to the current chroma sample at chroma location ( xCtbC + x, yCtbC + y ) is set equal to ( (xCtbC + x ) * SubWidthC, ( yCtbC + y ) * SubHeightC )

  - If y < CtbSizeY / SubWidthC – 2 and y * SubWidthC >= CtbSizeY – 4, yL is set equal to yCtbC * SubHeightC + CtbSizeY - 5

## Method 2

In the method 2, when a chroma sample and its co-located luma sample are located at different sides of an ALF VB, CC-ALF is not applied to this chroma sample.

Spec change on top of CE5 anchor:

  - The variable sum is derived as follows:

  - If y < CtbSizeY / SubWidthC – 2 and y * SubWidthC >= CtbSizeY – 4, sum = 0;

  - Otherwise:

sum = 	f[ 0 ] * recPictureL[ hx, vy + yM1 ] +
	f[ 1 ] * recPictureL[ hx+xM1, vy ] +
	f[ 2 ] * recPictureL[ hx, vy ] +
	f[ 3 ] * recPictureL[ hx+xP1, vy ] +		(8-1289)	f[ 4 ] * recPictureL[ hx+xM1, vy+yP1 ] +
	f[ 5 ] * recPictureL[ hx, vy+yP1 ] +
	f[ 6 ] * recPictureL[ hx+xP1, vy+yP1 ] +	
	f[ 7 ] * recPictureL[ hx, vy + yP2 ]

# Experimental results

The proposed methods are implemented on top of CE5 anchor and are tested by using 4:4:4 sequences. Following CE2 test conditions for 4:4:4 content, the common test conditions defined in JVET-N1010 are used with the additional configuration parameters for 4:4:4 sequences: IBF=1, HashME=1, BDPCM=2, PLT=1.

## Method 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE5Anchor_PLT1_IBC1_BDPCM2_HASHME1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 96% | 101% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 106% | 105% |
| Animation | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Mixed content | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Camera-Captured content | 0.00% | 0.01% | 0.04% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0_CE5Anchor_PLT1_IBC1_BDPCM2_HASHME1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | -0.01% | -0.02% | 99% | 99% |
| TGM 720p | -0.01% | -0.04% | 0.01% | 98% | 100% |
| Animation | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Mixed content | -0.05% | -0.11% | -0.05% | 101% | 102% |
| Camera-Captured content | 0.02% | 0.08% | 0.04% | 101% | 101% |
| Overall | -0.01% | -0.02% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0_CE5Anchor_PLT1_IBC1_BDPCM2_HASHME1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.02% | 0.00% | 102% | 103% |
| TGM 720p | 0.03% | 0.03% | 0.25% | 100% | 102% |
| Animation | 0.05% | 0.16% | 0.06% | 98% | 97% |
| Mixed content | -0.03% | 0.04% | 0.12% | 99% | 99% |
| Camera-Captured content | 0.05% | 0.06% | -0.18% | 101% | 105% |
| Overall | 0.02% | 0.06% | 0.07% | 100% | 101% |

## Method 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE5Anchor_PLT1_IBC1_BDPCM2_HASHME1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.01% | 0.01% | 98% | 100% |
| TGM 720p | 0.00% | 0.04% | 0.02% | 104% | 105% |
| Animation | 0.00% | -0.01% | 0.00% | 99% | 99% |
| Mixed content | 0.00% | 0.04% | 0.04% | 100% | 97% |
| Camera-Captured content | 0.00% | 0.03% | 0.08% | 104% | 106% |
| Overall | 0.00% | 0.02% | 0.02% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0_CE5Anchor_PLT1_IBC1_BDPCM2_HASHME1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | 0.00% | -0.01% | 100% | 100% |
| TGM 720p | -0.03% | 0.00% | 0.02% | 98% | 99% |
| Animation | 0.01% | 0.02% | 0.02% | 104% | 103% |
| Mixed content | 0.00% | 0.03% | 0.07% | 99% | 99% |
| Camera-Captured content | -0.02% | 0.00% | 0.07% | 102% | 100% |
| Overall | -0.01% | 0.01% | 0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0_CE5Anchor_PLT1_IBC1_BDPCM2_HASHME1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.09% | 0.08% | 0.05% | 100% | 101% |
| TGM 720p | 0.01% | 0.05% | 0.38% | 102% | 102% |
| Animation | 0.03% | 0.16% | 0.08% | 97% | 99% |
| Mixed content | -0.10% | 0.01% | 0.06% | 96% | 97% |
| Camera-Captured content | 0.10% | 0.01% | -0.12% | 103% | 102% |
| Overall | 0.02% | 0.07% | 0.12% | 100% | 100% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).