| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0254-v3 |
| --- | --- |

| Title: | [AHG9]: Override mechanism for ALF related syntax elements in slice header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra Semih Esenlik Biao Wang Han Gao Elena Alshina | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

_____________________________

# Abstract

This contribution proposes an override mechanism for the adaptive loop filter ALF syntax elements (ALF data) in the slice header to further reduce the parsing overhead of slice header. All the slices which share the same ALF data simply inherit the information from the picture header and the slices which have different ALF data as than the one present in picture header simply override and signal their own ALF related data. This mechanism reportedly reduces the signalling overhead in slice header for content with varied signal characteristics.

# Introduction

Picture header was introduced in the VVC draft [1] to mainly reduce the parsing overhead of slice header. Adaptive loop filter (ALF) related syntax elements can either be signalled in the picture header or the slice header but not both.  The ALF related syntax elements in the picture header and slice header is given below.

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| …… |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| …….. |  |

When all slices share the same ALF data, then the common ALF data can be signalled only once in the picture header and all the slices can simply inherit the information from picture header. However even if one of the slices has atleast one of the ALF related syntax element value different when compared to the other slices, then the entries in the picture header cannot be used anymore. Therefore each slice has to transmit the ALF data in its own slice header even if most of the information is common across all the slices. It is noted to be that the ALF data in each slice header in the worst case can be upto 34 bits and if CCALF tool [4] is adopted into VVC, then it would cost few more additional bits. When a picture has multiple slices then the overheard increase even further.  The current contribution proposes an override mechanism to address this problem.

# Proposed method

The proposal introduces an override mechanism for ALF data wherein the picture header initially signals a first set of ALF related syntax elements. Consequently each slice header signals a syntax element called “slice_alf_override_flag”.  When the “slice_alf_override_flag” is false, then the slice simply inherits the ALF data from the picture header. When “slice_alf_override_flag” is true, then the slice signals its own set of ALF related syntax elements.  This method ensures that the parsing overhead of slice header is reduced when content with mixed signal characteristics is encoded. The next section introduces the specification text changes

# Specification text changes

The specification text changes are provided on top of [1].

7.3.2.6  Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_override_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( sps_alf_enabled_flag && !pic_alf_override_enabled_present_flag ) { |  |
| slice_alf_override_flag | u(1) |
| if(slice_alf_override_flag) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } /* end of slice_alf_enabled_flag */ |  |
| } /* end of slice_alf_override_flag */ |  |
| } /* end of sps_alf_enabled_flag */ |  |
| ….. |  |

pic_alf_override_enabled_flag equal to 1 specifies the presence of slice_alf_ovderride_flag in the slice header referrring to the PH. Pic_alf_override_enabled_flag equal to 0 specifies the absence of slice_alf_ovderride_flag in the slice header referring to the PH. When not present, the value of pic_alf_override_enabled_flag is inferred to be equal to 0.

slice_alf_override_flag equal to 1 specifies that the ALF parameters are present in the slice header. slice_alf_override_flag equal to 0 specifies that ALF parameters are not present in the slice header. When not present, the value of slice_alf_override_flag is inferred to be equal to 0.

After the initial discussion in the HLS BOG on 7th January 2020, the following cleanup to the syntax is proposed.

7.3.2.6  Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_override_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| if( sps_alf_enabled_flag && !pic_alf_override_enabled_present_flag ) { |  |
| slice_alf_override_flag | u(1) |
| if(slice_alf_override_flag) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| …….. |  |

pic_alf_enabled_flag equal to 1 specifies that adaptive loop filter is enabled for all slices associated with the PH and may be applied to Y, Cb, or Cr colour component in the  slices. pic_alf_enabled_flag equal to 0 specifies that adaptive loop filter may be disabled for one, or more, or all slices associated with the PH. When not present, pic_alf_enabled_flag is inferred to be equal to 0.

According to the existing sematics of pic_alf_enabled_flag, It has be to noted that ALF filter may not be completely switched off for the whole picture when pic_alf_enabled_flag is set to 0.

pic_alf_override_enabled_flag equal to 1 specifies the presence of slice_alf_override_flag in the slice header referrring to the PH. pic_alf_override_enabled_flag equal to 0 specifies the absence of slice_alf_override_flag in the slice header referring to the PH. When not present, the value of pic_alf_override_enabled_flag is inferred to be equal to 0.

slice_alf_override_flag equal to 1 specifies that the ALF parameters are present in the slice header. slice_alf_override_flag equal to 0 specifies that ALF parameters are not present in the slice header. When not present, the value of slice_alf_override_flag is inferred to be equal to 0.

The different scenarios for overriding are as follows:

Case 1: when all slices use the same ALF information and the ALF needs to switched “Off” for the whole picture, then pic_alf_override_enabled_flag is set to 0 and pic_alf_enabled_flag is set to 0

Case 2: when all slices use the same ALF information and the ALF needs to be switched “On” for the whole picture, then pic_alf_override_enabled_flag is set to 0 and pic_alf_enabled_flag is set to 1.

Case 3: when a given slice wants to override the ALF information in the picture header, then  pic_alf_override_enabled_flag  is set to 1, pic_alf_enabled_flag can be set to 0 and the given slice  further signals the value of syntax element slice_alf_override_flag slice in its slice header. When slice_alf_override_flag is 1, then the given slice signalls its own ALF information. When slice_alf_override_flag is 0, given slice inheirts the information from picture header

# Conclusion

The current contribution provides an override mechanism for ALF related syntax elements in the slice header to further reduce the parsing overhead of slice header. All the slices which share the same ALF data simply inherit the information from the picture header and the slices which have different ALF data as than the one present in picture header simply override and signal their own ALF related data. This mechanism reportedly reduces the signalling overhead in slice header for content with mixed signal characteristics.

It is suggested to adopt the proposed ALF syntax changes.

# References

- Versatile Video Coding (Draft 7), Document JVET-P2001-vE, B. Bross, J. Chen, S. Liu,Y.K. Wang

- “JVET common test conditions and software reference configurations”, JVET-N1010

- Description of Core Experiment 5 (CE5): Cross Component Adaptive Loop Filtering, JVET-P2025

- CE5 Common Base: Cross Component Adaptive Loop Filter, JVET-Q0058

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).