| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0259 |
| --- | --- |

| Title: | AHG9: On Picture Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains five proposals related to the picture header that was introduced in the VVC at the 16th JVET meeting. It is asserted that the proposals reduce redundancy and improve consistency in the latest VVC draft. The proposals relate to:

- Signal two separate flags for indication of partition constraint override.

- Indicate in PPS if SAO data is signalled in picture header or in slice header.

- Indicate in PPS if ALF data is signalled in picture header or in slice header.

- Indicate in PPS if RPL information is signalled in picture header or in slice header.

- Indicate information about collocated picture in picture header when RPL information is signalled in the picture header.

The first four proposals all provide gains under common usage and will impact the common test conditions. These proposals have been implemented in VTM 7.1 and results are provided in combination with the proposed Coded Picture NAL unit type in JVET -Q00255, showing an average luma BD-rate gain of 0.63% for LDB. The fifth aspect provide consistency in the design of TMVP signalling and can provide coding gains when multiple slices are enabled.

# Introduction

The Picture Header NAL unit was introduced in VVC at the 16th JVET meeting. Information that is typically the same in all slices of a picture but that is likely to change from one picture to another is gathered in the picture header. The proposals presented in this contribution are intended to further improve efficiency and consistency in the design of the picture header.

# Proposal 1

In the current draft of VVC, the signalling of partition constraints override in the picture header includes different parameters for intra slices and inter slices, and parameters for both intra slices and inter slices are present when partition_constraints_override_flag is equal to 1. It is asserted that it is very common to encode pictures using either only intra slices or only inter slices. One example is when a picture is encoded with a single slice. Another example is an IRAP picture which is required to only consist of intra slices.

It is proposed to use two different flags for indicating partitioning override:  partition_constraints_override_intra_slice_flag which applies to intra slices and partition_constraints_override_inter_slice_flag which applies to inter slices.

## Proposed syntax:

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_intra_slice_flag | u(1) |
| if( partition_constraints_override_intra_slice_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| partition_constraints_override_inter_slice_flag | u(1) |
| if( partition_constraints_override_inter_slice_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

## Proposed semantics:

partition_constraints_override_intra_slice_flag equal to 1 specifies that partition constraint parameters for intra slices are present in the PH. partition_constraints_override_intra_slice_flag equal to 0 specifies that partition constraint parameters for intra slices are not present in the PH. When not present, the value of partition_constraints_override_intra_slice_flag is inferred to be equal to 0.

partition_constraints_override_inter_slice_flag equal to 1 specifies that partition constraint parameters for inter slices are present in the PH. partition_constraints_override_inter_slice_flag equal to 0 specifies that partition constraint parameters for inter slices are not present in the PH. When not present, the value of partition_constraints_override_inter_slice_flag is inferred to be equal to 0.

## Alternative version of proposal 1

In order to further reduce the signalling overhead, it is possible to first signal flags to indicate if intra slice parameters and inter slice parameters are present or not. These flags can then be used to make partition_constraints_override_intra_slice_flag and partition_constraints_override_inter_slice_flag conditionally present in the picture header. The flags can also be used to make other syntax elements in the picture header conditionally present, such as signalling of qp delta and chroma qp offset. It is assumed that if parameters related to intra slices are not present, then parameters related to inter slices will be present.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( partition_constraints_override_enabled_flag ¦ ¦ cu_qp_delta_enabled_flag ¦ ¦ pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| intra_parameters_flag | u(1) |
| if( intra_parameters_flag ) |  |
| inter_parameters_flag | u(1) |
| if( partition_constraints_override_enabled_flag ) { |  |
| if( intra_parameters_flag ) { |  |
| partition_constraints_override_intra_slice_flag | u(1) |
| if( partition_constraints_override_intra_slice_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( inter_parameters_flag ) |  |
| partition_constraints_override_inter_slice_flag | u(1) |
| if( partition_constraints_override_inter_slice_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| if( intra_parameters_flag ) |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| if( inter_parameters_flag ) |  |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| if( intra_parameters_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| if( inter_parameters_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| … |  |
| } |  |

intra_parameters_flag equal to 1 specifies that parameters for partition constraints override, cu qp delta, and chroma qp offset for intra slices may be present in the PH.  intra_parameters_flag equal to 0 specifies that parameters for partition constraints override, cu qp delta, and chroma qp offset for intra slices are not present in the PH. When not present, the value of intra_parameters_flag is inferred to be equal to 0.

inter_parameters_flag equal to 1 specifies that parameters for partition constraints override, cu qp delta, and chroma qp offset for inter slices may be present in the PH.  inter_parameters_flag equal to 0 specifies that parameters for partition constraints override, cu qp delta, and chroma qp offset for inter slices are not present in the PH. When not present, the value of inter_parameters_flag is inferred to be equal to 1.

# Proposal 2

In the current draft of VVC it is possible to signal SAO parameters in picture header or in slice header (but not in both places for the same picture). It is asserted that it is sufficient to decide on the PPS level whether to use signalling in picture header or in slice header. This provides bit savings since it will not be required to send information at each picture regarding where this information will be present.

The changes are highlighted:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_sao_enabled_present_in_pic_header_flag | u(1) |
| … |  |
| } |  |

pps_sao_enabled_present_in_pic_header_flag equal to 1 specifies that pic_sao_luma_flag and pic_sao_chroma_flag may be present in the PH. pps_sao_enabled_present_in_pic_header_flag equal to 0 specifies that pic_sao_luma_flag and pic_sao_chroma_flag are not present in the PH.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag && pps_sao_enabled_present_in_pic_header_flag) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag && !pic_sao_enabled_present_flag pps_sao_enabled_present_in_pic_header_flag ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| … |  |
| } |  |

# Proposal 3

In the current draft of VVC it is possible to signal ALF parameters in PH or in slice header (but not in both places for the same picture). It is asserted that it is sufficient to decide on the PPS level whether to use signalling in picture header or in slice header. This provides bit savings since it will not be required to send information at each picture regarding where this information will be present.

The changes are highlighted:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_alf_enabled_present_in_pic_header_flag | u(1) |
| … |  |
| } |  |

pps_alf_enabled_present_in_pic_header_flag equal to 1 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, and pic_alf_aps_id_chroma may be present in the PH. pps_alf_enabled_present_in_pic_header_flag equal to 0 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, and pic_alf_aps_id_chroma are not present in the PH.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && pps_alf_enabled_present_in_pic_header_flag) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag pps_alf_enabled_present_in_pic_header_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| … |  |
| } |  |

# Proposal 4

In the current draft of VVC it is possible to signal RPL parameters in PH or in slice header (but not in both places for the same picture). It is asserted that it is sufficient to decide on the PPS level whether to use signalling in picture header or in slice header. This provides bit savings since it will not be required to send information at each picture regarding where this information will be present.

The changes are highlighted:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_rpl_enabled_present_in_pic_header_flag | u(1) |
| … |  |
| } |  |

pps_rpl_enabled_present_in_pic_header_flag equals 1 specifies that reference picture list signalling may be present in the PH. pps_rpl_enabled_present_in_pic_header_flag equals 0 specifies that reference picture list signalling is not present in the PH and may be present in slice headers of slices of the picture.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_rpl_present_flag | u(1) |
| if( pps_rpl_enabled_present_in_pic_header_flag pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pps_rpl_enabled_present_in_pic_header_flag pic_rpl_present_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_sps_flag[ i ] | u(1) |
| if( slice_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && |  |
| ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| slice_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| slice_poc_lsb_lt[ i ][ j ] | u(v) |
| slice_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( slice_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| slice_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( pps_rpl_enabled_present_in_pic_header_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) { |  |
| if( ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

# Proposal 5

In the current draft, the indication of whether TMVP is used or not is provided in the picture header but the information related to which reference picture to use for predicting motion vectors, the collocated picture, is provided in the slice headers even though it has to be the same collocated picture in all slice headers. When RPL information is provided in the picture header it is proposed to indicate the collocated picture information in the picture header.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if( pic_temporal_mvp_enabled_flag && pic_rpl_present_flag) { |  |
| if( rpl1_idx_present_flag && !pps_collocated_from_l0_idc ) |  |
| pic_collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && num_ref_entries[ 0 ][ PicRplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && num_ref_entries[ 1 ][ PicRplsIdx[ 1 ] ] [ 1 ] > 1 ) ) |  |
| pic_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

pic_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. pic_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When pic_collocated_from_l0_flag is not present, the following applies:

- If rpl1_idx_present_flag is equal to 0, the value of pic_collocated_from_l0_flag is inferred to be equal to 1.

- Otherwise (rpl1_idx_present_ is equal to 1), the value of pic_collocated_from_l0_flag is inferred to be equal to pps_collocated_from_l0_idc − 1.

pic_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When pic_collocated_from_l0_flag is equal to 1, pic_collocated_ref_idx refers to a picture in list 0, and the value of pic_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 0 ][ PicRplsIdx[ 0 ] ] − 1, inclusive.

When pic_collocated_from_l0_flag is equal to 0, collocated_ref_idx refers to a picture in list 1, and the value of pic_collocated_ref_idx shall be in the range of 0 to num_ref_entries[ 1 ][ PicRplsIdx[ 1 ] ] − 1, inclusive.

When pic_collocated_ref_idx is not present, the value of pic_collocated_ref_idx is inferred to be equal to 0.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type != I ) { |  |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( !pic_rpl_present_flag && pic_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| slice_collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| slice_collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |
| } |  |

slice_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. slice_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When slice_collocated_from_l0_flag is not present, the following applies:

- If pic_rpl_present_flag is equal to 1, slice_collocated_from_l0_flag is inferred to be equal to pic_collocated_from_l0_flag.

- Otherwise, if slice_type is not equal to B, the value of slice_collocated_from_l0_flag is inferred to be equal to 1.

- Otherwise (slice_type is equal to B), the value of slice_collocated_from_l0_flag is inferred to be equal to pps_collocated_from_l0_idc − 1.

slice_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When slice_type is equal to P or when slice_type is equal to B and slice_collocated_from_l0_flag is equal to 1, slice_collocated_ref_idx refers to a picture in list 0, and the value of slice_collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and slice_collocated_from_l0_flag is equal to 0, slice_collocated_ref_idx refers to a picture in list 1, and the value of slice_collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When slice_collocated_ref_idx is not present the following applies:

- If pic_rpl_present_flag is equal to 1, slice_collocated_ref_idx is inferred to be equal to pic_collocated_ref_idx.

- Otherwise, if, the value of slice_collocated_ref_idx is inferred to be equal to 0.

It is a requirement of bitstream conformance that the picture referred to by slice_collocated_ref_idx shall be the same for all slices of a coded picture.

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by slice_collocated_ref_idx and the current picture shall be the same and RefPicIsScaled[ slice_collocated_from_l0_flag? 0 : 1 ][ slice_collocated_ref_idx  ] shall be equal to 0.

# Summary

Five proposals are proposed to be included in the next draft of VVC to provide improved efficiency and consistent design of the picture header.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).