| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0262 |
| --- | --- |

| Title: | AHG9: On reference picture resampling enabled flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains a proposal for modification of the constraint associated with ref_pic_resampling_enabled_flag. It is asserted that the current constraint is unnecessary strict and prohibits relevant coding structures that do not use Reference Picture Resampling.

# Introduction

In the current draft of VVC, the flag ref_pic_resampling_enabled_flag has been introduced to control if reference picture resampling is used or not. However, the way the current constraint is expressed, it prohibits all kinds of reduced resolution coding, even if there is no inter prediction is applied between pictures of different resolution. The current constraint is as follows:

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

There are two problems with how this is expressed. First, the constraint does not take into account the scaling window, so even when the coded resolution is the same, the two pictures may have different scaling windows. Secondly, the constraint prohibits pic_width_in_luma_samples and pic_height_in_luma_samples to be different from pic_width_max_in_luma_samples and pic_height_max_in_luma_samples, respectively, even if no inter prediction is performed between pictures of different resolution.

# Proposal

It is proposed to use the width and height after scaling window has been applied to express the constraint related to ref_pic_resampling_enabled_flag. This will ensure that reference picture resampling is correctly disabled when ref_pic_resampling_enabled_flag is set equal to 0.

When ref_pic_resampling_enabled_flag is equal to 0, it is a requirement of bitstream conformance that PicOutputWidthL is equal to refPicOutputWidthL and that PicOutputHeightL is equal to refPicOutputHeightL.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).