| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0266-v1 |
| --- | --- |

| Title: | On TPM and GEO merge modes in presence of weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Huanbang Chen, Vasily Rufitskiy, Elena Alshina | Email: | alexey.filippov@huawei.com chenhuanbang@huawei.com vasily.rufitskiy@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

The proposal addresses the current VVC design issue related to how TPM merge mode is used in presence of slice-level weighted prediction. As requested at the previous JVET meeting, examples of subjective artifacts caused by this issue are provided. 3 mechanisms to solve the claimed problem are proposed. The 1st one disables TPM merge mode in such slices where weighted prediction is enabled. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.16%/0.37%/0.33% and 0.40%/0.56%/0.53% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.18%/0.17%/0.13% and 0.31%/0.36%/0.63% in RA and LB cases, respectively. The 2nd one introduces a CU-level conformance constraint to disable the TPM coded block referring to the picture with weighted prediction factor. The 3rd mechanism enables deblocking filtering over the boundaries where blocking artifacts caused by this given design flaw can appear. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.00%/0.02%/-0.03% and -0.03%/-0.25%/0.07% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.08%/ - 0.03%/0.01% and -0.16%/-0.25%/-0.22% in RA and LB cases, respectively. In these tests, VTM-7.1 was used as the anchor. Additionally, 1st and 3rd mechanisms were tested for GEO, using the common base software of Core Experiment 4 as an anchor. Applied to GEO, results for the 1st mechanism could be reportedly summarized as follows. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.27%/0.36/0.48% and 0.96%/0.79%/1.33% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.16%/0.17%/0.32% and 0.72%/0.48%/0.46% in RA and LB cases, respectively. Applied to GEO, the results for 3rd mechanism could be reportedly summarized as follows. For the fade-to-black scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.04%/0.03/-0.03% and 0.03%/-0.18%/0.28% in RA and LB cases, respectively. For the fade-to-white scenario, the simulation results for this method reportedly show the Y/U/V BD-rate changes of 0.05%/0.02%/0.01% and 0.09%/-0.39%/-0.09% in RA and LB cases, respectively.

# Introduction and Problem Statement

As prescribed in [1], slice-level weighted prediction (WP) is not applied to a PU where triangle merge mode (TPM) is used. Hence, a PU is predicted using either WP weighting or TPM merge mode.

For slice-level weighted prediction, each element of the reference picture list that is signalled on the slice level has a pair of weight and offset assigned for the referenced picture. In this case, a block determines weights and offsets for the linear combination with respect to the pair of reference indices that indicate a pair of reference picture list elements, and hence, a set of parameters for linear combination (including weights and offsets). Hence, predicted sample f is determined as follows:

,

wherein pi is a sample of a reference block i, {wi, Oi} are weight and offset specified for an element of ith reference picture list, parameter WD is the bit depth of weights wi.

In TPM, weights applied to the two samples being of the reference blocks depend on the position (x,y) of a predicted sample within a predicted block. In particular, predicted sample p is determined as follows:

,

wherein the sum of weights{,} is constant, i.e. , and the weights’ values are normalized to the weight bitdepth s.

Once triangle merge mode is selected, the explicit weighted prediction is prohibited regardless of the slice-level weighting factor decided by the reference indices, which means that a block level mode may overwrite the slice level parameters. This problem was being discussed at the 16th JVET meeting in Geneva (Switzerland). The following is written in the meeting notes as to this topic: “Further study was suggested, particularly to consider disallowing the combination and to consider whether the other prohibited combinations should be allowed and to consider subjective effects.”

# Subjective artifacts

As requested at the previous JVET meeting, an analysis of subjective effects caused by the current design was performed using the YUView player developed by RWTH Aachen. An exemplified subjective artifact produced by VTM-7.1 encoder is Fig. 1-4. This artifact is observed for the sequence RaceHorses (POC = 22) that is part of class C according to the JVET CTC [2]. This sequence is encoded with LDB configuration settings for QP = 37.The provided example clearly illustrates what subjective artifacts the current design can result in even for elaborate encoders such as VTM-7.1 encoder. This confirms the existence of the problem being discussed and the necessity to solve it.

Fig. 1. General view

Fig. 2. General view with overlaid information

Fig. 3. Detailed view (enlarged picture region)

Fig. 4. Detailed view with overlaid information (enlarged picture region)

# Proposed methods

In this proposal, three alternative methods to resolve the blocking artifact issue are considered:

- Method #1. Disabling TPM on slice level, where WP is enabled;

- Method #2. CU-level conformance conditions; and

- Method #3. Deblocking filter modifications.

The following sections describe the listed methods.

## Method #1. Disabling TPM on slice level, where WP is enabled

In this method, TPM is disabled by inferring the maximum number of candidates to a value that is insufficient for TPM. The following specification draft represents this method (the proposed changes are highlighted by yellow):

Variable SliceMaxNumTriangleMergeCand is used on slice level in accordance with the following formula:

- SliceMaxNumTriangleMergeCand = slice_weighted_pred_flag ? 0 : MaxNumTriangleMergeCand ;

The following specification draft represents this method (the proposed changes are highlighted by yellow):

…

| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| --- | --- |
| slice_weighted_pred_flag | u(1) |
| if (slice_weighted_pred_flag) |  |
| pred_weight_table( ) |  |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |

…

slice_weighted_pred_flag equal to 0 specifies that weighted prediction is not applied to current slice. slice_weighted_pred_flag equal to 1 specifies that weighted prediction is applied to current slice. When not presented, the value of slice_weighted_pred_flag is inferred to 0. Variable SliceMaxNumTriangleMergeCand is defined as follows:

SliceMaxNumTriangleMergeCand = slice_weighted_pred_flag ? 0 : MaxNumTriangleMergeCand.

…

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦ ¦ ( sps_triangle_enabled_flag && SliceMaxNumTriangleMergeCand > 1 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if ( regular_merge_flag[ x0 ][ y0 ] = = 1 ){ |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_triangle_enabled_flag && SliceMaxNumTriangleMergeCand > 1 && weightedPredFlag = = 0 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && SliceMaxNumTriangleMergeCand > 1 ) { |  |
| merge_triangle_split_dir[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx0[ x0 ][ y0 ] | ae(v) |
| merge_triangle_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Method #2. CU-level conformance constraint

In this method, a CU-level conformance constraint is proposed to disable TPM with weighted reference pictures. If the TPM coded block refers to a picture with weighted prediction factor, the bitstream is invalid. In an alternative variant, if the TPM coded block refers to a picture with weighted prediction factor, the explicated weighted prediction is applied for this block, instead of using blending weighted prediction.

Compared to disable TPM for the whole slice, in local illumination change scenario, this method allows the encoder to select TPM mode referring to picture without weighted prediction factor. For example, in a scenario that only the illumination of background is changed, and one weighted reference picture is used for the background area, other un-weighed reference pictures are used for the foreground area. In such case, TPM can be applied to the foreground area for achieving better performance.

The suggested VVC specification changes are as follows:

#### 8.5.4.2 Derivation process for luma motion vectors for merge triangle mode

This process is only invoked when MergeTriangleFlag[ xCb ][ yCb ] is equal to 1, where ( xCb, yCb ) specify the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture.

Inputs to this process are:

- …

Outputs of this process are:

- …

The motion vectors mvA and mvB, the reference indices refIdxA and refIdxB and the prediction list flags predListFlagA and predListFlagB are derived by the following ordered steps:

- …

The varialbe lumaWeightedFlag and chromaWeightedFlag are derived as follow:

lumaWeightedFlagA = predListFlagA ? luma_weight_l1_flag[ refIdxA ] : luma_weight_l0_flag[ refIdxA ]

lumaWeightedFlagB = predListFlagB ? luma_weight_l1_flag[ refIdxB ] : luma_weight_l0_flag[ refIdxB ]

chromaWeightedFlagA = predListFlagA ? chroma_weight_l1_flag[ refIdxA ] : chroma weight_l0_flag[ refIdxA ]

chromaWeightedFlagB = predListFlagB ? chroma_weight_l1_flag[ refIdxB ] : chroma weight_l0_flag[ refIdxB ]

lumaWeightedFlag = lumaWeightedFlagA | | lumaWeightedFlagB

chromaWeightedFlag = chromaWeightedFlagA | | chromaWeightedFlagB

It is a requirement of bitstream conformance that lumaWeightedFlag and chromaWeightedFlag should be equal to 0.

## Method #3. Deblocking filter modifications for TPM

The method consists in modifying boundary strength for the areas that comprise a TPM block and a block that is predicted using WP. The method is shown in Fig. 5.

Fig.5. Deblocking modification

It is worth noting that in contrast to the previous two mechanisms, this technique does not remove the source of the discussed design flaw but alleviates the consequences caused by this problem. The proposed modification in the VVC specification draft is highlighted by yellow:

8.8.3.5	Derivation process of boundary filtering strength

Inputs to this process are:

- a picture sample array recPicture,

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags.

Output of this process is a two-dimensional (nCbW)x(nCbH) array bS specifying the boundary filtering strength.

The variables xDi, yDj, xN and yN are derived as follows:

- The variable gridSize is set as follows:

gridSize = cIdx  = =  0 ? 4 : 8		(1239)

- If edgeType is equal to EDGE_VER,

xDi = ( i * gridSize )		(1240)

yDj = cIdx = = 0  ?  ( j  <<  2 )  :  ( j  <<  1 )		(1241)

xN is set equal to Max( 0, ( nCbW / gridSize ) − 1 )		(1242)

yN = cIdx = = 0  ?  ( nCbH / 4 ) − 1  :  ( nCbH / 2 ) − 1	(1243)

- Otherwise (edgeType is equal to EDGE_HOR),

xDi = cIdx = = 0  ?  ( i  <<  2 )  :  ( i  <<  1 )		(1244)

yDj = ( j * gridSize )		(1245)

xN = cIdx = = 0  ?  ( nCbW / 4 ) − 1  :  ( nCbW / 2 ) − 1	(1246)

yN = Max( 0, ( nCbH / gridSize ) − 1 )		(1247)

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

- The sample values p0 and q0 are derived as follows:

    - If edgeType is equal to EDGE_VER, p0 is set equal to recPicture[ xCb + xDi − 1 ][ yCb + yDj ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

    - Otherwise (edgeType is equal to EDGE_HOR), p0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj − 1 ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples.

      - weightedPredFlag is set to 1 for both coding subblocks: for coding subblock containing the sample p0 and for coding subblock containing the sample q0, and one of these coding subblocks is predicted using a triangular merge mode.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

Fig. 6 and 7 reportedly illustrate how method #3 impacts the visual artefact mentioned in Section 2.

Fig. 6. General view after applying method #3

Fig. 7. Detailed view after applying method #3 (enlarged picture region)

## Method #4. Disabling GEO on slice level, where WP is enabled

In this method, GEO is disabled by inferring the maximum number of candidates to a value that is insufficient for GEO. Variable SliceMaxNumGeoMergeCand is used on slice level in accordance with the following formula:

- SliceMaxNumGeoMergeCand = (lumaWeightedFlag || chromaWeightedFlag) ? 0 : MaxNumGeoMergeCand.

The following specification draft represents this method (the proposed changes in comparison with [6] are highlighted by yellow):

…

| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| --- | --- |
| slice_weighted_pred_flag | u(1) |
| if (slice_weighted_pred_flag) |  |
| pred_weight_table( ) |  |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |

…

slice_weighted_pred_flag equal to 0 specifies that weighted prediction is not applied to current slice. slice_weighted_pred_flag equal to 1 specifies that weighted prediction is applied to current slice. When not presented, the value of slice_weighted_pred_flag is inferred to 0. Variable SliceMaxNumGeoMergeCand is defined as follows:

SliceMaxNumGeoMergeCand = slice_weighted_pred_flag ? 0 : MaxNumGeoMergeCand.

…

7.3.9.7	Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) ¦ ¦ ( sps_geo_enabled_flag && SliceMaxNumGeoMergeCand > 1 && cbWidth>=8 && cbHeight >=8 && slice_type = = B ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( regular_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_geo_enabled_flag && SliceMaxNumGeoMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth >= 8 && cbHeight >= 8 && cbWidth < 128 && cbHeight < 128 ) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && SliceMaxNumGeoMergeCand > 1 ) { |  |
| merge_geo_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( SliceMaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

Related picture header semantics is as follows:

pic_max_num_merge_cand_minus_max_num_geo_cand specifies the maximum number of geo merge mode candidates supported in the slices associated with the pictur header subtracted from MaxNumMergeCand.

When pic_max_num_merge_cand_minus_max_num_geo_cand is not present, and sps_geo_enabled_flag is equal to 1 and MaxNumMergeCand greater than or equal to 2, pic_max_num_merge_cand_minus_max_num_geo_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_geo_cand_plus1 − 1.

The maximum number of geo merge mode candidates, MaxNumGeoMergeCand is derived as follows:

MaxNumGeoMergeCand = 
	MaxNumMergeCand − pic_max_num_merge_cand_minus_max_num_geo_cand	(87)

When pic_max_num_merge_cand_minus_max_num_geo_cand is present, the value of MaxNumGeoMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive.

When pic_max_num_merge_cand_minus_max_num_geo_cand is not present, and (sps_geo_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumGeoMergeCand is set equal to 0.

When MaxNumGeoMergeCand is equal to 0, geo merge mode is not allowed for the slices associated with the PH.

## Method #3. Deblocking filter modifications for GEO

The method consists in modifying boundary strength for the areas that comprise a GEO block and a block that is predicted using WP. The method is shown in Fig. 8.

Fig. 8. Deblocking modification

It is worth noting that this technique does not remove the source of the discussed design flaw but alleviates the consequences caused by this problem.

The proposed modification in the VVC specification draft is highlighted by yellow:

8.8.3.5	Derivation process of boundary filtering strength

…

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

- The sample values p0 and q0 are derived as follows:

    - If edgeType is equal to EDGE_VER, p0 is set equal to recPicture[ xCb + xDi − 1 ][ yCb + yDj ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

    - Otherwise (edgeType is equal to EDGE_HOR), p0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj − 1 ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples.

      - weightedPredFlag is set to 1 for both coding subblocks: for coding subblock containing the sample p0 and for coding subblock containing the sample q0, and one of these coding subblocks is predicted using a geo merge mode.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

…

# Experimental result

All the tests are based on the VTM7.1 software. The further part of this section describes test conditions specified for this test and reports test results for the tests.

## Test Conditions

In our experiments, we use same conditions that was used for fade-in fade-out simulations in AVC [3], HEVC [4] and VVC [5]: a linear fade is applied to the first 2 seconds of the regular sequence set and the experiments are done with the 2-second sequences. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half (Figure 2). The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs.

In order to simplify simulations and avoid redefining configuration parameters of input CTC sequences, weighting of original and setting up then number of coded frames is implemented within proposed modifications to the VTM software.

Current test condition is designed for global illumination change, and cannot reflect the benefit of method #2, which is designed for the local illumination change scenario. Therefore, the simulation for method #2 is not conducted. We believe that method 2 have potential benefits in allowing TPM in such scenario.

Figure 2: Linear Fade-to-black or Fade-to-white weights applied on the sequence and the black/white pictures respectively [3].

## Test description

Test descriptions are summarized in Table 1. The results for Tests 1-5 are reportedly shown in Tables 2-11 for RA and LDB configurations as well as for the two fading scenarios. In both scenarios a frame of original picture is linearly blended with the “all-black” (scenario 1, Tables 2-6) or “all-white” (scenario 2, Tables 7-11) frame.

Table 1. Tests description

| Test number | Test | Anchor | Test conditions |
| --- | --- | --- | --- |
| Test 1 | WP tool-off test | Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 | About of 2-seconds length, fadein/fadeout sequences, Code base is VTM-7.1 |
| Test 2 | TPM tool-off test Proposed method #1: Slice-level TPM disabling Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 --Triangle=0 |  |  |
| Test 3 | Proposed method #3: Deblocking modification Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 |  |  |
| Test 4 | GEO tool-off test Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 --GEO=0 | Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 | About of 2-seconds length, fadein/fadeout sequences, Code base is CE4 common base (9d2db2e) |
| Test 5 | Proposed method #3 for Geo: Deblocking modification Weighted prediction is on: --WeightedPredB=1 --WeightedPredP=1 |  |  |

Table 2. Simulation results for Test 1 on fade-to-black sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.24% | 3.30% | 2.35% | 54% | 111% |
| Class A2 | 8.77% | 13.13% | 14.77% | 63% | 111% |
| Class B | 9.67% | 15.72% | 14.28% | 67% | 113% |
| Class C | 8.54% | 11.16% | 11.91% | 76% | 112% |
| Class E |  |  |  |  |  |
| Overall | 7.70% | 11.50% | 11.36% | 66% | 112% |
| Class D | 8.27% | 10.95% | 14.04% | 83% | 108% |
| Class F | 10.71% | 8.84% | 8.31% | 79% | 115% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 36.22% | 64.00% | 66.47% | 85% | 114% |
| Class C | 29.52% | 45.47% | 39.55% | 92% | 110% |
| Class E | 108.42% | 158.23% | 142.90% | 119% | 125% |
| Overall | 52.04% | 81.38% | 76.60% | 95% | 115% |
| Class D | 23.31% | 39.24% | 42.44% | 106% | 106% |
| Class F | 82.02% | 79.59% | 86.81% | 122% | 118% |

Table 3. Simulation results for Test 2 on fade-to-black sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | 0.44% | 0.19% | 100% | 100% |
| Class A2 | 0.25% | 0.27% | 0.25% | 99% | 100% |
| Class B | 0.09% | 0.33% | 0.11% | 99% | 101% |
| Class C | 0.44% | 0.46% | 0.76% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.37% | 0.33% | 99% | 100% |
| Class D | 0.07% | -0.09% | 0.51% | 99% | 100% |
| Class F | 0.18% | 0.15% | 0.39% | 99% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.31% | 0.32% | 0.17% | 100% | 101% |
| Class C | 0.50% | 0.96% | 0.88% | 99% | 101% |
| Class E | 0.42% | 0.44% | 0.67% | 99% | 100% |
| Overall | 0.40% | 0.56% | 0.53% | 99% | 101% |
| Class D | 0.33% | 1.25% | -0.02% | 98% | 99% |
| Class F | 0.77% | 0.93% | 0.44% | 99% | 100% |

Table 4. Simulation results for Test 3 on fade-to-black sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.06% | 0.06% | 101% | 101% |
| Class A2 | 0.05% | 0.08% | -0.03% | 101% | 101% |
| Class B | -0.03% | 0.02% | -0.04% | 101% | 102% |
| Class C | 0.01% | 0.00% | -0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | -0.05% | 100% | 101% |
| Class D | -0.01% | -0.02% | 0.01% | 100% | 99% |
| Class F | -0.01% | 0.02% | -0.05% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.14% | 0.12% | 101% | 102% |
| Class C | -0.12% | -0.03% | -0.34% | 100% | 102% |
| Class E | -0.03% | -0.51% | -0.08% | 100% | 102% |
| Overall | -0.03% | -0.08% | -0.08% | 101% | 102% |
| Class D | -0.03% | 0.53% | -0.22% | 100% | 100% |
| Class F | 0.08% | 0.60% | 0.65% | 100% | 101% |

Table 5. Simulation results for Test 4 on fade-to-black sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | 0.23% | 0.25% | 98% | 100% |
| Class A2 | 0.15% | 0.25% | 0.22% | 98% | 100% |
| Class B | 0.19% | 0.02% | 0.22% | 97% | 101% |
| Class C | 0.83% | 0.94% | 1.16% | 97% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.27% | 0.36% | 0.48% | 97% | 101% |
| Class D | 0.15% | -0.19% | 0.33% | 96% | 100% |
| Class F | 0.23% | 0.26% | 0.11% | 98% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.68% | 0.38% | 1.00% | 97% | 101% |
| Class C | 1.11% | 1.40% | 2.34% | 97% | 101% |
| Class E | 1.24% | 0.65% | 0.54% | 98% | 101% |
| Overall | 0.96% | 0.79% | 1.33% | 97% | 101% |
| Class D | 0.67% | 0.88% | 1.76% | 96% | 101% |
| Class F | 0.41% | 0.62% | 0.23% | 98% | 101% |

Table 6. Simulation results for Test 5 on fade-to-black sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.05% | -0.06% | 100% | 100% |
| Class A2 | -0.02% | -0.04% | -0.02% | 100% | 100% |
| Class B | 0.01% | 0.04% | -0.01% | 100% | 101% |
| Class C | -0.01% | 0.05% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.03% | -0.03% | 100% | 100% |
| Class D | 0.01% | -0.02% | 0.00% | 100% | 100% |
| Class F | -0.01% | 0.04% | -0.02% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.21% | -0.10% | 100% | 100% |
| Class C | 0.05% | -0.02% | 0.51% | 100% | 100% |
| Class E | -0.04% | -0.34% | 0.61% | 100% | 101% |
| Overall | 0.03% | -0.18% | 0.28% | 100% | 100% |
| Class D | -0.01% | -0.31% | 1.74% | 100% | 100% |
| Class F | 0.02% | -0.29% | -0.33% | 100% | 101% |

Table 7. Simulation results for Test 1 on fade-to-white sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5.46% | 7.93% | 7.74% | 57% | 115% |
| Class A2 | 11.58% | 16.87% | 17.23% | 67% | 111% |
| Class B | 11.50% | 17.42% | 16.38% | 72% | 113% |
| Class C | 10.04% | 13.10% | 14.66% | 83% | 113% |
| Class E |  |  |  |  |  |
| Overall | 9.92% | 14.26% | 14.37% | 70% | 113% |
| Class D | 8.60% | 11.65% | 14.63% | 89% | 106% |
| Class F | 7.41% | 4.94% | 4.97% | 83% | 112% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 37.12% | 61.21% | 65.47% | 97% | 111% |
| Class C | 32.12% | 48.28% | 41.57% | 105% | 108% |
| Class E | 114.90% | 154.68% | 149.39% | 132% | 126% |
| Overall | 54.90% | 80.26% | 78.49% | 107% | 114% |
| Class D | 23.91% | 39.61% | 40.68% | 119% | 106% |
| Class F | 76.32% | 75.91% | 86.23% | 131% | 118% |

Table 8. Simulation results for Test 2 on fade-to-white sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.29% | 0.29% | 99% | 100% |
| Class A2 | 0.08% | 0.02% | 0.11% | 99% | 100% |
| Class B | 0.20% | -0.07% | -0.07% | 99% | 101% |
| Class C | 0.38% | 0.50% | 0.26% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.18% | 0.17% | 0.13% | 99% | 101% |
| Class D | -0.05% | -0.23% | 0.05% | 99% | 99% |
| Class F | 0.26% | -0.04% | 0.65% | 100% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.36% | 0.13% | 0.30% | 99% | 101% |
| Class C | 0.46% | 0.64% | 1.00% | 99% | 101% |
| Class E | 0.04% | 0.36% | 0.67% | 99% | 101% |
| Overall | 0.31% | 0.36% | 0.63% | 99% | 101% |
| Class D | 0.25% | 0.67% | 0.04% | 98% | 100% |
| Class F | 0.34% | 1.09% | 0.75% | 99% | 100% |

Table 9. Simulation results for Test 3 on fade-to-white sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.01% | -0.01% | 101% | 101% |
| Class A2 | 0.47% | -0.02% | -0.08% | 101% | 101% |
| Class B | -0.02% | -0.08% | -0.01% | 101% | 102% |
| Class C | 0.01% | 0.06% | -0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | -0.02% | -0.03% | 100% | 101% |
| Class D | 0.00% | 0.04% | 0.05% | 100% | 98% |
| Class F | 0.00% | 0.00% | 0.02% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.1 fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.02% | 0.33% | 101% | 102% |
| Class C | -0.04% | -0.22% | 0.16% | 100% | 101% |
| Class E | -0.82% | 0.09% | -0.06% | 100% | 101% |
| Overall | -0.20% | -0.05% | 0.18% | 101% | 102% |
| Class D | 0.02% | 0.39% | 0.37% | 100% | 99% |
| Class F | -0.07% | -0.06% | 0.16% | 100% | 101% |

Table 10. Simulation results for Test 4 on fade-to-white sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | 0.12% | 0.32% | 98% | 100% |
| Class A2 | -0.11% | 0.18% | 0.17% | 97% | 100% |
| Class B | 0.13% | -0.10% | 0.01% | 97% | 100% |
| Class C | 0.70% | 0.54% | 0.83% | 96% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.17% | 0.32% | 97% | 100% |
| Class D | 0.10% | 0.15% | 0.13% | 96% | 100% |
| Class F | 0.18% | 0.24% | 0.45% | 98% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.56% | 0.19% | 0.20% | 97% | 100% |
| Class C | 0.96% | 1.52% | 0.98% | 95% | 100% |
| Class E | 0.66% | -0.45% | 0.20% | 97% | 98% |
| Overall | 0.72% | 0.48% | 0.46% | 96% | 100% |
| Class D | 0.35% | 0.77% | 0.17% | 95% | 100% |
| Class F | 0.14% | 0.17% | 0.50% | 97% | 99% |

Table 11. Simulation results for Test 5 on fade-to-white sequences

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.00% | 100% | 100% |
| Class A2 | 0.23% | -0.02% | -0.01% | 100% | 100% |
| Class B | -0.02% | 0.09% | 0.02% | 100% | 100% |
| Class C | 0.02% | -0.04% | 0.01% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.02% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Class F | 0.00% | -0.02% | -0.05% | 100% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over CE4 base (VTM7.0), fade 2 sec |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | -0.25% | -0.07% | 100% | 101% |
| Class C | -0.05% | -0.04% | 0.40% | 100% | 99% |
| Class E | 0.21% | -1.09% | -0.77% | 100% | 100% |
| Overall | 0.09% | -0.39% | -0.09% | 100% | 100% |
| Class D | -0.07% | -0.52% | -0.30% | 100% | 100% |
| Class F | -0.23% | -0.04% | -0.08% | 100% | 99% |

# Conclusion

In this contribution, the problem caused by the current VVC design issue related to how TPM merge mode is used in presence of slice-level weighted prediction is reportedly identified. Examples of observed visual artifacts are provided. 3 straightforward methods to remedy this issue are proposed. We recommend to adopt one of these techniques into the VVC specification draft and the VTM software.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th JVET meeting, Geneva, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

- Y. Kikuchi, T. Chujoh,” Simplification of the Weighted Prediction and Verification result,” JVT Document, JVT-F077, Awaji, December, 2002.

- A.Fujibayashi, S.Kanumuri, F.Bossen,” TE12: Performance of Partition Based Illumination Compensation (PBIC),” JVT Document, JCTVC-C041, Guangzhou, October, 2010.

- P. Bordes, E. François, F. Léannec “AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences”, JVET-L0201, 12th JVET Meeting, Macao, China, October 2018.

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, K. Reuze, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz, R.-L. Liao, J. Chen, Y. Ye, J. Luo, M. Blaser, J. Sauer, “CE4 Common Base: Geometric inter prediction”, JVET-Q0079, 17th JVET meeting, Brussels, January 2020.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).