| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0267-v1 |
| --- | --- |

| Title: | AHG16: On Propagation of Chroma CU QP Offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Minhua Zhou Wade Wan | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

After the adoption of JVET-P1001, the chroma QP now affects the deblocking filter process. Therefore, chroma QP values must be determined, even for non-coded TUs. In particular, this includes the CU-level chroma QP offsets introduced in JVET-O1168.

In non-coded TUs, CU-level chroma QP offsets can currently propagate across tiles boundaries and across entropy-coding sync points, because no process has been put in place to reset the CU-level chroma offsets until the start of the next slice. It is asserted that this cross tile / entropy-sync dependency was never intended, and that this problem needs to be addressed.

This contribution proposes to solve the problem by limiting chroma QP offset propagation to within the current chroma quantization group. Specifically, it proposes resetting CuQpOffsets to 0 at the start of each new group.

# Introduction

In the current draft text, CU-level chroma QP offsets (CuQpOffsetCb, CuQpOffsetCr, CuQpOffsetCbCr) are only reset to 0 at the start of a new slice. Specifically, they are reset in the following section of the JVET-Q0041 draft text.

#### 7.4.8.1	General slice header semantics

“The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr, specifying values to be used when determining the respective values of the Qp′Cb, Qp′Cr, and Qp′CbCr quantization parameters for the coding unit containing cu_chroma_qp_offset_flag, are all set equal to 0.”

When a TU is coded and the cu_chroma_qp_offset_flag is present, the CuQpOffsets are either explicitly updated (when cu_chroma_qp_offset_flag=1) or they are set to 0 (when cu_chroma_qp_offset_flag=0). Details of this process are specified in Section 7.4.10.10 of the draft text, provided below.

#### 7.4.10.10		Transform unit semantics

When cu_chroma_qp_offset_flag is present, the following applies:

- The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are derived as follows:

- If cu_chroma_qp_offset_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ cu_chroma_qp_offset_idx ]	(186)

CuQpOffsetCr = cr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(187)

CuQpOffsetCbCr = joint_cbcr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(188)

- Otherwise (cu_chroma_qp_offset_flag is equal to 0), CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are all set equal to 0.

When the cu_chroma_qp_offset_flag is not present (e.g. in transform units without coded chroma coefficients), nothing is changed, and the previous CuQpOffset values remain in effect.

Based on this definition, CuQpOffset values will propagate outside the chroma quantization group, wrap around from the right-edge of the picture back to the left, extend across entropy-coding sync points, and across tile boundaries. As mentioned, there is no additional process defined to reset these values until the start of the next slice.

Once outside the current chroma quantization group, propagated CU-level chroma QP offsets can only occur in non-coded TUs, because the first coded TU in each new chroma quantization group must explicitly code a new value. Therefore, this propagation used to be irrelevant because the QP offset assigned to a non-coded TU has no effect on the decoded residual.

However, with the adoption of JVET-P1001, the chroma QP now affects the deblocking filter. Therefore, in order to properly filter each CU, the actual chroma QP must be determined, even for non-coded TUs.

It is asserted that this cross-tile / entropy-sync dependency was an unintended side-effect of the JVET-P1001 adoption, and that this dependency should be removed by modifying the CU-level chroma QP offset derivation process.

# Proposal

This contribution proposes to reset CuQpOffset values (Cb/Cr/CbCr) to 0 at the start of each chroma quantization group. This is equivalent to how the luma delta QP value (CuQpDeltaVal) is reset at the start of each luma quantization group. Specifically, in the two places where ‘IsCuChromaQpOffsetCoded’ is reset to 0, the CuQpOffset values should be reset to 0 as well. Proposed changes to the draft text are highlighted red in the following tables.

Section 7.3.9.2

…

if( cu_chroma_qp_offset_enabled_flag && cbSubdiv <= CuChromaQpOffsetSubdiv ) {

IsCuChromaQpOffsetCoded = 0

CuQpOffsetCb = 0

CuQpOffsetCr = 0

CuQpOffsetCbCr = 0

}

…

Section 7.3.9.4

…

if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= CuChromaQpOffsetSubdiv ) {

IsCuChromaQpOffsetCoded = 0

CuQpOffsetCb = 0

CuQpOffsetCr = 0

CuQpOffsetCbCr = 0

}

…

It is asserted that adopting this change will remove the problematic cross tile / entropy-sync dependencies by preventing chroma QP offset propagation beyond the current chroma quantization group.

It is also asserted that this proposal is consistent with the existing handling of CuQpOffset. In particular, note that cu_chroma_qp_offset_flag equal to 0 does not indicate that the decoder should reuse some previous value; it indicates that the CuQpOffset values are set to 0. Essentially, the “default” offset in the current design is 0, without any propagation from previous groups.

# VTM Software Changes

3.1  Decoder Software

The changes required to the VTM decoder software are shown below. Specifically the following 2 lines need to be added in CABACReader.cpp, coding_tree( ) function.

if( cs.slice->getUseChromaQpAdj() && partitioner.currQgChromaEnable() )

{

cuCtx.isChromaQpAdjCoded  = false;

cs.chromaQpAdj = 0;

}

if (cs.slice->getUseChromaQpAdj() && pPartitionerChroma->currQgChromaEnable())

{

pCuCtxChroma->isChromaQpAdjCoded = false;

cs.chromaQpAdj = 0;

}

3.1  Encoder Software

Hypothetically, the encoder software would require a one line change to reset the chroma QP offset to 0 (rather than the previous offset value) for any CUs without chroma residuals at the start of a chroma quantization group:

cu.chromaQpAdj = 0;

However, as described in the Appendix at the end of this contribution, the CU-level chroma QP offset feature does not work in the current VTM software. If and when the existing issues are solved, resetting the offset to 0 rather than the previous value is a one line change.

# Conclusion

The propagation of CU-level chroma QP offsets currently creates a problematic cross-tile / cross-entropy-sync dependency. It is believed it was never intended to work the way it is currently defined in the draft text, and the software does not properly support that behavior either. Therefore, something needs to be done to address this problem. It can be solved using the modification proposed above, or through some equivalent means.

# References

[1] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

[2] Y. Han, A. Ramasubramonian, A. Nalci, G. Van Der Auweera, M. Coban, W. Chien, Y. Chao, M. Karczewicz, P. de Legrange, “Cu level chroma QP control for VVC”, document JVET-O1168, JVET, Gothenburg, Sweden, July 2019.

[3] A. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina, “Non-CE5:  Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539)”, document JVET-P1001, JVET, Geneva, Switzerland, October 2019.

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix – VTM Software Issues

The VTM software currently has the following issues with CU chroma QP offsets:

- JVET Bug Tracker #785

CU-level chroma QP offsets are arbitrarily assigned to square-shaped regions by cycling through the offset lists in round-robin fashion. These square shaped regions do not correspond to the rectangular shaped chroma quantization groups used in VVC, so this introduces encoder/decoder mismatches. This was never made to work correctly when the feature was adopted in JVET-O1168.

- JVET Bug Tracker #786

In order to use chroma QP in deblocking, both the encoder and decoder need to be using the same chroma QP. In particular, when the encoder codes a CU with no chroma residual, it cannot send any chroma QP offset. In these cases, it needs to reset the chroma QP offset for those CUs.

In order to match the current text, processes would have to be put in place to check for non-coded CUs at the start of a chroma quantization group, to track the previous chroma QP offset, (including across tiles, etc…), and to reset the chroma QP offset to this value for these non-coded TUs.

This logic was never added when JVET-P1001 was implemented. Therefore, enabling this feature causes additional encoder / decoder mismatches due to using the wrong chroma QP in deblocking.

The attached source code makes an attempt to resolve the mismatches caused be the two issues above.  The majority of the changes found there are related to fixing these two issues. As mentioned in Section 3, the code actually related to the current contribution, not bug fixes, consists of only 2 lines in the decoder and 1 line in the encoder.