| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, 6–17 January 2020 | Document: JVET-Q0268 |
| --- | --- |

| Title: | CE4-related: GEO with 32 modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kevin Reuzé Han Huang Wei-Jung Chien Vadim Seregin Marta Karczewicz | Tel: Email: | kreuz@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, the number of modes for geometric partition mode (GEO) is reduced to 32 (from 82 in the CE4 common base software). The selected GEO modes depend on the width/height ratio of the CU using GEO. It is claimed that this reduces the number of GEO modes while maintaining the gains GEO offers.

Compares to the CE4 common-base software, this test provides 0.00% average luma BD-rate difference in RA configuration and 0.04% in LDB configuration. When combined with CE4-2 (size restriction of GEO), the average luma BD-rate difference in RA configuration is 0.00%, and the average luma BD-rate difference in LDB configuration is 0.05%.

# Introduction

The geometric partition mode (GEO) was introduced during the Gothenburg meeting in JVET-O0489. The original design allowed 140 GEO modes for each partition that could use GEO. It was mentioned in the track that 140 modes were too many, and the common base for the CE study of GEO contained a reduced number of 82 modes. AHG16 mentioned that 82 modes may still be too many and the CE4 contains a test where GEO has 64 modes (CE4-1).

Th reasons invoked for asking to reduce the number of GEO modes were that

- The conformance check will need to test many combinations (19 CU sizes times 82 GEO modes)

- Some encoder may not afford to test that many modes

Therefore, this contribution proposes to reduce the number of modes allowed for GEO to 32 modes. To keep most of the gains intact, it is proposed to select the 32 modes depending on the CU width/ height ratio.

# Proposal

Each CU ratio will be allowed to use 32 different GEO modes. The selected partitions follow the following rules, depending on the value of the CU width/height ratio:

- Allow all partitions where the angle’s tangent is the same as the CU width/height ratio (i.e. former TPM angles with all displacements)

- Allow partitions with an angle neighbouring the previous condition, with min and max displacements (displacement 0 and 3) in both directions.

- If width/height > 1, allow the perfect horizontal angle with displacement 1 and 3 in both directions, and the perfect vertical angle with displacement 3 in both directions.
Otherwise (if width/height <= 1), allow the perfect vertical angle with displacement 1 and 3 in both directions, and the perfect horizontal angle with displacement 3 in both directions.

Therefore, the following modes can be used for each CU ratio:

Figure 1a: Example of allowed partitions for CU with width/height=1

Figure 1b: Example of allowed partitions for CU with width/height=2

Figure 1c: Example of allowed partitions for CU with width/height=4

Figure 1d: Example of allowed partitions for CU with width/height=8

In each example a x2 denotes where the same mode is also available in inverted direction (i.e. symmetric along vertical axis).

The modes allowed for CUs with width/height < 1 are the same as the ones presented in this Figure, rotated 90°.

Note that to allow TPM-like angles for CU with width/height = 8 or 1/8 (CU of size 8x64 and 64x8), the angles with ratio 1:8, 8:1, -1:8 and -8:1 are included. The total number of different modes that can be used is 106, but only a subset of 32 modes is allowed for a given CU.

# Simulation Results

## Results for proposed change

When the proposed method is implemented on top of GEO common base, the results over CE4 common base software is shown in Table 1.

Table 1

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.09% | 0.05% | 98% | 100% |
| Class A2 | -0.02% | 0.13% | -0.06% | 98% | 99% |
| Class B | -0.01% | -0.02% | -0.01% | 98% | 100% |
| Class C | 0.02% | 0.17% | 0.05% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.01% | 98% | 100% |
| Class D | 0.05% | 0.17% | 0.13% | 96% | 96% |
| Class F (optional) | 0.02% | 0.07% | 0.01% | 98% | 100% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.18% | 0.07% | 97% | 101% |
| Class C | 0.07% | 0.34% | 0.10% | 96% | 100% |
| Class E | 0.13% | -0.16% | 0.05% | 98% | 103% |
| Overall | 0.04% | 0.00% | 0.07% | 97% | 101% |
| Class D | 0.07% | -0.17% | 0.06% | 95% | 97% |
| Class F (optional) | 0.05% | 0.24% | -0.10% | 98% | 100% |

When combined with CE4-2, the results over common base software is shown in Table 2.

Table 2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4 common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | -0.03% | 95% | 96% |
| Class A2 | -0.02% | -0.03% | -0.11% | 94% | 95% |
| Class B | -0.01% | -0.02% | 0.02% | 94% | 96% |
| Class C | 0.01% | 0.13% | 0.11% | 94% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.01% | 95% | 96% |
| Class D | 0.01% | 0.11% | 0.13% | 84% | 100% |
| Class F | 0.02% | 0.01% | 0.00% | 95% | 94% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over CE4 common base |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.18% | 94% | 97% |
| Class C | 0.00% | 0.04% | -0.01% | 94% | 99% |
| Class E | 0.18% | -0.03% | -0.22% | 94% | 98% |
| Overall | 0.05% | 0.01% | 0.01% | 94% | 98% |
| Class D | 0.00% | -0.60% | 0.29% | 87% | 103% |
| Class F | -0.02% | 0.34% | 0.14% | 94% | 93% |

# Conclusion

This contribution addresses the issue regarding the number of GEO modes that were raised during the plenary and by the AHG16 in the JVET reflector. It proposes to reduce the number of GEO modes to 32, and make the allowed modes depend on the CU ratio, to maintain as much GEO gain as possible. As the BD-rate losses are small when comparing to the CE4 common base anchor, it is proposed to adopt this simplification of GEO.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).