| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0269-v3 |
| --- | --- |

| Title: | CE3-related: On transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Yan Ye, Jiancong Luo 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes two aspects of transform skip (TS) residual coding. In the first aspect, a rice parameter derivation method is proposed to code abs_remainder of the TS residual coding. The proposed rice-parameter derivation process is simpler than CE3-1.1.c/d and does not require any look-up table.  Following results of aspect 1 are reported as compared to VTM-7.0.

- Lossless test conditions:

- Overall: -4.32% (AI), -2.60% (RA), -1.98% (LB) bit rate saving

- Class TGM: -5.20% (AI), -6.68% (RA), -6.92% (LB) bit rate saving

- CTC test conditions:

- Overall: -0.01% (AI), 0.00% (RA), -0.01% (LB) luma BD-rate

- Class TGM: -0.33% (AI), -0.10% (RA), -0.12% (LB) luma BD-rate

- Low QP test conditions:

- Overall: -0.07% (AI), -0.02% (RA), -0.02% (LB) luma BD-rate

- Class TGM: -1.97% (AI), -2.54% (RA), -1.48% (LB) luma BD-rate

In the second aspect, context modelling for chroma transform skip residual coding is proposed. 7 additional context variables (as compared to VTM-7.0) are introduced to code sig_coeff_flag and greater than 1 flag of the residual coding of chroma TS mode. Following are the combined results of aspect 1 and 2.

- Lossless test conditions:

- Overall: -4.50% (AI), -3.08% (RA), -2.75% (LB) bit rate saving

- Class TGM: -5.35% (AI), -6.94% (RA), -7.08% (LB) bit rate saving

- CTC test conditions:

- Overall: -0.01% (AI), 0.01% (RA), -0.02% (LB) luma BD-rate

- Class TGM: -0.35% (AI), -0.21% (RA), -0.14% (LB) luma BD-rate

- Low QP test conditions:

- Overall: -0.09% (AI), -0.02% (RA), -0.01% (LB) luma BD-rate

- Class TGM: -2.07% (AI), -2.46% (RA), -1.52% (LB) luma BD-rate

# Introduction

In VVC 7, the transform skip (TS) residual levels of a coefficient group (CG) are coded in three passes over the scan positions as follows:

- Pass 1: context coding

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - parity (par_level_flag) flag

- Pass 2: context coding

  - greater than 3 flag (abs_level_gtx_flag[1])

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: by-pass coding: Rice-Golomb coding of remaining absolute level (abs_remainder)

The syntax element abs_remainder is binarized with a fixed rice parameter ( cRiceParam = 1) value [1]. In Geneva meeting, chroma TS mode is adopted, and luma and chroma shares the same context variables in VTM-7.0.

# Proposed methods

In this contribution, two aspects of TS residual coding are proposed. The first aspect is related to the rice parameter derivation of abs_remainder coding and the second aspect is related to the context modelling of chroma transform skip mode.

## Aspect 1: rice parameter derivation

In this aspect, the rice parameter is derived from the sum of absolute values of the top and left neighboring levels. The proposed rice-parameter derivation is simpler than the CE3-1.1.c/d and do not require any look-up table.

Assume the variable locSumAbs is the sum of absolute values of top and left levels. The rice parameter is derived as follows:

- locSumAbs = min(locSumAbs, 31)

- cRiceParam = (locSumAbs + 4) >> 3

## Aspect 2: Context modelling of TS residual coding

In VVC7, transform skip mode is allowed for both luma and chroma color components and both of them share same context variables. However, the signal statistics of luma and chroma blocks may be different. Therefore, in this contribution, it is proposed to use different context variables for luma and chroma color components to code sig_coeff_flag and greater than 1 (abs_level_gtx_flag[n][0]) flag.

### Context modelling of sig_coeff_flag

In the proposed method, 3 additional context variables (as compared to VTM-7.0) are allocated for sig_coeff_flag of the chroma component. Similar to VVC7, the context index for coding of sig_coeff_flag of transform skip mode is computed from the number of significant coefficients of top and left neighbors.

Given the syntax elements sig_coeff_flag[ x ][ y ] with component index cIdx and the top-left luma location ( x0, y0 ), the variables locNumSig are derived as specified by the following pseudo code:

locNumSig = 0
if( xC > 0 ) {
		locNumSig          += sig_coeff_flag[ xC − 1 ][ yC ]
		}
if( yC > 0 ) {
		locNumSig          += sig_coeff_flag[ xC ][ yC − 1 ] 
		}

If cIdx is equal to 0 ( i.e. luma component), ctxInc is derive as follows:

ctxInc = locNumSig

Otherwise ( i.e. chroma component), ctxInc is derive as follows:

ctxInc = locNumSig + 3

### Context modelling of gt1 flag

In VVC7, 4 context variables are used to code gt1 flag. In the proposed method, total 8 context variables (4 for luma and 4 for chroma) are used to code abs_level_gtx_flag[ n ][ 0 ] of the transform skip mode.

The context index is derived as follows.

- If BdpcmFlag[ x0 ][ y0 ][ cIdx ] is equal to 1, ctxInc is derived as follows:

ctxInc = 3

- Otherwise, if xC is greater than 0 and yC is greater than 0, ctxInc is derived as follows:

ctxInc = sig_coeff_flag[ xC − 1 ][ yC ] + sig_coeff_flag[ xC ][ yC − 1 ]

- Otherwise, if xC is greater than 0, ctxInc is derived as follows:

ctxInc = sig_coeff_flag[ xC − 1 ][ yC ]

- Otherwise, if yC is greater than 0, ctxInc is derived as follows:

ctxInc = sig_coeff_flag[ xC ][ yC − 1 ]

- Otherwise, ctxInc is derived as follows:

ctxInc = 0

- If cIdx is greater than 0, ctxInc is incremented as:

ctxInc = ctxInc + 4

# Simulation results

The proposed methods are implemented on top of VTM-7.0. Following tests are conducted.

- Test 1: Aspect 1 (rice parameter derivation)

- Test 2: Aspect 2 (context modelling of TS residual coding, 7 additional contexts as compared to VTM-7.0)

- Test 3: Test 1 + Test 2

Each test is evaluated under lossless, CTC and low QP test conditions. The detail of the test conditions are described in CE3 [2].

Table 1, Table 2 and Table 3 show the results of test1 under lossless, CTC and low QP test settings, respectively. Table 4, Table 5 and Table 6 show the results of test 2 and Table 7, Table 8, and Table 9 show the results of the test 3 under lossless, CTC and low QP test settings, respectively.

Table 1: results of test 1 under lossless test conditions

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | Test 1 |  | VTM-7.0 | Test 1 |  | VTM-7.0 | Test 1 |  |
| Class A1 | 2.2 | 2.3 | -3.49% | 2.2 | 2.3 | -3.33% |  |  |  |
| Class A2 | 1.6 | 1.7 | -7.56% | 1.7 | 1.8 | -5.09% |  |  |  |
| Class B | 2.2 | 2.2 | -2.96% | 2.3 | 2.4 | -1.01% | 2.3 | 2.4 | -1.00% |
| Class C | 1.9 | 2.0 | -4.24% | 2.4 | 2.5 | -2.18% | 2.4 | 2.5 | -2.03% |
| Class D | 1.9 | 2.0 | -4.89% | 2.8 | 2.8 | -2.37% | 2.7 | 2.8 | -2.21% |
| Class E | 2.8 | 2.9 | -4.28% |  |  |  | 3.1 | 3.2 | -3.53% |
| Class F | 5.3 | 5.5 | -3.82% | 33.7 | 34.9 | -3.02% | 50.7 | 52.3 | -2.84% |
| TGM | 11.8 | 12.5 | -5.20% | 107.1 | 116.6 | -6.68% | 124.9 | 137.0 | -6.92% |
| Overall | 2.1 | 2.2 | -4.32% | 2.2 | 2.3 | -2.60% | 2.6 | 2.6 | -1.98% |
| Enc Time[%] | 97% |  |  | 98% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 104% |  |  |

Table 2: results of test 1 under CTC test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 97% | 99% |
| Class A2 | 0.00% | -0.02% | -0.02% | 97% | 99% |
| Class B | 0.00% | 0.05% | -0.02% | 98% | 99% |
| Class C | -0.03% | -0.02% | -0.03% | 97% | 100% |
| Class E | 0.01% | -0.02% | -0.05% | 97% | 99% |
| Overall | -0.01% | 0.00% | -0.03% | 97% | 99% |
| Class D | -0.04% | -0.03% | -0.09% | 98% | 101% |
| Class F | -0.11% | -0.14% | 0.06% | 102% | 105% |
| Class SCC | -0.33% | -0.23% | -0.28% | 102% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 99% | 99% |
| Class A2 | -0.03% | -0.01% | 0.02% | 99% | 99% |
| Class B | 0.02% | 0.02% | 0.15% | 99% | 99% |
| Class C | -0.02% | 0.06% | 0.10% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.08% | 99% | 99% |
| Class D | 0.00% | -0.05% | 0.03% | 98% | 99% |
| Class F | -0.05% | 0.04% | -0.03% | 102% | 104% |
| Class SCC | -0.10% | -0.12% | -0.07% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.25% | 0.01% | 99% | 102% |
| Class C | 0.03% | -0.42% | -0.43% | 99% | 99% |
| Class E | -0.08% | 0.54% | -0.37% | 97% | 98% |
| Overall | -0.01% | -0.11% | -0.23% | 98% | 100% |
| Class D | -0.06% | 0.13% | 0.15% | 98% | 95% |
| Class F | -0.08% | -0.43% | 0.48% | 103% | 107% |
| Class SCC | -0.12% | -0.19% | -0.18% | 103% | 107% |

Table 3: results of test 1 under low QP test condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.04% | -0.04% | 102% | 101% |
| Class A2 | 0.00% | -0.02% | -0.02% | 102% | 101% |
| Class B | -0.03% | -0.04% | -0.07% | 102% | 101% |
| Class C | -0.12% | -0.13% | -0.12% | 101% | 100% |
| Class E | -0.17% | -0.29% | -0.36% | 101% | 101% |
| Overall | -0.07% | -0.10% | -0.12% | 102% | 101% |
| Class D | -0.16% | -0.18% | -0.20% | 102% | 102% |
| Class F | -1.55% | -1.01% | -0.99% | 102% | 102% |
| Class SCC | -1.97% | -1.77% | -1.78% | 103% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | -0.03% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | -0.02% | -0.04% | 102% | 101% |
| Class C | -0.04% | -0.05% | -0.02% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.02% | 101% | 101% |
| Class D | -0.06% | 0.07% | -0.12% | 101% | 101% |
| Class F | -2.01% | -0.98% | -1.00% | 102% | 103% |
| Class SCC | -2.54% | -1.56% | -1.54% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.02% | -0.01% | 102% | 101% |
| Class C | -0.03% | 0.00% | -0.01% | 101% | 104% |
| Class E | -0.04% | 0.02% | -0.01% | 102% | 105% |
| Overall | -0.02% | 0.00% | -0.01% | 101% | 103% |
| Class D | -0.08% | 0.05% | -0.13% | 101% | 103% |
| Class F | -0.65% | -0.53% | -0.66% | 102% | 105% |
| Class SCC | -1.48% | -1.08% | -1.09% | 103% | 105% |

Table 4: results of test 2 under lossless condition

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | Test 2 |  | VTM-7.0 | Test 2 |  | VTM-7.0 | Test 2 |  |
| Class A1 | 2.2 | 2.2 | -0.20% | 2.2 | 2.3 | -0.56% |  |  |  |
| Class A2 | 1.6 | 1.6 | -0.03% | 1.7 | 1.7 | -0.09% |  |  |  |
| Class B | 2.2 | 2.2 | -0.15% | 2.3 | 2.4 | -0.39% | 2.3 | 2.3 | -0.39% |
| Class C | 1.9 | 1.9 | -0.19% | 2.4 | 2.4 | -0.75% | 2.4 | 2.4 | -0.77% |
| Class D | 1.9 | 1.9 | -0.23% | 2.8 | 2.8 | -0.74% | 2.7 | 2.8 | -0.79% |
| Class E | 2.8 | 2.8 | -0.37% |  |  |  | 3.1 | 3.2 | -1.27% |
| Class F | 5.3 | 5.3 | -0.10% | 33.7 | 33.8 | -0.27% | 50.7 | 50.6 | -0.17% |
| TGM | 11.8 | 11.9 | -0.14% | 107.1 | 107.3 | -0.27% | 124.9 | 125.1 | -0.26% |
| Overall | 2.1 | 2.1 | -0.18% | 2.2 | 2.2 | -0.46% | 2.6 | 2.6 | -0.73% |
| Enc Time[%] | 102% |  |  | 102% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  | 105% |  |  |

Table 5: results of test 2 under CTC test condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.04% | 97% | 99% |
| Class A2 | 0.00% | -0.02% | -0.04% | 97% | 99% |
| Class B | 0.00% | -0.01% | -0.01% | 97% | 99% |
| Class C | 0.00% | -0.03% | -0.01% | 97% | 100% |
| Class E | 0.00% | 0.02% | -0.01% | 97% | 99% |
| Overall | 0.00% | -0.02% | -0.01% | 97% | 99% |
| Class D | 0.01% | 0.02% | 0.02% | 97% | 101% |
| Class F | 0.01% | 0.01% | 0.06% | 102% | 105% |
| Class SCC | 0.00% | -0.20% | -0.19% | 102% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.04% | 98% | 99% |
| Class A2 | -0.02% | -0.10% | 0.09% | 98% | 99% |
| Class B | 0.02% | -0.05% | 0.04% | 98% | 99% |
| Class C | 0.01% | 0.00% | 0.01% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | 0.04% | 98% | 99% |
| Class D | 0.00% | -0.26% | -0.07% | 98% | 99% |
| Class F | 0.02% | 0.03% | -0.04% | 102% | 104% |
| Class SCC | 0.02% | -0.20% | -0.15% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.26% | -0.09% | 99% | 102% |
| Class C | 0.01% | -0.33% | -0.25% | 99% | 99% |
| Class E | 0.02% | 0.27% | -0.29% | 97% | 98% |
| Overall | 0.00% | -0.15% | -0.19% | 98% | 100% |
| Class D | 0.00% | -0.16% | -0.25% | 98% | 95% |
| Class F | 0.02% | -0.34% | 0.28% | 103% | 108% |
| Class SCC | 0.00% | -0.14% | -0.27% | 103% | 107% |

Table 6: results of test 2 under low QP test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | -0.01% | 101% | 101% |
| Class B | -0.01% | 0.00% | -0.01% | 101% | 100% |
| Class C | 0.00% | -0.02% | -0.02% | 101% | 101% |
| Class E | -0.07% | 0.03% | 0.05% | 101% | 101% |
| Overall | -0.01% | 0.00% | 0.00% | 101% | 101% |
| Class D | -0.01% | 0.00% | -0.02% | 102% | 102% |
| Class F | -0.06% | -0.05% | -0.04% | 101% | 103% |
| Class SCC | -0.02% | -0.12% | -0.11% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class B | 0.01% | -0.03% | -0.03% | 101% | 102% |
| Class C | 0.00% | -0.06% | -0.08% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.04% | 101% | 101% |
| Class D | 0.00% | -0.02% | -0.11% | 100% | 101% |
| Class F | -0.06% | 0.04% | -0.10% | 102% | 104% |
| Class SCC | 0.06% | -0.16% | -0.16% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.03% | -0.02% | 101% | 101% |
| Class C | 0.00% | -0.06% | -0.03% | 101% | 104% |
| Class E | -0.03% | 0.01% | 0.07% | 101% | 105% |
| Overall | 0.00% | -0.03% | 0.00% | 101% | 103% |
| Class D | -0.01% | -0.03% | -0.12% | 101% | 101% |
| Class F | 0.03% | 0.03% | -0.06% | 102% | 106% |
| Class SCC | 0.03% | -0.14% | -0.15% | 102% | 105% |

Table 7: results of test 3 under lossless test condition

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-7.0 | Test 3 |  | VTM-7.0 | Test 3 |  | VTM-7.0 | Test 3 |  |
| Class A1 | 2.2 | 2.3 | -3.68% | 2.2 | 2.3 | -3.88% |  |  |  |
| Class A2 | 1.6 | 1.7 | -7.59% | 1.7 | 1.8 | -5.18% |  |  |  |
| Class B | 2.2 | 2.2 | -3.11% | 2.3 | 2.4 | -1.43% | 2.3 | 2.4 | -1.41% |
| Class C | 1.9 | 2.0 | -4.43% | 2.4 | 2.5 | -2.96% | 2.4 | 2.5 | -2.82% |
| Class D | 1.9 | 2.0 | -5.12% | 2.8 | 2.8 | -3.14% | 2.7 | 2.8 | -3.01% |
| Class E | 2.8 | 2.9 | -4.67% |  |  |  | 3.1 | 3.3 | -4.86% |
| Class F | 5.3 | 5.5 | -3.93% | 33.7 | 35.0 | -3.32% | 50.7 | 52.1 | -3.07% |
| TGM | 11.8 | 12.5 | -5.35% | 107.1 | 116.8 | -6.94% | 124.9 | 137.1 | -7.08% |
| Overall | 2.1 | 2.2 | -4.50% | 2.2 | 2.3 | -3.08% | 2.6 | 2.6 | -2.75% |
| Enc Time[%] | 97% |  |  | 99% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  | 110% |  |  |

Table 8: results of test 3 under CTC test condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.01% | 97% | 99% |
| Class A2 | 0.00% | -0.01% | -0.05% | 97% | 99% |
| Class B | 0.00% | 0.03% | -0.01% | 97% | 99% |
| Class C | -0.04% | 0.00% | 0.03% | 97% | 100% |
| Class E | 0.00% | -0.05% | -0.05% | 97% | 99% |
| Overall | -0.01% | -0.01% | -0.01% | 97% | 99% |
| Class D | -0.05% | -0.05% | -0.08% | 97% | 102% |
| Class F | -0.10% | -0.17% | 0.05% | 102% | 105% |
| Class SCC | -0.35% | -0.43% | -0.45% | 102% | 104% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.06% | 98% | 99% |
| Class A2 | -0.02% | -0.10% | -0.04% | 98% | 99% |
| Class B | 0.02% | 0.04% | 0.09% | 98% | 99% |
| Class C | 0.01% | -0.08% | -0.12% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | 0.00% | 98% | 99% |
| Class D | -0.01% | 0.07% | 0.19% | 98% | 100% |
| Class F | -0.07% | -0.05% | 0.00% | 102% | 103% |
| Class SCC | -0.21% | -0.39% | -0.32% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.20% | -0.08% | 98% | 102% |
| Class C | 0.04% | -0.21% | -0.29% | 98% | 99% |
| Class E | -0.08% | 0.78% | -0.50% | 97% | 98% |
| Overall | -0.02% | 0.04% | -0.25% | 98% | 100% |
| Class D | -0.02% | 0.20% | -0.30% | 97% | 98% |
| Class F | -0.10% | -0.43% | 0.08% | 103% | 107% |
| Class SCC | -0.14% | -0.35% | -0.31% | 103% | 107% |

Table 9: results of test 3 under low QP test conditions

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.05% | -0.05% | 102% | 101% |
| Class A2 | 0.00% | -0.02% | -0.02% | 102% | 101% |
| Class B | -0.04% | -0.03% | -0.08% | 102% | 101% |
| Class C | -0.14% | -0.17% | -0.17% | 101% | 101% |
| Class E | -0.22% | -0.30% | -0.34% | 101% | 101% |
| Overall | -0.09% | -0.11% | -0.13% | 101% | 101% |
| Class D | -0.18% | -0.26% | -0.28% | 102% | 106% |
| Class F | -1.62% | -1.05% | -1.02% | 102% | 102% |
| Class SCC | -2.07% | -1.90% | -1.90% | 102% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.02% | -0.05% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class B | 0.00% | -0.07% | -0.09% | 101% | 102% |
| Class C | -0.04% | -0.11% | -0.12% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.07% | 101% | 101% |
| Class D | -0.08% | -0.10% | -0.25% | 101% | 101% |
| Class F | -1.84% | -0.89% | -1.05% | 102% | 103% |
| Class SCC | -2.46% | -1.69% | -1.66% | 103% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.05% | -0.05% | 101% | 101% |
| Class C | -0.01% | -0.04% | -0.06% | 101% | 104% |
| Class E | -0.06% | 0.02% | -0.04% | 101% | 105% |
| Overall | -0.01% | -0.03% | -0.05% | 101% | 103% |
| Class D | -0.04% | -0.05% | -0.14% | 101% | 102% |
| Class F | -0.75% | -0.73% | -0.79% | 102% | 105% |
| Class SCC | -1.52% | -1.22% | -1.21% | 103% | 106% |

# Conclusion

In this contribution, two methods are proposed. The first method derives the rice parameter from the top and left neighboring level. The second method introduces additional 7 context variables (as compared to VTM-7.0) for chroma TS residual coding method. In lossless test conditions, the proposed methods jointly can achieve -4.50%(AI), -3.08% (RA) and -2.75% (LB) coding gain.

# References

Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang, “ Non-CE8: Transform skip residual coding simplification”, JVET-P0562, Geneva, Oct 2019

T. Nguyen, T. C. Ma, A. Nalci, “ Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).