| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0271 |
| --- | --- |

| Title: | AHG9: On Subpicture Ordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mitra Damghanian Rickard Sjöberg Martin Pettersson | Email: | mitra.damghanian@ericsson.com rickard.sjoberg@ericsson.com martin.m.pettersson@ericsson.com |  |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes that the following changes in VVC are adopted.

- To define an independent subpicture as a subpicture for which the subpicture boundaries are treated as picture boundaries in the decoding process and no loop filtering across the subpicture boundaries is done. (Editorial)

- To add a syntax element sps_independent_subpics_flag in the SPS. When equal to 1 it specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are signalled only when sps_independent_subpics_flag is equal to 0. (Signalling efficiency)

- To relax the availability rule for independent subpictures. (Functionality)

One motivation for this proposal is that the current restricted ordering of the subpictures is claimed by the proponents to be suboptimal for some low latency applications. For example, a decoder decoding in raster scan order is claimed to need to wait for all subpictures that complete a full CTU row within a picture, before being able to decode that CTU row while some subpictures might come late in the bitstream because of the availability rule even if the subpictures are independent. The proponents claim that this potentially increases latency and memory requirements which are critical in some applications, such as 360 video streaming.

Another motivation is that the current ordering constraints is claimed to potentially bring unnecessary complexity to processes such as bitstream merging. The proponents claim that additional complexity in the bitstream merging process is required to handle proper ordering of the subpictures in the bitstream even when the encoding/decoding of the content of a subpicture does not rely on the encoding/decoding of other subpictures in the picture.

# Introduction and Problem Statement

In the current version of the VVC draft specification, JVET-Q0041-v2_DraftText.docx, the order of subpictures is constrained by an availability rule specifying that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures. However, the current restrictions on the order of the subpictures are not necessary or optimal for all applications and use cases, for example when the subpictures are independent, and the optimal decoding order of the subpictures at the decoder side is not supported by the current ordering restrictions. Here we define an independent subpicture as a subpicture for which the subpicture boundaries are treated as picture boundaries in the decoding process and no loop filtering is allowed across the subpicture boundaries.

As one example, the current restricted ordering of the subpictures is suboptimal for some low latency applications. As shown in Figure 1, a decoder decoding in raster scan order needs to wait for subpicture 2 which comes late in the bitstream for the illustrated subpicture partition layout within a picture, before being able to decode the first CTU row even if the subpictures are independent. The reason for this is that the current constraints do not allow VCL NALUs of subpicture 2 to precede VCL NALUs of subpicture 1 in the bitstream. This potentially increases latency and memory requirements which are critical in some applications, such as 360 video streaming.

Figure 1 Constraints on the order of independent subpictures in the current version of VVC can increase latency for decoding CTUs in raster scan order.

Another problem with the ordering constraints in the current version of the VVC draft specification is that they potentially bring unnecessary complexity to processes such as bitstream merging. The additional complexity is that the bitstream merging process needs to handle the proper ordering of the subpictures in the bitstream even when the encoding/decoding of the content of a subpicture is not relying on the encoding/decoding of other subpictures in the picture.

Figure 2 illustrates how the latency can be improved by relaxing the availability constraint for the independent subpictures.

Figure 2 Proposed subpicture order for independent subpictures to improve latency.

In the current version of VVC, signalling that all the subpictures are independent costs two bits per subpicture in the SPS: one SPS flag per subpicture to specify treating subpicture boundaries as picture boundaries or not and another SPS flag per subpicture to specify that the loop filtering across the subpicture boundaries is enabled or disabled. However, since independent subpictures can be a common case, it would be desirable to have a more efficient signalling, such as one single SPS flag, to specify that the picture consists of independent subpictures only.

Moreover, the current signalling for subpicture boundary handling is not easily detectable by higher level processes such as sub-bitstream extraction and merge processes that target independent subpictures.

# Proposal

We propose the three following elements:

- To define an independent subpicture as a subpicture for which the subpicture boundaries are treated as picture boundaries in the decoding process and no loop filtering across the subpicture boundaries is done. (Editorial)

- To add a syntax element sps_independent_subpics_flag in the SPS that when equal to 1 it specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. When equal to 0 it specifies that intra prediction, inter prediction or in-loop filtering operations across the subpicture boundaries in the CLVS may be allowed. subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are signalled only when sps_independent_subpics_flag is equal to 0. (Signalling efficiency)

- To relax the availability rule for independent subpictures. (Functionality)

## Proposed syntax on top of JVET-Q0041-v2_DraftText.docx

The proposed syntax is shown below in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

## Semantics on top of JVET-Q0041-v2_DraftText.docx

The proposed changes to the semantics is shown below in yellow with removed text in red strike-through.

3. Definitions:

…

3.56 hypothetical stream scheduler (HSS): A hypothetical delivery mechanism used for checking the conformance of a bitstream or a decoder with regards to the timing and data flow of the input of a bitstream into the hypothetical reference decoder.

3.57 independent subpicture: a subpicture for which the subpicture boundaries are treated as picture boundaries in the decoding process and no loop filtering across the subpicture boundary is allowed.

3.573.58 informative: A term used to refer to content provided in this Specification that does not establish any mandatory requirements for conformance to this Specification and thus is not considered an integral part of this Specification.

…

sps_independent_subpics_flag equal to 1 specifies that no intra prediction, no inter prediction and no in-loop filtering operations may be performed across any subpicture boundary in the CLVS. sps_independent_subpics_flag equal to 0 specifies that intra prediction, inter prediction or in-loop filtering operations across the subpicture boundaries in the CLVS may be allowed. When not present, the value of sps_independent_subpics_flag is inferred to be equal to 0.

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0 sps_independent_subpics_flag.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1!sps_independent_subpics_flag.

It is a requirement of bitstream conformance that the following constraints apply:

- For any two subpictures subpicA and subpicB, when the subpicture index of subpicA is less than that of subpicB, any coded slice NAL unit of subPicA shall precede any coded slice NAL unit of subPicB in decoding order.

- When sps_independent_subpics_flag is equal to 0, the shape of subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).