| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0274-v2 |
| --- | --- |

| Title: | MIP with fixed-length mode coding and memory reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Thibaud Biatek Luong Pham Van Adarsh K. Ramasubramonian Geert Van der Auwera Marta Karczewicz | Email: | lphamvan@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to reduce the number of MIP matrices in order to harmonize the fixed-length coding for MIP mode signaling. In VTM-7.0, the MIP sets contain 16, 8 and 6 matrices, respectively for MipSizeId 0, 1 and 2. We propose to reduce the number of MIP matrices for MipSizeId 2 from 6 to 4. This enables to use fixed-length coding for all MipSizeId and brings the side benefit of saving some storage (18%). In addition, VTM-7.0 MIP weights are kept as we simply select a subset of existing matrices (no retraining). The proposed approach leads to an average (Y, U, V) Bd-rate of (0.01%, -0.04%, -0.01%) and (0.02%, 0.02%, 0.06%) for AI and RA, respectively. (Version 2: Update correct results)

It is proposed to adopt this cleanup in the next VVC specification.

# Introduction

During the 16th JVET meeting in Geneva, Switzerland, a simplified design of MIP was adopted for VVC working draft version 67 [1], providing design simplification and storage savings [2]. The VTM-7.0 MIP design comprises 30 matrices distributed as follows:

- MipSizeId=0  N=16 matrices (16x4), one offset vector (1x16) and one shift vector (1x16).

- MipSizeId=1  N=8 matrices (16x8), one offset vector (1x8) and one shift vector (1x8).

- MipSizeId=2  N=6 matrices (64x7), one offset vector (1x6) and one shift vector (1x6).

It is noticed that signalling of MIP matrices can be done using fixed-length coding for MipSizeId 0 and 1 while MipSizeId 2 required truncated binary.

# Proposed design

In order to clean further the MIP design, it is proposed to make it more consistent by introducing fixed length coding for MipSizeId=2. To avoid retraining MIP matrices and to keep the VTM design stable, a subset of VTM-7.0 matrices has been selected. The subset {1,2,4,5} illustrated in the Table below is selected:

Table 1: Selected matrices

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X | X |
| 1 | X | X | X | X | X | X | X | X | X | X |  |  |  |  |  |  |  |  |
| 2 |  | X | X |  | X | X |  |  |  |  |  |  |  |  |  |  |  |  |

The specification length is reduced as two matrices are removed, and the binarization section simplified, as illustrated below:

Table 2: intra_mip_mode binarization in VVC draft 7

| intra_mip_mode[ ][ ] | TB | cMax = (cbwidth = = 4 && cbHeight = = 4) ? 15 : ( ( ( cbWidth = = 4 ¦ ¦ cbHeight = = 4 ) ¦ ¦ (cbWidth = = 8 && cbHeight = = 8 ) ? 7 : 5 ) |
| --- | --- | --- |

Table 2: proposed intra_mip_mode binarization

| intra_mip_mode[ ][ ] | FL | cMax = 1 << (4-MipSizeId) |
| --- | --- | --- |

The modified specification is attached to this contribution.

# Memory footprint

The proposed design reduces the MIP memory footprint. The table below provides details on achieved storage savings. It is observed that the proposed design reduces the MIP memory footprint by 18.77% compared to previous design, with a total storage requirement below 3.5KB.

Table 3: MIP memory footprint of VTM-7.0 against proposal

# Experimental Results

The proposed design has been implemented into the VTM-7.0. Performance is assessed on. Results are provided in the following subsections. Please note that decoding runtimes are not accurate.

Table 4: AI performance under CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.22% | 0.07% | 101% | 99% |
| Class A2 | 0.02% | -0.01% | -0.07% | 100% | 99% |
| Class B | 0.02% | 0.00% | -0.09% | 102% | 101% |
| Class C | 0.00% | 0.05% | 0.01% | 89% | 114% |
| Class E | 0.00% | -0.06% | 0.06% | 103% | 110% |
| Overall | 0.01% | -0.04% | -0.01% | 99% | 105% |
| Class D | -0.01% | 0.04% | -0.06% | 100% | 88% |
| Class F | -0.03% | -0.03% | 0.07% | 102% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.00% | 102% | 52% |
| Class A2 | 0.00% | -0.02% | 0.05% | 100% | 33% |
| Class B | 0.01% | 0.07% | 0.16% | 102% | 36% |
| Class C | 0.03% | -0.01% | -0.01% | 102% | 57% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.06% | 102% | 43% |
| Class D | 0.03% | -0.07% | -0.11% | 101% | 50% |
| Class F | -0.01% | 0.02% | 0.10% | 101% | 61% |

# Conclusion

This contributions proposes a cleanup for MIP that enables harmonized fixed length coding with the side benefit of providing 18% of memory reduction. It comes with negligible losses. It is proposed to adopt the proposed cleanup in next version of VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang “Versatile Video Coding (Draft 7), 16th JVET Meeting, Geneva, Switzerland, Oct. 2019, JVET-P2001.

- J. Pfaff, B. Stallenberger, M. Schäfer, P. Merkle, T. Hinz, P. Helle, H. Schwarz, D. Marpe, T. Wiegand (HHI), T. Biatek, A. K. Ramasubramonian, G. Van der Auwera, M. Karczewicz (Qualcomm), H. Liu, L. Zhang, K. Zhang, Z. Deng, J. Xu (Bytedance), J. Choi, J. Heo, J. Lim, S. Kim (LGE), K. Naser, T. Poirier, F. Le Leannec, F. Galpin (Inter Digital), Z. Zhang, K. Andersson, D. Saffar, R. Sjöberg, J. Ström, R. Yu (Ericsson), “Non-CE3: Combined cleanup of MIP”, 16th JVET Meeting, Geneva, Switzerland, Oct. 2019, JVET-P2001.

- E. Augustsson and R. Timotfe, “NTIRE 2017 Challenge on Single Image Super-Resolution: Dataset and Study”, IEEE Conference on Computer Vision and Pattern Recognition (CVPR), July 2017.

- J. Ström, D. Saffar, P. Wennersten, R. Sjöberg, “AHG13: Tool-off tests on Open Images dataset”, 15th JVET Meeting, Gothenburg, Sweden, Jul. 2019, JVET-O0551

- https://ffmpeg.org/

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).