| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0277 |
| --- | --- |

| Title: | AHG8/AHG9: Layer dependency constraints for parameter set reference |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to impose additional constraints to make the support of parameter set reference across layers clear. The proposed constraint is that a coded slice NAL unit shall only refer to a PPS/APS in the same layer or in a (direct) reference layer, and a PPS NAL unit shall only refer to a SPS in the same layer or in a (direct) reference layer.

Problem statement

In NAL unit header semantics of the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), the value of nuh_layer_id for non-VCL NAL units is constrained as follows:

- 7.4.2.2 NAL unit header semantics … – If nal_unit_type is equal to PPS_NUT, PREFIX_APS_NUT, or SUFFIX_APS_NUT, nuh_layer_id shall be equal to the lowest nuh_layer_id value of the coded slice NAL units that refer to the NAL unit. – Otherwise, if nal_unit_type is equal to SPS_NUT, nuh_layer_id shall be equal to the lowest nuh_layer_id value of the PPS NAL units that refer to the SPS NAL unit.

The contraints are intended to allow the parameter sets (SPS, PPS, APS) reference across layers, so that a coded slice NAL unit can/shall only refer to a PPS/APS NAL unit in the same or a lower layer, and a PPS NAL unit can/shall only refer to a SPS NAL unit in the same or a lower layer.

In this proposal, however, it is asserted that the contraints need to be improved to address some corner cases that a coded slice VCL NAL unit refers to a PPS/APS NAL unit in a layer that is not a reference layer of  the coded slice VCL NAL unit. Note that a coded slice VCL NAL unit can refer to a PPS/APS NAL unit in a non-reference layer, with the given contraints.

For example, Fig. 1 shows a simple example with two layers, where the nuh_layer_id value of the layer B is greater than the nuh_layer_id value of the layer A, and the layer A is not a direct/indirect reference layer of the layer B. In that case, a coded slice VCL NAL unit in the layer B can refer to a PPS/APS with nuh_layer_id equal to nuh_layer_id of the layer A, because there is not a contraint to disallow a PPS/APS reference in a non-reference layer. Still, nuh_layer_id of the PPS/APS is equal to the lowest nuh_layer_id value of the coded slice NAL units that refer to the NAL unit.

Figure 1. An example of parameter set reference in a non-reference layer.

In the example, the layer A and the layer B may belong to different (output) layer sets, and the NAL units of the layer A can be discarded by a bitstream extraction process, while the NAL unit of the layer B are still present in the output bitstream. Then, the PPS/APS NAL units referred to by the coded slice NAL unit of the layer B may not be present in the output bitstream.

Proposal

In order to address the above issue, the current constraints need to be improved. The proposed constraint is that a coded slice NAL unit shall only refer to a PPS/APS in the same layer or in a (direct) reference layer, and a PPS NAL unit shall only refer to a SPS in the same layer or in a (direct) reference layer. In Fig. 1, if the layer A is a reference layer of the layer B, both layers shall belong to the same (output) layer set, and shall be present in an extracted bitstream, always. Then, the SPS/PPS/APS VCL NAL unit referred to by a NAL unit in another layer are never removed.

Proposed text option-1

- 7.4.2.2 NAL unit header semantics … – When a coded slice NAL unit refers to a non-VCL NAL unit with nal_unit_type equal to PPS_NUT, PREFIX_APS_NUT, or SUFFIX_APS_NUT, the layer of the non-VCL NAL unit shall be equal to the layer of the coded slice NAL unit or a direct reference layer of the coded slice NAL unit. – When a PPS NAL unit refers to a SPS NAL unit, the layer of the SPS NAL unit shall be equal to the layer of the PPS NAL unit or a direct reference layer of the PPS NAL unit.

Proposed text option-2

- 7.4.2.2 NAL unit header semantics … – When a coded slice NAL unit refers to a non-VCL NAL unit with nal_unit_type equal to PPS_NUT, PREFIX_APS_NUT, or SUFFIX_APS_NUT, the layer of the non-VCL NAL unit shall be equal to the layer of the coded slice NAL unit or a direct/indirect reference layer of the coded slice NAL unit. – When a PPS NAL unit refers to a SPS NAL unit, the layer of the SPS NAL unit shall be equal to the layer of the PPS NAL unit or a direct/indirect reference layer of the PPS NAL unit.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- References

[1]	B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019