| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0280 |
| --- | --- |

| Title: | AHG8/AHG9: On video parameter set ID |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

To make it clear that only one VPS is referred to by SPSs in a CVS, the following constraint is proposed:

- The value of sps_video_parameter_set_id shall be the same in all SPSs that are referred to by coded pictures in a CVS.

Introduction

In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), the sps_video_parameter_set_id signaled in a sequence parameter set (SPS), specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS. If each SPS in a CVS has a different value of sps_video_parameter_set_id, one or more video parameter sets (VPSs) may be referred to within a CVS. In the current VVC spec. there is no constraint that prohibit each SPS has a different value of sps_video_parameter_set_id within a CVS. In an extreme case, 20 layers belonging to the same (output) layer set can refer to 20 different VPSs.

There could be two approaches to address this issue. The first approach is to allow that any SPS in a CVS shall only refer to the same VPS. The second approach is to allow that multiple VPS with different nuh_layer_id values are referred to by different output layer sets. However, the benefit of the second approach is not clear, as compared to the first approach, while the second approach may increase burden to parse multiple VPS.

In this contribution, we simply propose to add the following constraint in the VVC spec.

Proposed text:

The value of sps_video_parameter_set_id shall be the same in all SPSs that are referred to by coded pictures in a CVS.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).