| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0283 |
| --- | --- |

| Title: | AHG8/AHG9/AHG12: On subbitstream extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

A subpictures extraction functionality is proposed. To enable this functionality in the current VVC spec., three following items needs to be addressd:

- Identification of adaptation parameter sets referenced by a specific sub-picture.

- Rewriting of picture size, conformance window and sub-picture layout information in parameter sets for the extracted sub-bitstream.

- Sub-bitstream extraction process for the target sub-pictures, with the solutions to address the items 1 and 2.

JVET-Q0285 (On adaptation parameter set identification) and JVET-Q0281 (Independently coded regions output window SEI message) are proposed in this meeting, to address the items 1 and 2, respectively. This contribution proposes the third item, the sub-picture extraction process from the bitstream, utilizing JVET-Q0285 and JVET-Q0281.

Proposal: Sub-pictures extraction

In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), the subpicture partitioning functionality is supported. When subpic_treated_as_pic_flag[i] in SPS is equal to 1, the i-th subpicture referring to the SPS has a motion contraint that a slice belonging to the subpicture cannot reference a slice out of the subpicture region. A subbitstream associated with one or more motion-contrained subpictures can be extracted from the bitstream containing the full picture and independently decoded. The decoded picture of the subbitstream is an independently coded region. As long as the indenpendently coded region has a rectangular shape, the subbitstream can be decoded and outputted as a single picture. This feature is useful for various immersive media use cases including tiled 360 video.

To enable this functionality in the current VVC spec., three following items needs to be addressd:

- Identification of adaptation parameter sets referenced by a specific sub-picture.

- Rewriting of picture size, conformance window and sub-picture layout information in parameter sets for the extracted sub-bitstream.

- Sub-bitstream extraction process for the target sub-pictures, with the solutions to address the items 1 and 2.

JVET-Q0285 (On adaptation parameter set identification) and JVET-Q0281 (Independently coded regions output window SEI message) are proposed in this meeting, to address the items 1 and 2, respectively. This contribution proposes the third item, the sub-picture extraction process from the bitstream, utilizing JVET-Q0285 and JVET-Q0281.

- Prosed spec text:

A.2 Sub-picture extraction process

This process may be invoked after Sub-bitstream extraction process (C.6).

Inputs to this process are a bitstream inBitstream, a target subpicture IDs targetSubPicIDs[].

Note: When the independently coded regions output window SEI is present, if an index of the target independent coded region is determined, the target subpicture IDs targetSubPicIDs [] are obtained from the SEI message. Otherwise, the target subpicture IDs targetSubPicIDs [] are inputted by an external mean.

Output of this process is a sub-bitstream outBitstream.

Note1: A coded region composed of the subpictures with targetSubPicIDs[] shall be a rectangular region.

Note2: The subpic_treated_as_pic_flag value of a subpicture with a subpicture ID equal to one of targetSubPicIDs[] shall be equal to 1.

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all VCL NAL units with slice_subpic_id not equal to one of subpicIDs[] values.

–	(if APS identification proposed by JVET-Q0285 is supported) Remove from outBitstream all NAL units with nal_unit_type equal to PREFIX_APS_NUT, or SUFFIX_APS_NUT, which do not include aps_subpic_id[ i ] equal to targetSubPicIDs[ j ] (only when aps_subpic_id_list_present_flag is present and equal to 1.)

–	Rewrite picture size (pic_width_in_luma_samples, pic_height_in_luma_samples), conformance window size (conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, conf_win_bottom_offset) and subpicture-related information, (if Independently coded regions output window SEI message proposed by JVET-Q0281 is present) referencing the associated Independently coded regions output window SEI messages.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).