| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0287 |
| --- | --- |

| Title: | AHG9: On wrap around motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution proposes two methods to address the issue that the wrap around motion compensation cannot correctly work when the reference picture width is different than the current picture width. The first option is to disable the wrap around motion compensation in high level syntax, when the layer of the current picture is a dependent layer, or the RPR is enabled for the current layer. The second option is to disable the wrap around processing during the interpolation process for motion compensation, when the reference picture width is different than the current picture width.

Problem statement

The wrap around motion compensation is a useful feature for coding a 360 projected picture with the equirectangular projection (ERP) format. It can reduce some visual artifacts at the seam line as well as improve the coding gain. In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), sps_ref_wraparound_offset_minus1 in SPS specifies the offset used for computing the horizontal wrap-around position. A problem is that the wrap around offset value is determined in relation to the picture width. If the picture width of the reference picture is different from the current picture width, the wrap around offset value should be changed in proportion to the scaling ratio between the current picture and the reference picture. In practical terms, however, it may considerably increase implementation and computation complexity to adjust the offset value according to the picture width of each reference picture, as compared to the advantage of the wrap around motion compensation. The inter-layer prediction with different picture sizes and the reference picture resampling (RPR) will result in awfully various combination of different picture resolutions across layers and across temporal pictures.

Proposal

This contribution proposes two practical methods to address this issue. The first option is to disable the wrap around motion compensation by setting sps_ref_wraparound_enabled_flag equal to 0, when the layer of the current picture is a dependent layer, or the RPR is enabled for the current layer. Thus, the wrap around motion compensation can be used only when the current layer is an independent layer and the RPR is disabled. Under this condition, the reference picture size shall be equal to the current picture size. The second option is to disable the wrap around motion compensation process during the interpolation process for motion compensation, when the reference picture width is different than the current picture width.

Proposed spec text – option 1

| seq_parameter_set_rbsp( ) { |  |
| --- | --- |
|  | u(1) |
| if( !ref_pic_resampling_enabled_flag) |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| sps_temporal_mvp_enabled_flag | u(1) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| sps_sbtmvp_enabled_flag | u(1) |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is less than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of sps_ref_wraparound_enabled_flag shall be equal to 0. It is a requirement of bitstream conformance that, when vps_independent_layer_flag[GeneralLayerIdx[nuh_layer_id]] is equal to 0, the value of sps_ref_wraparound_enabled_flag shall be equal to 0. When not present, the value of sps_ref_wraparound_enabled_flag is inferred to be equal to 0.

Proposed spec text – option 2

Let refPicWidthInLumaSamples be the pic_width_in_luma_samples, respectively, of the current reference picture of the current picture.

- If refPicWidthInLumaSamples are equal to pic_width_in_luma_samples of the current picture, refWraparoundEnabledFlag is set equal to sps_ref_wraparound_enabled_flag.

- Otherwise, refWraparoundEnabledFlag is set equal to 0.

- 8.5.3.2.2 Luma sample bilinear interpolation process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i )	(642)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )	(643)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag refWraparoundEnabledFlag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(644)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )

- 8.5.6.3.3 Luma integer sample fetching process

…

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL )	(966)

yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL )	(967)

–	Otherwise, the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag refWraparoundEnabledFlag?	(968)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(969)

The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt ][ yInt ] << shift3

        - Chroma sample interpolation process

…

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, xIntC + i − 1 )	(971)

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, yIntC + i − 1 )	(972)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag refWraparoundEnabledFlag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(973)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(974)

The chroma locations in full-sample units ( xInti, yInti ) are further modified as follows for i = 0..3:

xInti = Clip3( xSbIntC − 1, xSbIntC + sbWidth + 2, xInti )	(975)

yInti = Clip3( ySbIntC − 1, ySbIntC + sbHeight + 2, yInti )	(976)

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).