| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0288 |
| --- | --- |

| Title: | AHG9: On constant picture output with RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

Signaling constant picture size information in VUI is proposed, to indicate a guided output picture size for display, when each cropped output picture may have a different width and height values with reference picture resampling (RPR).

Introduction

In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), a reference picture resampling (RPR) may enable the change of decoded picture spatial resolutions picture by picture withing a coded video sequence (CVS). The decoded picture stored in a decoded picture buffer (DPB) will be outputted for display, probably after cropping. Depending on the picture width and height and cropping window offset values signalled in PPS, each output picture may have a different picture size than other output pictures. This is OK, as long as a display device has a capability to rescale the output pictures to a constant picture size to be fit into the display resolution. The post processing is purely the role of each display device.

Figure 1. A systematic illustration of resampling process for RPR and constant output picture size (originally proposed by JVET-N0052)

In some content service scenario, however, a content provider wants the video content to be consumed by a specific resolution, or want to indicate the best resolution for display, with a director’s intention. In order to address such a use case, JVET-N0052 originally proposed signaling the (constant) output picture size in SPS, but it was not adopted because JVET experts wanted to leave the process as a post processing out of decoding process. That was a reasonable argument.

To address two different requirements, which are carrying a director’s intention and leaving it as a freedom of a display, this contribution proposes to signal the constant output picture size in VUI, as an informative metadata. An end user’s device still has a freedom to choose the display picture resolution, but may be able to accept a director’s suggestion, optionally.

Proposed spec text

VUI parameters syntax

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_output_pic_size_flag | u(1) |
| if( constant_output_pic_size_flag ) { |  |
| guided_constant_output_pic_size_present_flag | u(1) |
| if( guided_constant_output_pic_size_present_flag ) { |  |
| constant_output_pic_width_in_luma_samples | u(16) |
| constant_output_pic_height_in_luma_samples | u(16) |
| } |  |
| } |  |
| … |  |
| } |  |

constant_output_pic_size_flag equal to 1 specifies that any post-resampling process is applied to each cropped output picture so that each resampled output picture shall have the constant picture size, specified by constant_output_pic_width_in_luma_samples and constant_output_pic_height_in_luma_samples. constant_output_pic_size_flag equal to 0 specifies that a post-resampling process may or may not be applied to each cropped output picture.

Note: the post-resampling process is not specified in this specification.

guided_constant_output_pic_size_present_flag equal to 1 specifies that constant_output_pic_width_in_luma_samples and constant_output_pic_height_in_luma_samp are present in this VUI. guided_constant_output_pic_size_present_flag equal to 0 specifies that constant_output_pic_width_in_luma_samples and constant_output_pic_height_in_luma_samp are not present in this VUI.

constant_output_pic_width_in_luma_samples specifies the width of each output picture after a post-resampling process in units of luma samples. When not present, the value of constant_output_pic_width_in_luma_samples is inferred to be equal to pic_width_max_in_luma_samples in SPS.

constant_output_pic_height_in_luma_samples specifies the height of each output picture after a post-resampling process in units of luma samples. When not present, the value of constant_output_pic_height _in_luma_samples is inferred to be equal to pic_height_max_in_luma_samples in SPS.

- Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).