| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0292-v3 |
| --- | --- |

| Title: | Cleanup of intra reference sample filter selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Junghak Nam hyeongmun Jang Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, jangwon84.choi, junghak.nam, hm.jang, moonmo.koo jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VVC draft, the intra reference sample filter decision process relies on the size-dependent threshold to select intra reference sample filters, and the intra reference sample filtering process requires multiple checks because intra mode information should be considered to select intra reference sample filters. This contribution introduces two methods to simplify this intra reference sample filtering process. In Method 1, the selection of filters for both the reference sample filter and the interpolation filter are directly derived according to the size-dependent threshold. In Method 2, the reference sample filter process is completely removed in the intra coding process and the interpolation filter are selected based on the size-dependent threshold. Method 1 provides the coding performance changes of 0.00%, 0.00%, and -0.04% BD-rate in AI, RA, and LD configuration, respectively. Method 2 provides the coding performance changes of 0.01%, 0.02%, and 0.03% BD-rate in AI, RA, and LD configuration, respectively.

Introduction

Since the intra reference sample filter decision process uses the size-dependent threshold to select intra reference sample filters, intra reference sample filters should be selected in difference process according to the size-dependent threshold. Moreover, the process of determining intra reference sample filters has still multiple checks, as intra mode information should be considered to select intra reference sample filters. This contribution proposes to simplify the intra reference sample filter selection.

Proposed method

This contribution proposes to determine intra reference sample filter selection without the intra mode checking process. Table 1 and Table 2 describe the proposed intra reference sample filter selection process for Method 1 and Method 2, respectively.

In Table 1 and Table 2, the refFilterFlag and the interpolationFlag represent the reference sample filter flag and the interpolation filter flag, respectively. If the refFilterFlag is set equal to true or false, the intra reference sample smoothing filter is enabled or disabled. The intraMode, HorMode, and VerMode represent the current intra mode, horizontal mode, and vertical mode. If the current intra mode is one of {-14, -12, -10, -6, 2, 34, 66, 72, 76, 78, 80} (the intra prediction mode refers to integer reconstructed boundary samples), isIntegerSlop is true. Otherwise, isIntegerSlop returns false. If the interpolationFlag is set equal to true or false, Gaussian or DCT-IF filter is applied, respectively.

The only difference between Method 1 and Method 2 is the reference sample filter selection process for Planar.

Table 1. Proposed intra reference sample filter selection process for Method 1

Table 2. Proposed intra reference sample filter selection process for Method 2

In Method 1, the reference sample filter is selected as the same way as VTM-7.0 except for the angular mode. Intra reference sample filters for the angular mode are selected as follows.

- If diff is greater than size-dependent threshold - refFilterFlag is set to false. - interpolationFlag is set to Gaussian. - Otherwise, - refFilterFlag is set to false. - interpolationFlag is set to DCT-IF.

Since intra reference sample filters are directly determined according to the size-dependent threshold, intra mode information is unnecessary for Method 1.

Method 2 proposes to remove the reference sample filter flag decision process. This means in all intra prediction the reference sample filter process is completely removed in intra coding process.

Experimental result

The proposed method has been implemented using VTM-7.0 and under the common test conditions [1].

- Method 1: intra reference sample filters are directly derived according to the MDIS condition.

Table 3. Coding performance of the proposed method (AI)

Table 4. Coding performance of the proposed method (RA)

Table 5. Coding performance of the proposed method (LD)

- Method 2: reference sample filtering process for planar mode is disabled based on Method 1.

Table 6. Coding performance of the proposed method (AI)

Table 7. Coding performance of the proposed method (RA)

Table 8. Coding performance of the proposed method (LD)

Conclusion

This contribution proposes to simplify the intra reference sample filter selection. The proposed method provides the minor coding performance changes. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).