| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0295 |
| --- | --- |

| Title: | On residual coding for MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne, J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@canon.com.au jonathan.gan@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

In the VVC WD7 text mts_idx is signalled in the CU footer, conditioned on the luma last position being within the upper-left 16×16 region of the TB. As this condition check could still allow significant coefficients outside the 16×16 region, either a conformance constraint was suggested to ensure such coefficients need to be zero when MTS is applied or additional decoder-side checks for further conditioning of mts_idx. This contribution proposes a scan pattern such that the last position check is sufficient to ensure no significant coefficients are present outside of the 16×16 region.

# Introduction

In VVC WD7 [1], residual coefficient scanning progresses diagonally from one sub-block to the next sub-block over a transform block (TB).

Multiple Transform Selection (MTS) is applicable to luma TBs of up to 32×32 in size, for example the TB of Figure 1. When MTS is in use, significant coefficients are anticipated only in the upper-left 16×16 region of the TB (shown in reddish shading).

Application of MTS is signalled via ‘mts_idx’, which is inferred to be zero if the last significant position is outside of the upper-left 16×16 region. Testing the last significant residual coefficient alone results in scanning sub-blocks outside of this upper-left 16×16 region (shown in orange shading). Any residual coefficients decoded in the orange sub-blocks would not be used if MTS were to be applied.

A decoder that conditions mts_idx parsing on last position alone may decode nonzero coefficients in the orange sub-blocks even if MTS is applied, a situation ideally avoided with a bitstream conformance constraint mandating that when MTS is applied only significant coefficients are present in the upper-left 16×16 region.

Alternatively, the decoder may additionally condition parsing of mts_idx on having non-significant residual coefficients outside of the upper-left 16×16 region, i.e. with the last position check, the orange sub-blocks also need to be tested.

This contribution asserts that an overly expressive syntax requiring conformance constraints or additional decoder-side checks are undesirable.

Figure 1 - Example 32x32 luma transform block scan.

# Proposal

This contribution proposes an alternative residual scan that allows the existing check on last significant position to be sufficient to determine that significant residual coefficients are present in the upper-left 16×16 region of a TB. Essentially, the sub-blocks of TBs larger in width or height of 16 samples are scanned in a tiled manner, such that the scan progresses from one 16×16 region to the next 16×16 region. This property ensures that for a TB with a last significant position is within (15,15), significant residual coefficients are present only within the upper-left 16×16 region.

Figure 2 shows an example of the proposed scan for a 32×32 TB. The corresponding scans for 32×16 and 16×32 also divide the TB into 16×16 regions. For 8×32 and 32×8 the region size is 8×16 and 16×8, respectively. Note that for 32×8 TBs, the division into regions does not change the scan compared to VVC WD7, however the 8×32 TBs, the sub-block progression is changed. For TBs of size 4×32 and 32×4, there is no change in the scan compared to VVC WD7.

Figure 2 - Proposed transform block scan (32x32 TB example).

To avoid increasing storage requirements, the scan patterns are changed globally, so chroma TBs use the same region-wise scanning even though MTS is not applicable to chroma TBs.

# Coding performance measurement

Coding performance was measured under JVET SDR common test conditions (CTC) [2] and is shown in Table 1.

Table 1. Modified scan pattern for MTS index signalling result.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.09% | 0.12% | 100% | 100% |
| Class A2 | 0.03% | 0.07% | -0.02% | 100% | 100% |
| Class B | 0.03% | 0.03% | -0.06% | 101% | 100% |
| Class C | 0.01% | 0.00% | 0.02% | 100% | 100% |
| Class E | 0.02% | -0.09% | -0.05% | 100% | 101% |
| Overall | 0.03% | -0.01% | -0.01% | 100% | 100% |
| Class D | 0.00% | 0.02% | 0.05% | 101% | 101% |
| Class F | -0.01% | 0.01% | 0.02% | 100% | 101% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.02% | 0.04% | 100% | 100% |
| Class A2 | 0.02% | 0.03% | 0.10% | 100% | 100% |
| Class B | 0.03% | -0.03% | 0.11% | 100% | 100% |
| Class C | 0.01% | 0.04% | 0.16% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.01% | 0.11% | 100% | 100% |
| Class D | -0.03% | -0.27% | -0.03% | 100% | 101% |
| Class F | 0.01% | -0.01% | -0.01% | 100% | 100% |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.52% | -0.22% | 100% | 99% |
| Class C | 0.02% | -0.21% | -0.01% | 100% | 100% |
| Class E | -0.17% | 0.30% | -0.50% | 100% | 100% |
| Overall | -0.02% | -0.21% | -0.22% | 100% | 100% |
| Class D | 0.03% | 0.30% | -0.01% | 100% | 101% |
| Class F | 0.11% | -0.65% | 0.07% | 100% | 100% |

# Conclusion

This contribution presents alternative residual coefficient scan patterns that permit MTS index signalling to be conditioned on last significant position of a TB without the need for additional bitstream conformance constraints or additional residual coefficient significance tests.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, Geneva, CH, October, 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March, 2019.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).