| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0297 |
| --- | --- |

| Title: | AHG16: Merge estimation region with constraint in HMVP update |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, Ted Hsieh, Vadim Seregin, Chun-Chi Chen, Kevin Reuzé, Marta Karczewicz (Qualcomm) Yu-Ling Hsiao, Chun-Chia Chen, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei (MediaTek) | Email: | {hanhuang, wchien}@qti.qualcomm.com {yuling.hsiao, chunchia.chen, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Qualcomm Inc., MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it’s proposed to adopt merge estimation region (MER) into VVC draft for parallel encoder design. The HMVP update is constrained to remove the dependency within a merge estimation region.  In first method, the HMVP is updated if and only if both width and height of a CU are larger or equal to the MER size. In second method, the HMVP is also updated if the CU locates at the bottom-right corner of a MER.

# Introduction

The merge estimation region (MER) was introduced in HEVC to allow independent derivation of merge candidate list for the coding units in the same MER. A candidate block that is in the same MER with the current coding unit is not included in the merge candidate list. The availability of a spatial merge candidate is set as false if xPb >> Log2ParMrgLevel is equal to xNbA1 >> Log2ParMrgLevel and yPb >> Log2ParMrgLevel is equal to yNbA1 >> Log2ParMrgLevel. Wherein Log2ParMrgLevel is equal to 2 + log2_parallel_merge_level_minus2, xPb and yPb are the coordinates of the current block, xNbA1 and yNbA1 are the coordinates of the spatial neighboring block. The MER size is adaptive and signaled as log2_parallel_merge_level_minus2 in the picture parameter set.

In VVC, it’s identified that the HEVC MER can’t be directly applied in VVC due to the flexible BT and TT split [1][2]. And the current history-based motion vector prediction (HMVP) in VVC also prevents the parallelization of merge candidate list derivation.

# Proposed techniques

In this contribution, it’s proposed to adopt MER in VVC. Two methods of normative changes to the current VVC draft are presented.

In the first method, the normative changes are as the following:

- A syntax element log2_parallel_merge_level_minus2 is added in PPS to specify the size of MER. The variable Log2ParMrgLevel is set equal to 2 + log2_parallel_merge_level_minus2

- A spatial merge candidate (either in regular merge or subblock based merge) that is derived from location (xNb, yNb) is defined as unavailable if the xPb >> Log2ParMrgLevel is equal to xNb >> Log2ParMrgLevel and yPb >> Log2ParMrgLevel is equal to yNb >> Log2ParMrgLevel.

- The updating process for the history-based motion vector predictor candidate list is enabled if and only if both cbHeight and cbWidth are larger than or equal to 1 << Log2ParMrgLevel.

In the second method, the normative changes are as the following:

- A syntax element log2_parallel_merge_level_minus2 is added in PPS to specify the size of MER. The variable Log2ParMrgLevel is set equal to 2 + log2_parallel_merge_level_minus2

- A spatial merge candidate (either in regular merge or subblock based merge) that is derived from location (xNb, yNb) is defined as unavailable if the xPb >> Log2ParMrgLevel is equal to xNb >> Log2ParMrgLevel and yPb >> Log2ParMrgLevel is equal to yNb >> Log2ParMrgLevel.

- The updating process for the history-based motion vector predictor candidate list is enabled if and only if ( (xCb+cbWidth) >> Log2ParMrgLevel > (xCb >> Log2ParMrgLevel) and ( (yCb+cbHeight) >> Log2ParMrgLevel > (yCb >> Log2ParMrgLevel).

At encoder side, non-normative constraints on BT and TT split are applied in both methods as following:

- If cbWidth > R and cbHeight <= R, then horizontal BT split is disabled for current block.

- If cbWidth <= R and cbHeight > R, then vertical BT split is disabled for current block.

- If (cbWidth > R || cbHeight > R) and cbHeight <= 2 * R, then horizontal TT split is disabled for current block.

- If cbWidth <= 2 * R and (cbWidth > R || cbHeight > R), then vertical TT split is disabled for current block.

Wherein R is equal to 1 << Log2ParMrgLevel.

# Text changes

Added text is highlight in yellow, deleted text is mark as red strike.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| log2_parallel_merge_level_minus2 | ue(v) |
| picture_header_extension_present_flag | u(1) |
| … |  |
| } |  |

log2_parallel_merge_level_minus2 plus 2 specifies the value of the variable Log2ParMrgLevel, which is used in the derivation process for spatial merging candidates as specified in clause 8.5.2.3. and the derivation process for motion vectors and reference indices in subblock merge mode as specified in clause 8.5.5.2. The value of log2_parallel_merge_level_minus2 shall be in the range of 0 to CtbLog2SizeY − 2, inclusive.

The variable Log2ParMrgLevel is set equal to log2_parallel_merge_level_minus2 + 2.

NOTE – The value of Log2ParMrgLevel indicates the built-in capability of parallel derivation of the merging candidate lists. For example, when Log2ParMrgLevel is equal to 6, the merging candidate lists for all the coding units (CUs) contained in a 64x64 block can be derived in parallel.

8.5.2	Derivation process for motion vector components and reference indices

8.5.2.1	General

…

If cbWidth  is larger or equal to (1<<Log2ParMrgLevel) and cbHeight is larger than or equal to (1<<Log2ParMrgLevel), Tthe updating process for the history-based motion vector predictor list as specified in clause 8.5.2.16 is invoked with luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], reference indices refIdxL0 and refIdxL1, prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the bi-prediction weight index bcwIdx, and the half-sample interpolation filter index hpelIfIdx.

8.5.2.3	Derivation process for spatial merging candidates

…

For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1, mvLXB1, hpelIfIdxB1 and bcwIdxB1 the following applies:

- The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1,  yCb − 1 ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbB1, yNbB1 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableB1.

- When xCb >> Log2ParMrgLevel is equal to xNbB1 >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbB1 >> Log2ParMrgLevel, availableB1 is set equal to FALSE

…

For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1, mvLXA1, hpelIfIdxA1 and bcwIdxA1 the following applies:

- The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb + cbHeight − 1 ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ). the neighbouring luma location ( xNbA1, yNbA1 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableA1.

- When xCb >> Log2ParMrgLevel is equal to xNbA1 >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbA1 >> Log2ParMrgLevel, availableA1 is set equal to FALSE

…

For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0, mvLXB0, hpelIfIdxB0 and bcwIdxB0 the following applies:

- The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbB0, yNbB0 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableB0.

- When xCb >> Log2ParMrgLevel is equal to xNbB0 >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbB0 >> Log2ParMrgLevel, availableB0 is set equal to FALSE

…

For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0, mvLXA0, hpelIfIdxA0 and bcwIdxA0 the following applies:

- The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbWidth ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ). the neighbouring luma location ( xNbA0, yNbA0 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableA0.

- When xCb >> Log2ParMrgLevel is equal to xNbA0 >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbA0 >> Log2ParMrgLevel, availableA0 is set equal to FALSE

…

For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2, mvLXB2, hpelIfIdxB2 and bcwIdxB2 the following applies:

- The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ).

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbB2, yNbB2 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableB2.

- When xCb >> Log2ParMrgLevel is equal to xNbB2 >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbB2 >> Log2ParMrgLevel, availableB2 is set equal to FALSE

…

8.5.5.2	Derivation process for motion vectors and reference indices in subblock merge mode

- When sps_sbtmvp_enabled_flag is equal to 1, the following applies:

- For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 the following applies:

    - The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1,  yCb + cbHeight − 1 ).

    - The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbA1, yNbA1 ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableA1.

    - When xCb >> Log2ParMrgLevel is equal to xNbA1 >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbA1 >> Log2ParMrgLevel, availableA1 is set equal to FALSE

…

- When sps_affine_enabled_flag is equal to 1, the variable availableFlagA is set equal to FALSE and the following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ):

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbAk, yNbAk ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableAk.

- When xCb >> Log2ParMrgLevel is equal to xNbAk >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbAk >> Log2ParMrgLevel, availableAk is set equal to FALSE

- When sps_affine_enabled_flag is equal to 1, the variable availableFlagB is set equal to FALSE and the following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ):

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ), the neighbouring luma location ( xNbBk, yNbBk ), checkPredModeY set equal to TRUE, and cIdx set equal to 0 as inputs, and the output is assigned to the block availability flag availableBk.

- When xCb >> Log2ParMrgLevel is equal to xNbBk >> Log2ParMrgLevel and yCb >> Log2ParMrgLevel is equal to yNbBk >> Log2ParMrgLevel, availableBk is set equal to FALSE

Method 2 is differentiated by the 3rd normative change, wherein the text change in 8.5.2 is as follows:

8.5.2	Derivation process for motion vector components and reference indices

8.5.2.1	General

…

If ( (xCb+cbWidth) >> Log2ParMrgLevel > (xCb >> Log2ParMrgLevel) and ( (yCb+cbHeight) >> Log2ParMrgLevel > (yCb >> Log2ParMrgLevel), Tthe updating process for the history-based motion vector predictor list as specified in clause 8.5.2.16 is invoked with luma motion vectors mvL0[ 0 ][ 0 ] and mvL1[ 0 ][ 0 ], reference indices refIdxL0 and refIdxL1, prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ], the bi-prediction weight index bcwIdx, and the half-sample interpolation filter index hpelIfIdx.

# Test Results

The test results of method 1for different MER sizes are shown from Table 1 to Table 5.

Table 1 Test results of method 1 for MER size 128x128

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.91% | 2.52% | 3.00% | 110% | 99% |
| Class A2 | 4.45% | 5.12% | 5.42% | 112% | 93% |
| Class B | 4.24% | 3.84% | 4.01% | 114% | 98% |
| Class C | 4.74% | 4.91% | 5.19% | 112% | 96% |
| Class E |  |  |  |  |  |
| Overall | 4.15% | 4.12% | 4.40% | 112% | 97% |
| Class D | 4.52% | 4.57% | 4.81% | 110% | 99% |
| Class F | 2.48% | 2.66% | 2.74% | 110% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 5.50% | 5.41% | 6.21% | 104% | 92% |
| Class E | 3.75% | 3.20% | 2.92% | 100% | 92% |
| Overall |  |  |  |  |  |
| Class D | 5.49% | 5.81% | 7.20% | 104% | 94% |
| Class F |  |  |  |  |  |

Table 2 Test results of method 1 for MER size 64x64

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.47% | 1.38% |  | 1.61% |  | 107% |  | 102% |  |
| Class A2 | 2.77% | 3.33% |  | 3.55% |  | 106% |  | 95% |  |
| Class B | 2.78% | 2.51% |  | 2.61% |  | 107% |  | 99% |  |
| Class C | 3.54% | 3.68% |  | 3.98% |  | 106% |  | 92% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 2.72% | 2.76% |  | 2.97% |  | 107% |  | 97% |  |
| Class D | 3.59% | 3.54% |  | 3.84% |  | 106% |  | 97% |  |
| Class F | 1.70% | 1.87% |  | 1.94% |  | 104% |  | 98% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.97% |  | 2.45% |  | 2.40% |  | 107% |  | 100% |  |
| Class C | 4.19% |  | 3.98% |  | 4.52% |  | 103% |  | 93% |  |
| Class E | 1.97% |  | 2.38% |  | 1.49% |  | 98% |  | 93% |  |
| Overall | 3.13% |  | 2.94% |  | 2.88% |  | 103% |  | 96% |  |
| Class D | 4.44% |  | 4.62% |  | 5.91% |  | 103% |  | 96% |  |
| Class F | 2.65% |  | 2.45% |  | 3.36% |  | 103% |  | 102% |  |

Table 3 Test results of method 1 for MER size 32x32

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.09% | 2.32% |  | 2.88% |  | 64% |  | 102% |  |
| Class A2 | 3.33% | 3.91% |  | 3.88% |  | 68% |  | 97% |  |
| Class B | 2.48% | 2.70% |  | 2.88% |  | 71% |  | 100% |  |
| Class C | 2.72% | 2.67% |  | 2.89% |  | 75% |  | 93% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 2.64% | 2.86% |  | 3.08% |  | 70% |  | 98% |  |
| Class D | 2.51% | 2.33% |  | 2.57% |  | 83% |  | 102% |  |
| Class F | 1.84% | 1.93% |  | 2.05% |  | 80% |  | 98% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.83% | 3.50% | 3.59% | 68% | 103% |
| Class C | 3.13% |  | 3.32% |  | 3.61% |  | 74% |  | 97% |  |
| Class E | 3.27% |  | 3.73% |  | 2.93% |  | 70% |  | 95% |  |
| Overall | 3.04% | 3.50% | 3.43% | 71% | 99% |
| Class D | 3.09% |  | 3.51% |  | 3.48% |  | 84% |  | 96% |  |
| Class F | 3.42% | 3.62% | 4.45% | 77% | 102% |

Table 4 Test results of method 1 for MER size 16x16

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.64% | 1.98% |  | 2.45% |  | 65% |  | 102% |  |
| Class A2 | 3.21% | 4.29% |  | 4.14% |  | 62% |  | 97% |  |
| Class B | 2.48% | 3.20% |  | 3.22% |  | 63% |  | 100% |  |
| Class C | 3.30% | 3.78% |  | 4.04% |  | 59% |  | 93% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 2.68% | 3.33% |  | 3.47% |  | 62% |  | 98% |  |
| Class D | 2.82% | 2.97% |  | 2.97% |  | 63% |  | 96% |  |
| Class F | 2.62% | 3.05% |  | 3.24% |  | 73% |  | 99% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.86% | 4.20% | 4.33% | 62% | 102% |
| Class C | 3.84% |  | 4.64% |  | 5.20% |  | 56% |  | 95% |  |
| Class E | 3.59% |  | 4.35% |  | 3.47% |  | 72% |  | 88% |  |
| Overall | 3.37% | 4.39% | 4.41% | 62% | 96% |
| Class D | 3.51% |  | 4.58% |  | 3.79% |  | 62% |  | 100% |  |
| Class F | 5.08% | 6.55% | 7.10% | 68% | 103% |

Table 5 Test results of method 1 for MER size 8x8

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.43% | 0.44% |  | 0.72% |  | 84% |  | 101% |  |
| Class A2 | 1.03% | 1.56% |  | 1.69% |  | 81% |  | 97% |  |
| Class B | 1.05% | 1.52% |  | 1.52% |  | 80% |  | 100% |  |
| Class C | 2.34% | 3.06% |  | 3.22% |  | 74% |  | 94% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.26% | 1.72% |  | 1.85% |  | 79% |  | 98% |  |
| Class D | 2.41% | 3.20% |  | 3.28% |  | 73% |  | 97% |  |
| Class F | 1.83% | 2.42% |  | 2.51% |  | 83% |  | 98% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.14% |  | 1.80% |  | 2.24% |  | 82% |  | 103% |  |
| Class C | 2.45% |  | 3.18% |  | 3.53% |  | 72% |  | 94% |  |
| Class E | 1.24% |  | 1.63% |  | 1.42% |  | 88% |  | 96% |  |
| Overall | 1.60% |  | 2.22% |  | 2.47% |  | 80% |  | 98% |  |
| Class D | 3.04% |  | 4.42% |  | 4.71% |  | 70% |  | 93% |  |
| Class F | 3.73% | 4.29% | 6.11% | 80% | 103% |

The test results of method 2 for different MER sizes are shown from Table 6 to Table 10.

Table 6 Test results of method 2 for MER size 128x128

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.66% | 2.37% | 2.70% | 108% | 100% |
| Class A2 | 4.01% | 4.46% | 4.74% | 108% | 93% |
| Class B | 3.75% | 3.48% | 3.51% | 111% | 99% |
| Class C | 4.53% | 4.65% | 4.99% | 111% | 97% |
| Class E |  |  |  |  |  |
| Overall | 3.79% | 3.76% | 3.99% | 110% | 97% |
| Class D | 4.46% | 4.39% | 4.91% | 110% | 99% |
| Class F | 2.34% | 2.47% | 2.60% | 109% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 4.20% | 3.80% | 4.04% | 108% | 102% |
| Class C | 5.36% | 5.26% | 5.69% | 104% | 93% |
| Class E | 3.86% | 4.11% | 3.27% | 100% | 94% |
| Overall | 4.50% | 4.36% | 4.40% | 105% | 97% |
| Class D | 5.43% | 5.76% | 7.29% | 105% | 94% |
| Class F | 3.88% | 3.68% | 4.66% | 106% | 103% |

Table 7 Test results of method 2 for MER size 64x64

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.22% | 1.04% |  | 1.33% |  | 105% |  | 102% |  |
| Class A2 | 2.24% | 2.49% |  | 2.70% |  | 104% |  | 96% |  |
| Class B | 2.26% | 2.10% |  | 2.18% |  | 106% |  | 100% |  |
| Class C | 3.12% | 3.16% |  | 3.33% |  | 105% |  | 93% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 2.28% | 2.25% |  | 2.42% |  | 105% |  | 98% |  |
| Class D | 3.24% | 3.04% |  | 3.39% |  | 105% |  | 95% |  |
| Class F | 1.50% | 1.66% |  | 1.73% |  | 105% |  | 99% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.40% |  | 1.94% |  | 1.95% |  | 106% |  | 102% |  |
| Class C | 3.68% |  | 3.40% |  | 3.82% |  | 103% |  | 95% |  |
| Class E | 1.85% |  | 1.99% |  | 0.86% |  | 100% |  | 97% |  |
| Overall | 2.69% |  | 2.44% |  | 2.30% |  | 104% |  | 98% |  |
| Class D | 4.04% |  | 4.08% |  | 4.83% |  | 105% |  | 98% |  |
| Class F | 2.34% |  | 2.49% |  | 3.08% |  | 105% |  | 103% |  |

Table 8 Test results of method 2 for MER size 32x32

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.04% | 2.23% |  | 2.75% |  | 64% |  | 102% |  |
| Class A2 | 3.03% | 3.35% |  | 3.39% |  | 67% |  | 97% |  |
| Class B | 2.24% | 2.52% |  | 2.55% |  | 71% |  | 102% |  |
| Class C | 2.37% | 2.20% |  | 2.46% |  | 75% |  | 95% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 2.39% | 2.54% |  | 2.73% |  | 70% |  | 99% |  |
| Class D | 2.23% | 2.09% |  | 2.29% |  | 83% |  | 96% |  |
| Class F | 1.70% | 1.77% |  | 1.85% |  | 81% |  | 100% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.65% |  | 3.67% |  | 3.73% |  | 69% |  | 106% |  |
| Class C | 2.83% |  | 2.93% |  | 3.31% |  | 74% |  | 96% |  |
| Class E | 3.28% |  | 4.07% |  | 3.19% |  | 70% |  | 95% |  |
| Overall | 2.87% |  | 3.53% |  | 3.46% |  | 71% |  | 100% |  |
| Class D | 2.75% |  | 3.21% |  | 3.56% |  | 83% |  | 98% |  |
| Class F | 3.16% |  | 3.24% |  | 4.38% |  | 78% |  | 105% |  |

Table 9 Test results of method 2 for MER size 16x16

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.66% | 2.07% |  | 2.48% |  | 64% |  | 101% |  |
| Class A2 | 3.21% | 4.23% |  | 4.13% |  | 62% |  | 97% |  |
| Class B | 2.49% | 3.15% |  | 3.20% |  | 63% |  | 100% |  |
| Class C | 3.20% | 3.62% |  | 3.90% |  | 59% |  | 95% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 2.66% | 3.27% |  | 3.43% |  | 62% |  | 99% |  |
| Class D | 2.73% | 2.71% |  | 2.74% |  | 63% |  | 99% |  |
| Class F | 2.55% | 2.93% |  | 3.10% |  | 73% |  | 99% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.90% |  | 4.28% |  | 4.15% |  | 62% |  | 103% |  |
| Class C | 3.77% |  | 4.67% |  | 4.93% |  | 56% |  | 95% |  |
| Class E | 3.50% |  | 4.44% |  | 3.34% |  | 74% |  | 92% |  |
| Overall | 3.34% |  | 4.45% |  | 4.21% |  | 63% |  | 97% |  |
| Class D | 3.41% |  | 4.68% |  | 4.31% |  | 61% |  | 98% |  |
| Class F | 4.83% |  | 6.28% |  | 6.79% |  | 68% |  | 105% |  |

Table 10 Test results of method 2 for MER size 8x8

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.43% | 0.42% |  | 0.71% |  | 85% |  | 102% |  |
| Class A2 | 1.06% | 1.64% |  | 1.68% |  | 81% |  | 97% |  |
| Class B | 1.05% | 1.52% |  | 1.49% |  | 80% |  | 99% |  |
| Class C | 2.31% | 3.02% |  | 3.19% |  | 74% |  | 94% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 1.27% | 1.72% |  | 1.82% |  | 80% |  | 98% |  |
| Class D | 2.43% | 3.26% |  | 3.23% |  | 74% |  | 97% |  |
| Class F | 1.87% | 2.46% |  | 2.54% |  | 84% |  | 100% |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.14% |  | 1.80% |  | 1.91% |  | 83% |  | 104% |  |
| Class C | 2.49% |  | 3.23% |  | 3.54% |  | 72% |  | 94% |  |
| Class E | 1.25% |  | 1.58% |  | 1.16% |  | 87% |  | 93% |  |
| Overall | 1.62% |  | 2.22% |  | 2.27% |  | 80% |  | 98% |  |
| Class D | 3.07% |  | 4.33% |  | 4.40% |  | 71% |  | 96% |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Semih Esenlik, Han Gao, Biao Wang, Anand Meher Kotra, Jianle Chen, “Non-CE4: Parallel Merge Estimation for VVC”, JVET-L0216, 12th Meeting: Macao, CN, 3–12 Oct. 2018

- Han Gao, Semih Esenlik, Biao Wang, Anand Meher Kotra, Jianle Chen, “CE4: Parallel Merge Estimation for VVC (CE4.3.2)”, JVET-M0289, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019