| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0299 |
| --- | --- |

| Title: | Residual coding simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungah Choi Sunmi Yoo Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 jungah.choi@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VVC draft, Rice parameter for transform skip is derived from the predetermined equation. However, a look-up table of 32 entries is still used to derive Rice parameter in regular residual coding. This contribution proposes a few candidate equations to derive the Rice parameter without the look-up table. The proposed equations require shift and additions, and it generates the exactly same or almost same Rice parameters as the look-up table. The coding performance of each proposed method is shown below:

Method 1:

Exactly same with the VTM-7.0 anchor

Method 2:

CTC QPs: 0.00% (AI), 0.00% (RA), -0.03% (LD)

Low QPs: -0.02% (AI), 0.08% (RA), 0.06% (LD)

Method 3:

CTC QPs: 0.00% (AI), 0.01% (RA), 0.02% (LD)

Low QPs: 0.00% (AI), 0.00% (RA), -0.01% (LD)

Introduction

In 16th JVET meeting (Geneva, CH), simplification methods for deriving the Rice parameter of transform skip residual coding (TSRC) and deriving the variable ZeroPos[n] of regular residual coding (RRC) are adopted. From theses methods, VVC7 can remove two look-up tables in RRC and TSRC, compared VVC6. However, one look-up table for deriving the Rice parameter in RRC (Table 1) is still remained. As shown in Table 1, the Rice parameter is determined by the sum of the absolute values of fully reconstructed coefficient levels of five neighboring coefficients (indicated by locSumAbs).

Table 1. Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

Proposed methods

This contribution proposes three simplified methods for deriving the Rice parameter associated with the binarization of the remaining absolute levels (abs_remainder) and the intermediate level value (dec_abs_level). Instead of the look-up table, various Rice parameter derivation equations are proposed.

Method 1

In the proposed method 1, the Rice parameter is determined by the following equation:

RiceParam = locSumAbs / 7

cRiceParam = (RiceParam > 2)? RiceParam -1 : RiceParam

The derived Rice parameter from the above equation is exactly same with VVC7 so that there is no coding performance change. In this way, the 32 entries in Table 1 associated with deriving the Rice parameter in RRC can be removed.

Method 2

In the proposed method 2, the Rice parameter is determined by the following equation:

RiceParam = (locSumAbs + 2) >> 3

cRiceParam = (RiceParam > 2)? RiceParam -1 : RiceParam

The derived Rice parameter from the above equation is slightly different with VVC7. Table 2 shows output values of the proposed equation and changed values are highlighted in yellow. In this way, the 32 entries in Table 1 associated with deriving the Rice parameter in RRC can be removed.

Table 2. Derived cRiceParam from the proposed method 2

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 |

Method 3

In the proposed method 3, the Rice parameter is determined by the following equation:

cRiceParam = (locSumAbs < 14)? (locSumAbs + 1) >> 3 : (locSumAbs + 18) >> 4

The derived Rice parameter from the above equation is slightly different with VVC7. Table 3 shows output values of the proposed equation and changed values are highlighted in yellow. In this way, the 32 entries in Table 1 associated with deriving the Rice parameter in RRC can be removed.

Table 3. Derived cRiceParam from the proposed method 3

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 |

Experimental results

From Table 4 to Table 6, the simulation results of the proposed methods are presented. These simulations are conducted using VTM-7.0 under common test conditions [1] with standard QPs and low QPs.

Table 4. Experimental results of method 1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 102% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 98% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| TGM | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 101% | 97% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| TGM | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - | - |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 98% | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 101% | 99% |

Table 5. Experimental results of method 2

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | 0.06% | 101% | 100% | 0.01% | 0.00% | 0.01% | 99% | 98% |
| Class A2 | -0.01% | -0.04% | -0.04% | 100% | 100% | 0.04% | 0.05% | 0.06% | 99% | 99% |
| Class B | 0.00% | 0.03% | -0.06% | 100% | 100% | -0.03% | 0.00% | 0.01% | 100% | 101% |
| Class C | 0.00% | 0.00% | -0.02% | 100% | 97% | -0.05% | -0.06% | -0.05% | 101% | 104% |
| Class E | 0.00% | -0.09% | 0.02% | 101% | 98% | -0.03% | 0.01% | -0.03% | 100% | 100% |
| Overall | 0.00% | -0.03% | -0.01% | 100% | 99% | -0.02% | 0.00% | 0.00% | 100% | 101% |
| Class D | -0.02% | 0.05% | 0.06% | 101% | 99% | -0.05% | -0.07% | -0.05% | 101% | 102% |
| Class F | 0.00% | -0.03% | 0.08% | 100% | 96% | 0.01% | -0.05% | -0.02% | 100% | 101% |
| TGM | -0.01% | 0.03% | 0.02% | 100% | 101% | 0.02% | -0.02% | -0.02% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.12% | 0.05% | 100% | 101% | 0.06% | 0.07% | 0.04% | 102% | 100% |
| Class A2 | -0.04% | -0.04% | -0.03% | 100% | 101% | 0.15% | 0.24% | 0.24% | 101% | 99% |
| Class B | -0.01% | 0.12% | 0.09% | 100% | 100% | 0.06% | 0.08% | 0.08% | 102% | 104% |
| Class C | 0.01% | -0.06% | -0.11% | 100% | 100% | 0.05% | 0.07% | 0.06% | 100% | 101% |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% | 0.08% | 0.11% | 0.10% | 101% | 101% |
| Class D | -0.02% | -0.20% | 0.01% | 100% | 100% | 0.06% | 0.16% | 0.06% | 100% | 99% |
| Class F | -0.01% | 0.01% | 0.02% | 100% | 100% | -0.02% | 0.05% | 0.04% | 101% | 102% |
| TGM | -0.01% | -0.04% | 0.02% | 100% | 99% | 0.04% | 0.02% | 0.02% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - | - |
| Class B | -0.03% | -0.28% | 0.02% | 100% | 99% | 0.08% | 0.09% | 0.09% | 100% | 99% |
| Class C | 0.01% | -0.30% | 0.02% | 100% | 100% | 0.08% | 0.12% | 0.15% | 100% | 99% |
| Class E | -0.07% | 0.06% | 0.15% | 100% | 101% | 0.03% | 0.00% | 0.01% | 100% | 99% |
| Overall | -0.03% | -0.20% | 0.06% | 100% | 100% | 0.06% | 0.08% | 0.09% | 100% | 99% |
| Class D | -0.01% | 0.16% | -0.15% | 100% | 100% | 0.08% | 0.20% | 0.07% | 100% | 98% |
| Class F | -0.02% | -0.13% | 0.48% | 100% | 100% | 0.01% | -0.02% | -0.16% | 100% | 100% |
| TGM | -0.09% | -0.18% | -0.14% | 100% | 100% | -0.13% | -0.17% | -0.18% | 101% | 100% |

Table 6. Experimental results of method 3

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | -0.01% | 101% | 100% | 0.00% | -0.03% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | -0.05% | 101% | 100% | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class B | 0.00% | 0.03% | -0.03% | 101% | 99% | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.05% | 0.04% | 101% | 97% | 0.01% | 0.01% | 0.03% | 101% | 100% |
| Class E | 0.00% | -0.01% | -0.01% | 101% | 98% | -0.02% | 0.05% | 0.03% | 100% | 100% |
| Overall | 0.00% | 0.01% | -0.01% | 101% | 99% | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class D | -0.01% | 0.03% | -0.02% | 101% | 99% | 0.05% | -0.01% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.01% | 0.00% | 101% | 96% | 0.02% | 0.00% | 0.02% | 101% | 99% |
| TGM | 0.01% | 0.02% | 0.00% | 101% | 100% | 0.06% | -0.01% | 0.01% | 101% | 95% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.10% | 100% | 100% | -0.01% | 0.02% | 0.00% | 102% | 99% |
| Class A2 | -0.02% | 0.07% | -0.02% | 100% | 101% | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 100% | 0.00% | -0.01% | -0.01% | 102% | 104% |
| Class C | 0.04% | -0.01% | -0.06% | 100% | 100% | 0.00% | -0.03% | -0.05% | 101% | 101% |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | 0.01% | 0.02% | 0.00% | 100% | 100% | 0.00% | -0.01% | -0.01% | 102% | 101% |
| Class D | -0.02% | -0.10% | 0.21% | 100% | 100% | 0.00% | 0.08% | -0.09% | 101% | 98% |
| Class F | -0.02% | 0.05% | -0.03% | 100% | 101% | -0.05% | 0.05% | -0.06% | 101% | 101% |
| TGM | 0.03% | 0.02% | 0.03% | 100% | 101% | 0.06% | 0.01% | 0.01% | 102% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - | - |
| Class B | 0.01% | -0.12% | -0.08% | 100% | 100% | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class C | 0.03% | -0.03% | -0.03% | 100% | 101% | -0.01% | -0.01% | -0.02% | 100% | 99% |
| Class E | 0.02% | 0.48% | -0.48% | 100% | 101% | -0.02% | 0.02% | 0.01% | 100% | 101% |
| Overall | 0.02% | 0.06% | -0.16% | 100% | 101% | -0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.03% | 0.45% | -0.13% | 100% | 99% | -0.02% | 0.02% | -0.11% | 100% | 98% |
| Class F | -0.06% | -0.37% | -0.03% | 100% | 99% | 0.01% | 0.10% | -0.14% | 100% | 99% |
| TGM | 0.00% | 0.00% | -0.03% | 101% | 99% | -0.07% | -0.22% | -0.28% | 101% | 101% |

Conclusion

This contribution proposes the simplified methods for deriving the Rice parameter without using look-up table. The experimental results reportedly show all three proposed methods have negligible impact on BD rate performance while saving 32 look-up table entries.

References

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).