| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0301 |
| --- | --- |

| Title: | CE5-related: CCALF filter for 4:2:2 and 4:4:4 color format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Junghak Nam Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, junghak.nam, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to use simplified cross-component ALF (CCALF) filter shape for 4:2:2 and 4:4:4 color format. Considering the different chroma sampling structure of 4:2:0, 4:2:2 and 4:4:4, the CCALF filter shape is reduced from 3x4 diamond (non-square 8-tap) to 3x3 diamond (square 5-tap) in 4:2:2 and 4:4:4 color format, so that the symmetrical luma samples can be used for the chroma refinement and the number of CCALF operations could be reduced by 38% without any syntax change of the CCALF common base.

Experimental results for the proposed method with 4:4:4 test sequences show 0.08%, -1.33%, -1.95% BD-rate changes on Y, Cb, and Cr components in AI, 0.10%, -1.44%, -2.32% in RA, and -0.04%, -2.49%, -3.64% in LDB configuration, compare to the VTM7.0 anchor.

In addition, compared to the common base of CCALF, it provides -0.01%, 0.15%, 0.16% BD-rate changes in AI, -0.02%, 0.17%, 0.22% in RA, and 0.02%, 0.36%, 0.26% in LDB configuration

Proposed method

In the common base of cross-component ALF (CCALF)[1], non-square 3x4 diamond filter (8-tap) is designed because of the sampling structure of 4:2:0 chroma format shown in Fig. 1, so that the chroma sample is refined using the co-located neighbor 8 luma symmetry samples. And this filter is also used in the 4:2:2 and 4:4:4 color format as shown in Fig 2, so that the chroma sample is refined using the co-located neighbor 8 luma non-symmetry samples.

In this proposal, square 3x3 diamond filter (5-tap) is introduced for CCALF in 4:2:2 and 4:4:4 color format, so that the chroma sample can be refined using co-located neighbor 5 luma symmetry samples, as shown in Fig. 3. With the reduced filter coefficient (8-tap to 5-tap), CCALF operation complexity for 4:2:2 and 4:4:4 color format could be reduced by 38%, compare to the current common base of CCALF.

Figure 1. 4:2:0 chroma format sampling structure and associated 3x4 diamond filter

Figure 2. 4:2:2 and 4:4:4 chroma format sampling structure and associated 3x4 diamond filter

Figure 3. 4:2:2 and 4:4:4 chroma format sampling structure and associated 3x3 diamond filter

For the minimal change of the draft text of CCALF common base, only some semantics are added for this proposal, as below:

… - The array of cross component filter coefficients f[ j ] is derived as follows with j = 0..7: f[ j ] = CcAlfCoeff[ j ] - The variable sum is derived as follows: - If SubHeightC is equal to 2, the following applies: sum = f[ 0 ] * recPictureL[ hx, vy + yM1 ] + f[ 1 ] * recPictureL[ hx+xM1, vy ] + f[ 2 ] * recPictureL[ hx, vy ] + f[ 3 ] * recPictureL[ hx+xP1, vy ] + ) f[ 4 ] * recPictureL[ hx+xM1, vy+yP1 ] + f[ 5 ] * recPictureL[ hx, vy+yP1 ] + f[ 6 ] * recPictureL[ hx+xP1, vy+yP1 ] + f[ 7 ] * recPictureL[ hx, vy + yP2 ] - Otherwise (SubHeightC is equal to 1), the following applies: sum = f[ 0 ] * recPictureL[ hx, vy + yM1 ] + f[ 1 ] * recPictureL[ hx+xM1, vy ] + f[ 2 ] * recPictureL[ hx, vy ] + f[ 3 ] * recPictureL[ hx+xP1, vy ] + xx) f[ 4 ] * recPictureL[ hx, vy+yP1 ] + deltaBitDepth = BitDepthY − BitDepthC scaledSum = ( sum + ( 1 << ( 6 + deltaBitDepth ) ) ) >> ( 7 + deltaBitDepth ) scaledSum = Clip3( −( 1 << ( BitDepthC − 1 ) ), ( 1 << ( BitDepthC − 1 ) ) − 1, scaledSum ) ) sum = curr + scaledSum …

As shown in the proposed semantic changes, the 8 filter coefficients of CCALF are always decoded, but only 5 filter coefficients are used for CCALF in 4:2:2 and 4:4:4 color format.

Experimental result

The proposed method has been implemented on top of CCALF common base[1], and tested under AI, RA and LDB configuration of the 4:4:4 test conditions defined in [2]. Table 1 to Table 3 show the test results of proposed method in AI, RA and LDB configuration.

Table 1 Experimental results for all-intra (AI) test condition; anchor is VTM7.0

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 DualITree=1 YUV444 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM | 0.02% | -0.47% | -0.49% | 100% | 102% |
| TGM | 0.08% | -1.16% | -1.70% | 100% | 102% |
| A | 0.10% | -0.96% | -0.86% | 100% | 103% |
| M | 0.06% | -2.41% | -2.61% | 100% | 104% |
| CC | 0.16% | -2.27% | -5.99% | 100% | 105% |
| Overall | 0.08% | -1.33% | -1.95% | 100% | 103% |

Table 2 Experimental results for random-access (RA) test condition; anchor is VTM7.0

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 DualITree=1 YUV444 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM | 0.04% | -0.35% | -0.45% | 100% | 103% |
| TGM | 0.12% | -0.63% | -1.24% | 100% | 103% |
| A | 0.11% | -1.02% | -0.75% | 100% | 100% |
| M | 0.10% | -2.38% | -2.54% | 100% | 102% |
| CC | 0.17% | -4.51% | -10.25% | 99% | 104% |
| Overall | 0.10% | -1.44% | -2.32% | 100% | 102% |

Table 3 Experimental results for low-delay B (LDB) test condition; anchor is VTM7.0

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 DualITree=1 YUV444 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM | -0.06% | -0.78% | -0.54% | 100% | 102% |
| TGM | 0.01% | -2.21% | -3.29% | 100% | 102% |
| A | 0.06% | -3.01% | -2.48% | 101% | 104% |
| M | -0.09% | -3.00% | -3.08% | 100% | 102% |
| CC | -0.19% | -4.91% | -13.13% | 101% | 106% |
| Overall | -0.04% | -2.49% | -3.64% | 100% | 103% |

Table 4 to Table 6 also show the test results of proposed method in AI, RA and LDB configuration compare to the common base of CCALF.

Table 4 Experimental results for all-intra (AI) test condition; anchor is CCALF common base

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CCALF common base DualITree=1 YUV444 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM | -0.01% | 0.09% | 0.08% | 100% | 100% |
| TGM | -0.02% | 0.15% | 0.16% | 100% | 101% |
| A | -0.01% | 0.04% | 0.13% | 100% | 99% |
| M | 0.00% | 0.22% | 0.21% | 100% | 101% |
| CC | 0.00% | 0.30% | 0.29% | 100% | 98% |
| Overall | -0.01% | 0.15% | 0.16% | 100% | 100% |

Table 5 Experimental results for random-access (RA) test condition; anchor is CCALF common base

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CCALF common base DualITree=1 YUV444 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM | -0.02% | 0.11% | 0.07% | 100% | 98% |
| TGM | -0.03% | 0.06% | 0.14% | 100% | 98% |
| A | -0.01% | 0.13% | 0.26% | 100% | 97% |
| M | -0.03% | 0.07% | 0.11% | 99% | 98% |
| CC | 0.00% | 0.74% | 0.77% | 99% | 99% |
| Overall | -0.02% | 0.17% | 0.22% | 100% | 98% |

Table 6 Experimental results for low-delay B (LDB) test condition; anchor is CCALF common base

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CCALF common base DualITree=1 YUV444 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM | 0.08% | 0.18% | 0.18% | 100% | 101% |
| TGM | 0.02% | 0.39% | 0.19% | 100% | 100% |
| A | -0.02% | 0.30% | 0.30% | 101% | 102% |
| M | -0.05% | 0.15% | 0.16% | 99% | 98% |
| CC | 0.10% | 1.07% | 0.67% | 100% | 101% |
| Overall | 0.02% | 0.36% | 0.26% | 100% | 100% |

Conclusion

This contribution proposes to use 3x3 diamond CCALF filter shape for 4:2:2 and 4:4:4 color format. The proposed method provides the symmetrical CCALF filtering in 4:2:2 and 4:4:4 color format with the reduced computational complexity.

Reference

- K. Misra, et al., “CE5 Common Base: Cross Component Adaptive Loop Filter”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q0058, 17th meeting, Brussels, BE, 7–17 Jan. 2020.

- X. Xu, et al., “Description of Core Experiment 2 (CE2): Palette Mode Coding”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P2022, 16th meeting, Geneva, CH, 1-11 Oct. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).