| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0302 |
| --- | --- |

| Title: | Simplified contexts in intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

Theoretically, context coded bins provides better coding performance compared to bypass coded bins because it can estimate the probability more accurately. However, it requires more computational complex and delay to estimate the probability so that the number of context coded bins determines throughput efficiency of the system. This contribution investigates justification of using context coded bins for some indications of intra prediction tools, such as cclm_mode_idx, intra_luma_ref_idx, and intra_subpartitions_split_flag. Based on the investigation, following three methods are proposed.   :

- Method 1: all bins of cclm_mode_idx are coded in bypass mode.

- Method 2: second bin of intra_luma_ref_idx is coded in bypass mode.

- Method 3: intra_subpartitions_split_flag is coded in bypass mode.

Experimental results (BD-rate changes in AI configuration, compare to the VTM7.0 anchor) for the proposed method 1, 2 and 3 are as below:

- Method 1: Y 0.01%, Cb -0.08%, Cr -0.06%

- Method 2: Y 0.01%, Cb -0.05%. Cr 0.02%

- Method 3: Y 0.01%, Cb 0.00%. Cr 0.02%

Proposed method

In this proposal, three methods of simplified context are proposed to reduce the CABAC data throughput in the parsing of intra prediction syntax.

In the proposed method 1, all bins of cclm_mode_idx are coded in the bypass mode. In the current VVC, first bin of cclm_mode_idx is coded with 1 context model, and the second bin is coded with bypass mode. To simplify the contexts of cclm_mode_idx, two bins of cclm_mode_idx are coded in bypass mode, as below:

Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process. Syntax structure; Syntax element; ctxTable; initType. 0; 1; 2. coding_unit( ); cclm_mode_idx; Table 74; 0; 1; 2. Table 74 – Specification of initValue and shiftIdx for ctxInc of cclm_mode_idx Initialization variable; ctxIdx of cclm_mode_idx. 0; 1; 2. initValue; EP; EP; EP. shiftIdx; 0; 0; 0. Table 128 – Assignment of ctxInc to syntax elements with context coded bins Syntax element; binIdx. 0; 1; 2; 3; 4; >= 5. cclm_mode_idx; 0 bypass; bypass; na; na; na; na.

In the proposed method 2, second bin of intra_luma_ref_idx is coded in the bypass mode. In the current VVC, first bin of intra_luma_ref_idx is coded with 1 context model, and the second bin is also coded with 1 context model. To simplify the contexts of intra_luma_ref_idx without coding loss, second bin of intra_luma_ref_idx is coded in bypass mode, as below:

Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process. Syntax structure; Syntax element; ctxTable; initType. 0; 1; 2. coding_unit( ); intra_luma_ref_idx[ ][ ]; Table 67; 0 0..1; 1 2..3; 2 4..5. Table 67 – Specification of initValue and shiftIdx for ctxIdx of intra_luma_ref_idx Initialization variable; ctxIdx of intra_luma_ref_idx. 0; 1; 2; 3; 4; 5. initValue; EP; EP; EP; EP; EP; EP. shiftIdx; 0; 0; 0; 0; 0; 0. Table 128 – Assignment of ctxInc to syntax elements with context coded bins Syntax element; binIdx. 0; 1; 2; 3; 4; >= 5. intra_luma_ref_idx[ ][ ]; 0; 1bypass; na; na; na; na.

In the proposed method 3, intra_subpartitions_split_flag is coded in the bypass mode. In the current VVC, intra_subpartitions_split_flag is coded with 1 context model. To simplify the contexts of intra_subpartitions_split_flag, it is coded in bypass mode, as below:

Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process. Syntax structure; Syntax element; ctxTable; initType. 0; 1; 2. coding_unit( ); intra_subpartitions_split_flag; Table 69; 0; 1; 2. Table 69 – Specification of initValue and shiftIdx for ctxInc of intra_subpartitions_split_flag Initialization variable; ctxIdx of intra_subpartitions_split_flag. 0; 1; 2. initValue; EP; EP; EP. shiftIdx; 0; 0; 0. Table 128 – Assignment of ctxInc to syntax elements with context coded bins Syntax element; binIdx. 0; 1; 2; 3; 4; >= 5. intra_subpartitions_split_flag; 0bypass; na; na; na; na; na.

Experimental result

The proposed method has been implemented using VTM7.0 as software base according to the common test conditions defined in JVET-N1010[1].

Test1 (Method 1)

In the test 1, all bins of cclm_mode_idx are coded in the bypass mode. Table 1 and Table 2 show the test results in AI, RA configuration.

Table 1 Experimental results of test1 for all-intra (AI) test condition; anchor is VTM7.0

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.25% | -0.12% | 101% | 101% |
| Class A2 | 0.01% | -0.03% | -0.10% | 101% | 100% |
| Class B | 0.01% | -0.02% | -0.14% | 101% | 99% |
| Class C | 0.01% | -0.05% | -0.04% | 101% | 98% |
| Class E | 0.00% | -0.08% | 0.12% | 101% | 99% |
| Overall | 0.01% | -0.08% | -0.06% | 101% | 99% |
| Class D | 0.03% | -0.07% | -0.06% | 101% | 99% |
| Class F | 0.03% | -0.13% | -0.07% | 101% | 97% |

Table 2 Experimental results of test1 for random-access (RA) test condition; anchor is VTM7.0

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.48% | -0.20% | 101% | 101% |
| Class A2 | -0.03% | 0.06% | -0.04% | 101% | 100% |
| Class B | 0.01% | 0.11% | 0.05% | 101% | 101% |
| Class C | 0.02% | -0.05% | -0.16% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.06% | -0.07% | 101% | 101% |
| Class D | 0.01% | -0.26% | 0.11% | 101% | 100% |
| Class F | 0.01% | -0.20% | -0.26% | 101% | 101% |

Test2 (Method 2)

In the test 2, second bin of intra_luma_ref_idx is coded in the bypass mode. Table 3 and Table 4 show the test results in AI, RA configuration

Table 3 Experimental results of test2 for all-intra (AI) test condition; anchor is VTM7.0

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.21% | 0.10% | 101% | 100% |
| Class A2 | 0.02% | 0.02% | 0.01% | 101% | 100% |
| Class B | 0.02% | -0.04% | -0.07% | 101% | 99% |
| Class C | 0.01% | -0.02% | 0.00% | 101% | 98% |
| Class E | 0.01% | -0.04% | 0.12% | 101% | 99% |
| Overall | 0.01% | -0.05% | 0.02% | 101% | 99% |
| Class D | -0.02% | -0.10% | -0.02% | 101% | 99% |
| Class F | -0.03% | 0.03% | 0.06% | 100% | 97% |

Table 4 Experimental results of test2 for random-access (RA) test condition; anchor is VTM7.0

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.04% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.06% | 0.06% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.16% | 100% | 101% |
| Class C | -0.01% | 0.00% | -0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.06% | 100% | 100% |
| Class D | -0.02% | -0.12% | -0.18% | 100% | 100% |
| Class F | -0.01% | -0.06% | 0.02% | 100% | 102% |

Test3 (Method 3)

In the test 3, intra_subpartitions_split_flag is coded in the bypass mode. Table 5 and Table 6 show the test results in AI, RA configuration

Table 5 Experimental results of test3 for all-intra (AI) test condition; anchor is VTM7.0

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.08% | 101% | 99% |
| Class A2 | 0.01% | 0.00% | -0.03% | 101% | 99% |
| Class B | 0.02% | 0.02% | -0.02% | 101% | 99% |
| Class C | 0.00% | 0.04% | -0.01% | 101% | 99% |
| Class E | 0.02% | -0.02% | 0.13% | 101% | 98% |
| Overall | 0.01% | 0.00% | 0.02% | 101% | 99% |
| Class D | 0.00% | -0.11% | -0.05% | 101% | 99% |
| Class F | 0.01% | 0.06% | 0.13% | 101% | 97% |

Table 6 Experimental results of test3 for random-access (RA) test condition; anchor is VTM7.0

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | -0.04% | 101% | 100% |
| Class A2 | 0.01% | 0.08% | 0.05% | 101% | 100% |
| Class B | 0.02% | 0.09% | 0.18% | 100% | 101% |
| Class C | 0.02% | 0.04% | -0.12% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.03% | 101% | 100% |
| Class D | -0.02% | -0.20% | -0.07% | 100% | 100% |
| Class F | 0.00% | -0.08% | 0.04% | 100% | 101% |

Conclusion

In this proposal, three methods of simplified context are proposed to reduce the CABAC data throughput in the parsing of intra prediction syntax with the negligible coding performance change.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th meeting, Geneva, CH, 19-27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).