| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0304-v1 |
| --- | --- |

| Title: | Non-CE5: Non-Linear Cross Component Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Yao, L.-H. Xu, J.-Q. Zhu, K. Kazui 601,Yongda International Tower, 2277 Longyang Road Shanghai, 201204, China | Tel: Email: | +86-21-63350606 ext. 5809 yaojie@cn.fujitsu.com xuluhang@cn.fujitsu.com zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This contribution introduce Non-Linear Cross Component Adaptive Loop Filter (NL-CCALF) which make CCALF more efficient by using a simple clipping function to reduce the impact of neighbor sample values when they are too different with the current sample value being filtered.

The results reportedly show the average BDR gains and encoding/decoding runtimes are respectively -0.08% - 91%/102% for the All Intra configuration, -0.11% - 105%/101% for Random Access configuration, and -0.16% - 104%/100% for Low Delay configuration over CE5 anchor using the AHG13 YUV metric.

Introduction

In 16th JVET meeting, Cross Component Adaptive Loop Filter (CC-ALF) was proposed and studied in JVET-P1008 [1]. CC-ALF makes use of luma sample values to refine chroma components. It operates by applying an adaptive linear filter to the luma channel and then using the output of this filtering operation for the chroma components refinement. Figure 1 illustrates the placement of CC-ALF with respect to the other loop filters.

Filtering in CC-ALF is accomplished by applying a linear, diamond shaped filter (Figure 2) to the luma sample values and generate a residual correction for the chroma channels, which is expressed as

ΔIix,y=x0,y0∈SiI0xC+x0,yC+y0cix0,y0, (1)

Where x,y is chroma component i location being refined. xC,yC is the luma location based on x,y. Si is filter support in luma for chroma component i. cix0,y0 represents the filter weights. I0xC+x0,yC+y0 is input luma samples.

Equation (2) can be reformulated, without coding efficiency impact, in the following expression:

ΔIix,y=(x0,y0)≠(0,0)(x0,y0)∈SiI0xC+x0,yC+y0-I0xC,yCcix0,y0, (2)

Using equation (2), we can introduce non linearity to make CC-ALF more efficient by using a clipping function to reduce the impact of neighbor sample values.

Figure 1. Placement of CC-ALF with respect to other loop filters

Si

(xC,yC)

Figure 2. 3x4 Diamond Shape Filter with 8 unique weights

Proposed method

This contribution propose Non-Linear Cross Component Adaptive Loop Filter (NL-CCALF) to further improve CCALF coding efficient. Based on equation (2), we use a clipping function to reduce the impact of neighbor sample values when they are too different with the current sample value being filtered. The NL-CCALF filtering process is expressed as

ΔIix,y=(x0,y0)≠(0,0)(x0,y0)∈SiKI0xC+x0,yC+y0-I0xC,yC,k(x0,y0)cix0,y0, (4)

Where Kd,b=min⁡(b,max-b,d) is the clipping function, d is input, b is the clipping range. And k(x0,y0) are clipping parameters relate to position x0,y0. The clipping parameter is derived in table 1 depending on BitDepth of luma and clipIdx in the range of 0 to 3.

| BitDepth | clipIdx |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 8 | 255 | 50 | 10 | 2 |
| 9 | 511 | 100 | 20 | 4 |
| 10 | 1023 | 201 | 39 | 8 |
| 11 | 2047 | 402 | 79 | 15 |
| 12 | 4095 | 803 | 158 | 31 |
| 13 | 8191 | 1607 | 315 | 62 |
| 14 | 16383 | 3214 | 630 | 124 |
| 15 | 32767 | 6427 | 1261 | 247 |
| 16 | 65535 | 12855 | 2521 | 495 |

Table 1. Clipping parameter depending on BitDepth and clipIdx

The goal of this adaptive clipping is to cap the difference between the input sample value to be filtered and the neighboring input samples of the filter. The encoder performs the optimization to find the best clipping parameter for each correlated filter coefficient.

The index of clipping value (clipIdx) needs to be signaled into bit-streams. The clipping parameter are specified per filter coefficient. That means that up to 7 clipping values need to be signaled for each filter.

What’s more, A flag is signaled to indicate whether NL-CCALF is performed and associated clipping values are signaled. If the flag is false, the clipping parameters are not signaled. The flag is separately introduced for each filter. The corresponding specification of NL-CCALF signaling is shown in table 2.

The syntax and semantics for NL-CCALF are provided in the attached word document.

| alf_data( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( alf_cross_component_cb_filter_signal_flag ) { |  |
| alf_cross_component_cb_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cb_filters_signalled_minus1+1); k++ ) { |  |
| for ( j = 0; j < 7; j++ ) |  |
| alf_cross_component_cb_coeff_plus32[ k ][ j ] | u(6) |
| ccalf_cb_clip_flag[k] | u(1) |
| if( ccalf_cb_clip_flag[k] ) { |  |
| for( j = 0; j < 7; j++ ) |  |
| ccalf_cb_clip_idx[ k ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| if ( alf_cross_component_cr_ filter_signal_flag ) { |  |
| alf_cross_component_cr_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cr_filters_signalled_minus1+1); k++ ) { |  |
| ccalf_cr_clip_flag[k] | u(1) |
| for ( j = 0; j < 7; j++ ) |  |
| alf_cross_component_cr_coeff_plus32[ k ][ j ] | u(6) |
| ccalf_cr_clip_flag[k] | u(1) |
| if( ccalf_cr_clip_flag[k] ) { |  |
| for( j = 0; j < 7; j++ ) |  |
| ccalf_cr_clip_idx[ k ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |
| } |  |

Table 2. Syntax specification of NL-CCALF

Results

The proposed method is implemented upon CE5 anchor [2] and is evaluated for all-intra, random-access and low delay configuration according to the common test conditions in JVET-N1010 [3].

Table 3 shows summarized experimental results of the proposed method compared to CE5 anchor. Results show the coding gain of the proposed method to be 0.08%, 0.11%, and 0.16% using the AHG13 YUV metric for the All Intra, Random Access and Low Delay configurations, respectively. Encoding time is not reliable according to CPU resources.

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.01% | -0.15% | -0.36% | -0.04% | 92% | 102% |
| Class A2 | 0.01% | -0.87% | -1.17% | -0.20% | 87% | 102% |
| Class B | 0.01% | -0.61% | -0.37% | -0.09% | 90% | 102% |
| Class C | 0.00% | -0.16% | -0.20% | -0.03% | 95% | 101% |
| Class E | 0.00% | -0.14% | -0.20% | -0.03% | 88% | 101% |
| Overall | 0.01% | -0.40% | -0.44% | -0.08% | 91% | 102% |
| Class D | 0.00% | -0.07% | -0.07% | -0.01% | 81% | 98% |
| Class F | 0.03% | -0.45% | -0.35% | -0.06% | 90% | 101% |
|  |  |  | Random access Main10 |  |  |  |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.02% | -0.31% | -0.59% | -0.07% | 102% | 100% |
| Class A2 | 0.01% | -0.71% | -0.53% | -0.11% | 100% | 101% |
| Class B | 0.01% | -0.84% | -0.86% | -0.16% | 115% | 101% |
| Class C | -0.01% | -0.35% | -0.17% | -0.06% | 99% | 102% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | -0.58% | -0.56% | -0.11% | 105% | 101% |
| Class D | -0.02% | -0.06% | 0.03% | -0.02% | 112% | 100% |
| Class F | 0.01% | -0.62% | -0.52% | -0.11% | 112% | 100% |
|  |  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over CE5 anchor |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.03% | -1.46% | -1.07% | -0.23% | 100% | 100% |
| Class C | -0.07% | -0.28% | -0.25% | -0.11% | 111% | 100% |
| Class E | 0.00% | -0.68% | -0.47% | -0.11% | 100% | 100% |
| Overall | -0.01% | -0.87% | -0.65% | -0.16% | 104% | 100% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3. Proposed method over CE5 anchor

Conclusions

This contribution proposes Non-Linear Cross Component Adaptive Loop Filter which make CCALF more efficient by using a simple clipping function to reduce the impact of neighbor sample values when they are too different with the current sample value being filtered. Results show the coding gain of the tool to be 0.08%, 0.11%, and 0.16% using the AHG13 YUV metric for the All Intra, Random Access and Low Delay configurations, respectively. It is proposed to adopt this contribution for CCALF tool.

Acknowledgments

The contribution has been kindly cross-checked by xxxx. The cross-check results are available in the contribution JVET-xxxx.

References

- K. Misra, F. Bossen, A. Segall, N. Hu, J. Dong, V. Seregin, M. Karczewicz, P. Onno, C. Gisquet, G. Laroche, J. Li, C.S. Lim, C.-W. Kuo, J. Nam, J. Choi, J. Lim, S. Kim, O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei, "CE5-related: On the design of CC-ALF, " JVET-P1008, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019.

- K. Misra, F. Bossen, A. Segall, and etc, “CE5 Common Base: Cross Component Adaptive Loop Filter,” JVET-Q0058, Dec 2019.

- F. Bossen, J. Boyce, and etc, “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-N1010, May 2019.

Patent rights declaration(s)

FUJITSU LIMITED may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).