| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0306 |
| --- | --- |

| Title: | Non-CE4: Removal of MVD scaling process for 4×8/8×4 blocks in MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang, Hongbin Liu, Li Zhang, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | zhangna.cynthia@bytedance.com liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to remove the MVD scaling process in bi-predicted MMVD mode for 4×8/8×4 blocks. Specifically, if the base merge candidate for MMVD utilizes bi-prediction for a 4×8/8×4 block, the signaled MVD is directly used as the MVD of reference picture list 0. Simulation results reportedly show that the BD-rate changes are 0.00% for RA, and 0.03% for LB with almost no impact on encoding/decoding time compared to VTM-7.0.

# Introduction

In VTM-7.0, if the base merge candidate for MMVD utilizes bi-prediction, the MVD of one reference picture list X (X = 0 or 1, determined according to a certain criteria) is signaled directly and the MVD of the other reference picture list Y (Y = 1 – X) is derived from the MVD of list X according to the POC distances between the reference pictures and the current picture and the reference picture types (e.g., long-term or short-term reference picture).

However, bi-prediction is forbidden for 4×8/8×4 blocks and a bi-directional MV is converted to a uni-directional MV from reference picture list 0 for such blocks in VTM-7.0. Therefore, it is unnecessary to derive the MVD of L1 for 4×8/8×4 blocks in bi-predicted MMVD mode.

- Proposed methods

To address the abovementioned issue, for a 4×8/8×4 block coded in bi-predicted MMVD mode, the MVD of reference picture list 0 is directly signaled and the MVD of reference picture list 1 is not derived.

# Experimental Results

The proposed method has been implemented on the top of VTM-7.0. Results under RA/LDB configurations are summarized in Table 1. The simulations were performed following the the common test conditions [1]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.01% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.01% | -0.03% | 0.03% | 100% | 99% |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.00% | 99% | 100% | 0.00% | -0.25% | 0.02% | 99% | 100% |
| Class C | 0.00% | -0.01% | 0.05% | 99% | 99% | 0.01% | -0.08% | -0.11% | 99% | 99% |
| Class E |  |  |  |  |  | 0.10% | 0.43% | 0.27% | 101% | 103% |
| Overall | 0.00% | -0.01% | 0.02% | 100% | 99% | 0.03% | -0.03% | 0.04% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | -0.02% | 0.06% | -0.31% | 100% | 99% |
| Class F | 0.00% | 0.01% | -0.03% | 99% | 99% | -0.03% | -0.25% | 0.01% | 99% | 99% |
| Class SCC | -0.02% | -0.03% | -0.02% | 99% | 99% | 0.01% | -0.02% | -0.07% | 99% | 100% |

- Conclusions

This contribution removes the MVD scaling process in bi-predicted MMVD mode for 4×8/8×4 blocks. There is a minor overall coding performance change with almost no impact on encoding/decoding time compared to VTM-7.0. It is recommended to adopt this method into VVC.

- Suggested changes to working draft

The proposed changes on top of JVET-O2001-vE are specified as below, with newly added text highlighted in yellow.

8.5.2.7	Derivation process for merge motion vector difference

Inputs to this process are:

- a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- reference indices refIdxL0 and refIdxL1,

- prediction list utilization flags predFlagL0 and predFlagL1.

Outputs of this process are the luma merge motion vector differences in 1/16 fractional-sample accuracy mMvdL0 and mMvdL1.

The variable currPic specifies the current picture.

The luma merge motion vector differences mMvdL0 and mMvdL1 are derived as follows:

- If both predFlagL0 and predFlagL1 are equal to 1 and the sum of the width and height of the current luma coding block is larger than 12, the following applies:

currPocDiffL0  =  DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] )	(564)

currPocDiffL1  =  DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] )	(565)

- If currPocDiffL0 is equal to currPocDiffL1, the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(566)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(567)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(568)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(569)

- Otherwise, if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies:

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(570)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(571)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(572)

tb = Clip3( −128, 127, currPocDiffL1 )		(573)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(574)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(575)

mMvdL1[ 0 ] = Clip3( −217, 217 − 1, (distScaleFactor * mMvdL0[ 0 ] + 	(576)
	128 − ( distScaleFactor * mMvdL0[ 0 ] >= 0 ) )  >>  8 )

mMvdL1[ 1 ] = Clip3( −217, 217 − 1, (distScaleFactor * mMvdL0[ 1 ] + 	(577)
	128 − ( distScaleFactor * mMvdL0[ 1 ] >= 0 ) )  >>  8 )

    - Otherwise, the following applies:

mMvdL1[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL0[ 0 ]  :  −mMvdL0[ 0 ]	(578)

mMvdL1[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL0[ 1 ]  :  −mMvdL0[ 1 ]	(579)

- Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies:

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(580)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(581)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture, the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(582)

tb = Clip3( −128, 127, currPocDiffL0 )		(583)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(584)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(585)

mMvdL0[ 0 ] = Clip3( −217, 217 − 1, (distScaleFactor * mMvdL1[ 0 ] +	(586)
	 128 − (distScaleFactor * mMvdL1[ 0 ] >= 0) )  >>  8 )

mMvdL0[ 1 ] = Clip3( −217, 217 − 1, , (distScaleFactor * mMvdL1[ 1 ] +	(587)
	 128 − (distScaleFactor * mMvdL1[ 1 ] >= 0) )  >>  8 ) )

    - Otherwise, the following applies:

mMvdL0[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL1[ 0 ]  :  −mMvdL1[ 0 ]	(588)

mMvdL0[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL1[ 1 ]  :  −mMvdL1[ 1 ]	(589)

- Otherwise ( predFlagL0 or predFlagL1 are equal to 1 or the sum of the width and height of the current luma coding block is smaller than or equal to 12 ), the following applies for X being 0 and 1:

mMvdLX[ 0 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 0 ]  :  0	(590)

mMvdLX[ 1 ] = ( predFlagLX = = 1 )  ?  MmvdOffset[ xCb ][ yCb ][ 1 ]  :  0	(591)

- References

[1]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Mar. 2019.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).