| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0307-v2 |
| --- | --- |

| Title: | CE4-related: Block-dimension based GEO mode selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Hongbin Liu, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents two methods to reduce the number of GEO modes according to block dimensions. Method #1 is to use 32 GEO modes and the set of GEO modes is dependent on the block dimension. Method #2 is based on method #1 but provides a one-level scalability to use either 32 modes or only the first 16 modes among the 32 modes according to a SPS flag. As compared with CE4 common base, it is asserted that the number of hardware validation tests for GEO can be reduced to 33% by using the proposed 32 modes GEO. Four tests are conducted to verify the proposed methods on top of CE tests, i.e., CE4-1 + CE4-2.1 + method #1 (test 1), CE4-1 + CE4-2.1 + CE4-3.1 + method #1 (test 2), CE4-1 + CE4-2.1 + method #2 with 16 modes (test 3), and CE4-1 + CE4-2.1 + CE4-3.1 + method # 2 with 16 modes (test 4), respectively.

The results are reported as follows, as compared with CE4 common base and VTM-7.0, respectively.

| Test 1 vs. CE4 common base: | RA: 0.01%/0.05%/0.03% with 98% EncT and 100% DecT |
| --- | --- |
|  | LDB: 0.08%/-0.05%/0.05% with 100% EncT and 99% DecT |
| Test 2 vs. CE4 common base: | RA: 0.05%/0.06%/0.04% with 98% EncT and 99% DecT |
|  | LDB: 0.14%/0.01%/0.39% with 97% EncT and 99% DecT |
| Test 3 vs. CE4 common base: | RA: 0.16%/0.24%/0.27% with 98% EncT and 99% DecT |
|  | LDB: 0.43%/0.37%/0.63% with 97% EncT and 100% DecT |
| Test 4 vs. CE4 common base: | RA: 0.19%/0.26%/0.25% with 99% EncT and 99% DecT |
|  | LDB: 0.47%/0.38%/0.50% with 96% EncT and 100% DecT |
| Test 1 vs. VTM-7.0: | RA: -0.26%/-0.31%/-0.31% with 101% EncT and 99% DecT |
|  | LDB: -0.62%/-0.72%/-0.71% with 102% EncT and 98% DecT |
| Test 2 vs. VTM-7.0: | RA: -0.22%%/-0.29%/-0.29% with 101% EncT and 98% DecT |
|  | LDB: -0.56%/-0.66%/-0.38% with 100% EncT and 98% DecT |
| Test 3 vs. VTM-7.0: | RA: -0.12%/-0.11%/-0.07% with 101% EncT and 99% DecT |
|  | LDB: -0.28%/-0.31%/-0.14% with 100% EncT and 98% DecT |
| Test 4 vs. VTM-7.0: | RA: -0.09%/-0.10%/-0.09% with 101% EncT and 99% DecT |
|  | LDB: -0.24%/-0.30%/-0.26% with 99% EncT and 98% DecT |

# This updated version provides results of test 5 (i.e., a combination of JVET-Q0307 test 1 and JVET-Q0309 test 1 with 32 GEO modes and 14 block shapes on top of CE4-1 and CE4-2.1). The results of test 5 is reported as follows.

| Test 5 vs. VTM-7.0: | RA: -0.25%/-0.28%/-0.26% with 102% EncT and 99% DecT |
| --- | --- |
|  | LDB: -0.60%/-0.59%/-0.50% with 101% EncT and 96% DecT |

# Introduction

In the core experiment 4, several tests were defined to investigate GEO. A common base of GEO was released on top of VTM-7.0, and all the related techniques are required to be implemented on top of the common base. In the CE4 common base, 82 different GEO modes are implemented as a replacement of TPM mode in VVC WD7. GEO is allowable for 19 different block shapes ranging from block size 8×8 to block size 128×128. In CE4-1, the number of GEO modes is reduced from 82 to 64. In CE4-2.1, the number of block shapes allowed for GEO is reduced from 19 to 16. In CE4-3.1, SBT is disabled for GEO modes.

At the last JVET meeting, experts concerned on the large amount of GEO modes for encoder, and several hardware experts addressed concerns on the hardware validation workload for GEO. Even for the simplified version with the joint proposal [1], 64 modes *16 block shapes=1024 validation tests are still required for GEO. Therefore, it is desirable to further reduce the GEO modes to lessen the hardware validation workload.

# Proposed methods

Figure 1. Illustration of the 24 GEO angles in CE4 common base

In this contribution, the number of GEO modes is further reduced by half, i.e., from CE4-1 64 modes to 32 modes. Figure 1 illustrates the 24 GEO angles in CE4 common base. The 32 GEO modes are selected from CE4-1 64 modes and based on the observation that narrow blocks (i.e., block height greater than block width) are likely to choose the modes with GEO angles close to the vertical direction, while wide blocks are not. To project the signalled GEO mode index (ranging from 0 to 31) to the allowed set of GEO angles and distances, it is proposed to use a block-dimension based GEO mode selection, i.e., in a way that one mapping is used for narrow blocks with height greater than width, and the other mapping is used for blocks with height not greater than width.

Two methods are proposed as follows:

Method #1: Reduce the number of GEO modes to 32 as described above.

Method #2: Based on method #1, a SPS flag is added to control whether to use 32 modes or only the first 16 modes of the 32 modes. This method enables a one-level scalability considering the 32 modes may still be a giant number for light encoder.

The proposed 32 modes in method #1 and method #2 are the same. Although a 1-bit SPS control flag is added to method #2, the 32 modes in method #2 provides identical PSNR values with method #1.

It is asserted that, the number of hardware validation tests can be reduced to 32*16=512 by the proposed 32 modes and 16 block shapes, which is 33% of the common base.

# Simulation results

To verify the proposed method #1 and method #2, the following tests are conducted:

- For method #1, two tests are conducted on top of CE tests, i.e., CE4-1+CE4-2.1+method #1 (test 1), and CE4-1+CE4-2.1+CE4-3.1+method #1 (test 2), respectively.

- For method #2,

  - The proposed 32 modes in method #2 are the same as method #1, excepting a 1-bit SPS control flag. The 32 modes in method #2 provides identical PSNR values with method #1. Therefore, we believe there is no need to list the results of method #2 with 32 modes again.

  - To verify method #2 with 16 modes, two tests are conducted on top of CE tests, i.e., CE4-1+CE4-2.1+method #2 with 16modes (test 3), and CE4-1+CE4-2.1+CE4-3.1+method #2 with 16 modes (test 4), respectively.

The tests were implemented on top of CE4 common base [2] under JVET CTC conditions [3]. As compared with CE4 common base, the performance of test 1 to test 4 are summarized in Table 1 to Table 4, respectively. As compared with VTM-7.0, the performance of test 1 to test 4 are summarized in Table 5 to Table 8, respectively. The performance of test 5 (i.e., 32 modes with 14 block shapes on top of CE4-1 and CE4-2.1) as compared with VTM-7.0 is summarized in Table 5.

Table 1: Performance of test 1 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.10% | -0.02% | 96% | 99% |
| Class A2 | 0.04% | 0.04% | -0.06% | 98% | 99% |
| Class B | -0.02% | 0.03% | 0.03% | 99% | 101% |
| Class C | 0.02% | 0.20% | 0.13% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.03% | 98% | 100% |
| Class D | 0.02% | -0.05% | -0.03% | 100% | 100% |
| Class F | 0.02% | -0.03% | -0.05% | 99% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.01% | 0.11% | 101% | 100% |
| Class C | 0.03% | 0.02% | -0.08% | 98% | 98% |
| Class E | 0.23% | -0.25% | 0.12% | 99% | 99% |
| Overall | 0.08% | -0.05% | 0.05% | 100% | 99% |
| Class D | -0.08% | -0.27% | -0.09% | 98% | 97% |
| Class F | 0.00% | -0.02% | -0.16% | 99% | 100% |

Table 2: Performance of test 2 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.06% | 98% | 100% |
| Class A2 | 0.06% | 0.10% | -0.03% | 97% | 100% |
| Class B | 0.02% | 0.04% | 0.02% | 96% | 97% |
| Class C | 0.10% | 0.16% | 0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.06% | 0.04% | 98% | 99% |
| Class D | 0.07% | 0.00% | 0.24% | 99% | 99% |
| Class F | 0.06% | 0.05% | 0.05% | 97% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | 0.00% | 0.65% | 96% | 101% |
| Class C | 0.11% | 0.10% | 0.06% | 97% | 99% |
| Class E | 0.28% | -0.09% | 0.38% | 98% | 98% |
| Overall | 0.14% | 0.01% | 0.39% | 97% | 99% |
| Class D | 0.10% | -0.16% | 0.25% | 96% | 97% |
| Class F | 0.03% | -0.11% | 0.10% | 97% | 100% |

Table 3: Performance of test 3 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.12% | 0.18% | 97% | 99% |
| Class A2 | 0.13% | 0.12% | 0.12% | 98% | 101% |
| Class B | 0.10% | 0.15% | 0.18% | 97% | 99% |
| Class C | 0.31% | 0.54% | 0.56% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.24% | 0.27% | 98% | 99% |
| Class D | 0.19% | 0.23% | 0.37% | 99% | 101% |
| Class F | 0.14% | 0.18% | 0.15% | 100% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | 0.23% | 0.55% | 98% | 101% |
| Class C | 0.49% | 0.69% | 0.70% | 96% | 98% |
| Class E | 0.71% | 0.16% | 0.67% | 97% | 99% |
| Overall | 0.43% | 0.37% | 0.63% | 97% | 100% |
| Class D | 0.30% | 0.03% | 0.76% | 96% | 99% |
| Class F | 0.18% | 0.42% | 0.35% | 97% | 101% |

Table 4: Performance of test 4 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.01% | 0.14% | 98% | 100% |
| Class A2 | 0.17% | 0.20% | 0.09% | 99% | 100% |
| Class B | 0.10% | 0.20% | 0.18% | 98% | 99% |
| Class C | 0.37% | 0.58% | 0.55% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.26% | 0.25% | 99% | 99% |
| Class D | 0.27% | 0.42% | 0.40% | 97% | 97% |
| Class F | 0.13% | 0.21% | 0.15% | 99% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.28% | 0.30% | 0.30% | 96% | 101% |
| Class C | 0.53% | 0.78% | 0.63% | 96% | 98% |
| Class E | 0.70% | 0.00% | 0.67% | 97% | 98% |
| Overall | 0.47% | 0.38% | 0.50% | 96% | 100% |
| Class D | 0.40% | -0.08% | 0.43% | 96% | 99% |
| Class F | 0.14% | 0.22% | 0.21% | 99% | 101% |

Table 5: Performance of test 1 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.20% | -0.20% | 99% | 99% |
| Class A2 | -0.17% | -0.15% | -0.08% | 99% | 98% |
| Class B | -0.16% | -0.08% | -0.14% | 103% | 101% |
| Class C | -0.57% | -0.79% | -0.77% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.31% | -0.31% | 101% | 99% |
| Class D | -0.41% | -0.71% | -0.65% | 104% | 96% |
| Class F | -0.30% | -0.41% | -0.38% | 101% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.50% | -0.48% | 105% | 98% |
| Class C | -0.76% | -1.23% | -0.97% | 101% | 96% |
| Class E | -0.94% | -0.43% | -0.76% | 100% | 100% |
| Overall | -0.62% | -0.72% | -0.71% | 102% | 98% |
| Class D | -0.76% | -0.73% | -1.13% | 100% | 95% |
| Class F | -0.50% | -0.93% | -0.46% | 102% | 97% |

Table 6: Performance of test 2 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.16% | -0.12% | 101% | 100% |
| Class A2 | -0.16% | -0.09% | -0.05% | 99% | 99% |
| Class B | -0.12% | -0.06% | -0.15% | 100% | 98% |
| Class C | -0.49% | -0.83% | -0.79% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.22% | -0.29% | -0.29% | 101% | 98% |
| Class D | -0.36% | -0.67% | -0.38% | 103% | 95% |
| Class F | -0.26% | -0.33% | -0.28% | 99% | 95% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.50% | 0.06% | 100% | 99% |
| Class C | -0.68% | -1.15% | -0.83% | 100% | 96% |
| Class E | -0.89% | -0.28% | -0.51% | 99% | 98% |
| Overall | -0.56% | -0.66% | -0.38% | 100% | 98% |
| Class D | -0.58% | -0.60% | -0.79% | 99% | 95% |
| Class F | -0.48% | -1.01% | -0.20% | 100% | 97% |

Table 7: Performance of test 3 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.03% | 0.00% | 100% | 99% |
| Class A2 | -0.09% | -0.07% | 0.10% | 100% | 100% |
| Class B | -0.04% | 0.04% | 0.00% | 101% | 99% |
| Class C | -0.28% | -0.45% | -0.35% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.11% | -0.07% | 101% | 99% |
| Class D | -0.24% | -0.44% | -0.25% | 103% | 97% |
| Class F | -0.18% | -0.21% | -0.18% | 102% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.28% | -0.04% | 102% | 99% |
| Class C | -0.31% | -0.57% | -0.20% | 99% | 96% |
| Class E | -0.47% | -0.04% | -0.22% | 98% | 100% |
| Overall | -0.28% | -0.31% | -0.14% | 100% | 98% |
| Class D | -0.38% | -0.43% | -0.29% | 99% | 96% |
| Class F | -0.33% | -0.50% | 0.04% | 100% | 98% |

Table 8: Performance of test 4 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.10% | -0.04% | 100% | 99% |
| Class A2 | -0.05% | 0.01% | 0.07% | 100% | 99% |
| Class B | -0.04% | 0.10% | 0.01% | 101% | 100% |
| Class C | -0.22% | -0.41% | -0.35% | 103% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.10% | -0.09% | 101% | 99% |
| Class D | -0.16% | -0.25% | -0.22% | 102% | 93% |
| Class F | -0.19% | -0.17% | -0.18% | 102% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.21% | -0.30% | 99% | 99% |
| Class C | -0.27% | -0.48% | -0.26% | 99% | 96% |
| Class E | -0.48% | -0.20% | -0.22% | 98% | 98% |
| Overall | -0.24% | -0.30% | -0.26% | 99% | 98% |
| Class D | -0.29% | -0.56% | -0.61% | 99% | 97% |
| Class F | -0.37% | -0.69% | -0.09% | 102% | 97% |

Table 9: Performance of test 5 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.15% | -0.15% | 102% | 100% |
| Class A2 | -0.17% | -0.11% | -0.03% | 101% | 100% |
| Class B | -0.13% | -0.10% | -0.04% | 104% | 101% |
| Class C | -0.55% | -0.71% | -0.76% | 102% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.28% | -0.26% | 102% | 99% |
| Class D | -0.39% | -0.71% | -0.66% | 102% | 95% |
| Class F | -0.29% | -0.34% | -0.34% | 102% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.31% | -0.57% | -0.18% | 103% | 97% |
| Class C | -0.76% | -1.08% | -0.77% | 100% | 93% |
| Class E | -0.87% | 0.03% | -0.65% | 99% | 99% |
| Overall | -0.60% | -0.59% | -0.50% | 101% | 96% |
| Class D | -0.67% | -0.90% | -1.22% | 100% | 96% |
| Class F | -0.51% | -0.94% | -0.29% | 103% | 97% |

# Conclusions

In this contribution, the number of GEO modes is further reduced to 32 to lessen the hardware validation workload. Method #1 is to just use 32 modes. Method #2 is a one-level scalable solution to be able to either use 32 modes or only use the first 16 modes of 32 modes for light encoder usage. It is asserted that the proposed 32 modes GEO can reduce the hardware validation tests to 33% over the CE4 common base. As compared to VTM-7.0 with TPM mode, the coding performance of the proposed 32 modes combing with CE4.1 and CE4-2.1 is -0.26%/-0.62% in RA/LDB (i.e., 0.01%/0.08% in RA/LDB over CE4 common base). And the first 16 modes combing with CE4.1 and CE4-2.1 can still get decent gain over VTM-7.0 with TPM mode such as -0.12/-0.28% in RA/LDB. It is recommended to adopt the proposed method #2 to the next version of VVC.

# References

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, K. Reuzé, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, R.-L. Liao, J. Chen, Y. Ye, J. Luo, L. Xu, F. Chen, L. Wang, Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang, M Bläser, J. Sauer, “CE4-related: Combined Test of CE4-1, CE4-2 and CE4-3”, JVET-Q0061, January 2020, Brussels, BE.

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0884, October 2019, Geneva, Switzerland.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).