| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0309 |
| --- | --- |

| Title: | CE4-related: Further constraints on block shapes for GEO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng, Li Zhang, Hongbin Liu, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | +1(858)999-7093 zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to further constrain the block shapes applicable for GEO. On top of 16 block shapes in CE4-2.1, two methods are proposed to further reduce the number of allowed block shapes for GEO to 14 (method #1) or 12 (method #2). It is asserted that the number of allowed block shapes used for GEO can be reduced to 74% by method #1 with 14 block shapes and 63% by method #2 with 12 block shapes, respectively. Four tests are conducted to verify the proposed method on top of CE tests, i.e., CE4-1 + CE4-2.1 + method #1 (test 1), and CE4-1 + CE4-2.1 + CE4-3.1 + method #1 (test 2), CE4-1 + CE4-2.1 + method #2 (test 3), and CE4-1 + CE4-2.1 + CE4-3.1 + method #2 (test 4), respectively.

The results are reported as follows, as compared with CE4 common base and VTM-7.0, respectively.

| Test 1 vs. CE4 common base: | RA: -0.02%/0.02%/-0.02% with 100% EncT and 100% DecT |
| --- | --- |
|  | LDB: 0.05%/0.09%/0.07% with 100% EncT and 100% DecT |
| Test 2 vs. CE4 common base: | RA: 0.03%/0.00%/-0.01% with 100% EncT and 100% DecT |
|  | LDB: 0.14%/-0.02%/0.20% with 99% EncT and 99% DecT |
| Test 3 vs. CE4 common base: | RA: -0.01%/0.01%/-0.05% with 100% EncT and 100% DecT |
|  | LDB: 0.11%/0.16%/0.27% with 99% EncT and 99% DecT |
| Test 4 vs. CE4 common base: | RA: 0.04%/-0.03%/0.01% with 100% EncT and 100% DecT |
|  | LDB: 0.12%/0.12%/0.19% with 100% EncT and 100% DecT |
| Test 1 vs. VTM-7.0: | RA: -0.29%/-0.34%/-0.35% with 103% EncT and 99% DecT |
|  | LDB: -0.66%/-0.59%/-0.70% with 103% EncT and 98% DecT |
| Test 2 vs. VTM-7.0: | RA: -0.25%/-0.35%/-0.35% with 103% EncT and 99% DecT |
|  | LDB: -0.56%/-0.70%/-0.56% with 102% EncT and 98% DecT |
| Test 3 vs. VTM-7.0: | RA: -0.28%/-0.35%/-0.39% with 102% EncT and 99% DecT |
|  | LDB:-0.60%/-0.52%/-0.49% with 102% EncT and 98% DecT |
| Test 4 vs. VTM-7.0: | RA: -0.23%/-0.39%/-0.33% with 103% EncT and 99% DecT |
|  | LDB: -0.59%/-0.55%/-0.58% with 103% EncT and 99% DecT |

# Introduction

In the core experiment 4, several tests were defined to investigate GEO. A common base of GEO was released on top of VTM-7.0. In the CE4 common base, 82 different GEO modes are implemented as a replacement of the TPM mode in VVC WD7. GEO is allowable for 19 different block shapes ranging from block size 8×8 to block size 128×128. In CE4-2.1, GEO is disabled for block shapes greater than 64×64, and the number of block shapes allowed for GEO is reduced from 19 to 16.

# Proposed methods

In this contribution, two methods are proposed to further constrain the block shapes applicable for GEO to 14 or 12, on top of CE4-2.1.

Method #1: Based on CE4-2.1, further disable GEO for 1:8 or 8:1 blocks, e.g., 64x8 and 8x64 block shapes.

Method #2: Based on method #1, further disable GEO for 4:1 blocks, e.g., 64x16 and 32x8 block shapes.

In stead of allowing GEO for 19 block shapes in the common base, the number of block shapes allowed for GEO is reduced to 74% by method #1 with 14 block shapes and 63% by method #2 with 12 block shapes, respectively.

# Simulation results

To verify the proposed method, the following tests are conducted:

- Two tests are conducted to verify method #1 on top of CE tests, i.e., CE4-1 + CE4-2.1 + method #1 (test 1), CE4-1 + CE4-2.1 + CE4-3.1 + method #1 (test 2), respectively.

- Two tests are conducted to verify method #2 on top of CE tests, i.e., CE4-1 + CE4-2.1 + method #2 (test 3), CE4-1 + CE4-2.1 + CE4-3.1 + method #2 (test 4), respectively.

The tests are implemented on top of CE4 common base [1] under JVET CTC conditions [2]. As compared with the CE4 common base, the performance of test 1 to test 4 are summarized in Table 1 to Table 4, respectively. As compared with VTM-7.0, the performance of test 1 to test 4 are summarized in Table 5 to Table 8, respectively.

Table 1: Performance of test 1 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.00% | 100% | 100% |
| Class A2 | 0.03% | 0.10% | -0.03% | 100% | 100% |
| Class B | -0.04% | 0.01% | 0.01% | 100% | 99% |
| Class C | -0.04% | 0.09% | -0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.02% | -0.02% | 100% | 100% |
| Class D | -0.01% | 0.08% | -0.12% | 100% | 100% |
| Class F | 0.02% | 0.04% | -0.01% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.11% | 0.00% | 100% | 100% |
| Class C | -0.03% | 0.10% | -0.15% | 100% | 100% |
| Class E | 0.21% | 0.03% | 0.45% | 99% | 101% |
| Overall | 0.05% | 0.09% | 0.07% | 100% | 100% |
| Class D | 0.01% | -0.35% | -0.45% | 101% | 100% |
| Class F | -0.03% | -0.06% | -0.52% | 100% | 100% |

Table 2: Performance of test 2 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.18% | -0.02% | 100% | 100% |
| Class A2 | 0.04% | 0.05% | -0.03% | 100% | 100% |
| Class B | 0.01% | -0.01% | 0.03% | 100% | 100% |
| Class C | 0.04% | 0.12% | -0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | -0.01% | 100% | 100% |
| Class D | 0.05% | -0.09% | -0.01% | 99% | 99% |
| Class F | 0.04% | 0.06% | 0.01% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.05% | 0.34% | 99% | 100% |
| Class C | 0.08% | 0.07% | 0.05% | 100% | 98% |
| Class E | 0.34% | -0.27% | 0.19% | 99% | 99% |
| Overall | 0.14% | -0.02% | 0.20% | 99% | 99% |
| Class D | -0.02% | -0.43% | 0.43% | 100% | 100% |
| Class F | 0.13% | 0.22% | -0.05% | 100% | 101% |

Table 3: Performance of test 3 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.02% | 99% | 101% |
| Class A2 | 0.03% | 0.07% | -0.06% | 100% | 101% |
| Class B | -0.02% | -0.04% | -0.02% | 98% | 98% |
| Class C | -0.01% | 0.03% | -0.09% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.05% | 100% | 100% |
| Class D | 0.02% | 0.01% | 0.04% | 101% | 100% |
| Class F | 0.04% | 0.03% | -0.01% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.07% | 0.42% | 100% | 100% |
| Class C | 0.03% | 0.13% | 0.24% | 99% | 99% |
| Class E | 0.29% | 0.34% | 0.07% | 98% | 98% |
| Overall | 0.11% | 0.16% | 0.27% | 99% | 99% |
| Class D | -0.05% | -0.54% | -0.76% | 100% | 99% |
| Class F | 0.02% | 0.26% | 0.08% | 100% | 100% |

Table 4: Performance of test 4 over VTM-7.0_CE4_common_base

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.17% | 0.01% | 100% | 100% |
| Class A2 | 0.07% | 0.04% | 0.00% | 100% | 100% |
| Class B | 0.03% | -0.03% | -0.01% | 100% | 100% |
| Class C | 0.05% | 0.02% | 0.04% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.03% | 0.01% | 100% | 100% |
| Class D | 0.11% | 0.02% | -0.07% | 100% | 99% |
| Class F | 0.07% | 0.02% | 0.02% | 102% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0_CE4_common_base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | 0.28% | 0.27% | 100% | 101% |
| Class C | 0.08% | 0.08% | 0.04% | 101% | 100% |
| Class E | 0.22% | -0.06% | 0.24% | 100% | 100% |
| Overall | 0.12% | 0.12% | 0.19% | 100% | 100% |
| Class D | 0.11% | -0.08% | 0.22% | 102% | 104% |
| Class F | 0.08% | -0.17% | 0.57% | 100% | 103% |

Table 5: Performance of test 1 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.23% | -0.18% | 103% | 99% |
| Class A2 | -0.18% | -0.09% | -0.05% | 102% | 100% |
| Class B | -0.18% | -0.09% | -0.16% | 103% | 99% |
| Class C | -0.63% | -0.90% | -0.95% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.34% | -0.35% | 103% | 99% |
| Class D | -0.44% | -0.59% | -0.74% | 104% | 96% |
| Class F | -0.29% | -0.34% | -0.34% | 103% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.40% | -0.59% | 104% | 97% |
| Class C | -0.83% | -1.15% | -1.03% | 103% | 97% |
| Class E | -0.96% | -0.16% | -0.43% | 100% | 101% |
| Overall | -0.66% | -0.59% | -0.70% | 103% | 98% |
| Class D | -0.67% | -0.80% | -1.49% | 103% | 98% |
| Class F | -0.53% | -0.97% | -0.82% | 103% | 97% |

Table 6: Performance of test 2 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.27% | -0.20% | 102% | 100% |
| Class A2 | -0.17% | -0.14% | -0.05% | 102% | 99% |
| Class B | -0.13% | -0.11% | -0.15% | 104% | 100% |
| Class C | -0.55% | -0.87% | -0.92% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.35% | -0.35% | 103% | 99% |
| Class D | -0.38% | -0.76% | -0.63% | 104% | 95% |
| Class F | -0.27% | -0.32% | -0.32% | 103% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.45% | -0.26% | 103% | 98% |
| Class C | -0.72% | -1.18% | -0.84% | 103% | 96% |
| Class E | -0.83% | -0.46% | -0.69% | 99% | 100% |
| Overall | -0.56% | -0.70% | -0.56% | 102% | 98% |
| Class D | -0.70% | -0.88% | -0.62% | 103% | 98% |
| Class F | -0.38% | -0.69% | -0.36% | 103% | 98% |

Table 7: Performance of test 3 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.10% | -0.20% | 102% | 100% |
| Class A2 | -0.19% | -0.12% | -0.08% | 102% | 100% |
| Class B | -0.17% | -0.15% | -0.20% | 102% | 98% |
| Class C | -0.60% | -0.96% | -0.99% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.35% | -0.39% | 102% | 99% |
| Class D | -0.41% | -0.65% | -0.58% | 106% | 96% |
| Class F | -0.27% | -0.36% | -0.34% | 103% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.44% | -0.18% | 103% | 98% |
| Class C | -0.77% | -1.12% | -0.65% | 103% | 96% |
| Class E | -0.88% | 0.15% | -0.81% | 99% | 99% |
| Overall | -0.60% | -0.52% | -0.49% | 102% | 98% |
| Class D | -0.73% | -1.00% | -1.79% | 103% | 97% |
| Class F | -0.49% | -0.66% | -0.22% | 103% | 96% |

Table 8: Performance of test 4 over VTM-7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.26% | -0.17% | 103% | 100% |
| Class A2 | -0.14% | -0.15% | -0.02% | 101% | 99% |
| Class B | -0.11% | -0.13% | -0.18% | 104% | 100% |
| Class C | -0.54% | -0.97% | -0.86% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.23% | -0.39% | -0.33% | 103% | 99% |
| Class D | -0.32% | -0.64% | -0.68% | 104% | 95% |
| Class F | -0.25% | -0.36% | -0.31% | 104% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.23% | -0.32% | 104% | 99% |
| Class C | -0.71% | -1.17% | -0.85% | 104% | 97% |
| Class E | -0.95% | -0.25% | -0.65% | 101% | 100% |
| Overall | -0.59% | -0.55% | -0.58% | 103% | 99% |
| Class D | -0.57% | -0.54% | -0.82% | 104% | 102% |
| Class F | -0.43% | -1.09% | 0.27% | 103% | 100% |

# Corresponding spec changes

The corresponding specification changes on top of CE4_1_CommonBaseWD_w_fixes [3] and CE4-2.1 [4] are provided as follows, in which grey and grey with stickthrough are modifications by CE4-2.1, and yellow are modifications by the proposed method in this contribution.

## Proposed specification changes for method #1 on top of CE4-2.1

#### 7.3.9.7 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(cbWidth <= 64 && cbHeight <= 64 && (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) ¦ ¦ ( sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && cbWidth>=8 && cbHeight >=8 && cbWidth/cbHeight <=4 && cbHeight/cbWidth <=4 && slice_type = = B ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( regular_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth >= 8 && cbHeight >= 8 && cbWidth < 128 && cbHeight < 128 && cbWidth/cbHeight <=4 && cbHeight/cbWidth <=4 ) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumGeoMergeCand > 1 ) { |  |
| merge_geo_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Proposed specification changes for method #2 on top of CE4-2.1

#### 7.3.9.7 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if(cbWidth <= 64 && cbHeight <= 64 && (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) ¦ ¦ ( sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && cbWidth>=8 && cbHeight >=8 && cbWidth/cbHeight <=2 && cbHeight/cbWidth <=4 && slice_type = = B ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( regular_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth >= 8 && cbHeight >= 8 && cbWidth < 128 && cbHeight < 128 && cbWidth/cbHeight <=2 && cbHeight/cbWidth <=4 ) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumGeoMergeCand > 1 ) { |  |
| merge_geo_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Conclusions

In this contribution, two methods are propoded to further reduce the number of block shapes applicable for GEO to 14 (method #1) or 12 (method #2). It is asserted that the the number of allowed block shapes for GEO is reduced to 74% or 63% accordingly. It is recommended to adopt method #2 with 12 block shapes for GEO into the next version of VVC.

# References

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0884, October 2019, Geneva, Switzerland.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, CH.

- M. Zhou, “CE4-related: CE4-1 spec text with suggested fixes”,  JVET-Q0160, January 2020, Brussels, BE.

- Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang, “CE4-2: Block size restriction of GEO” ,  JVET-Q0077, January 2020, Brussels, BE.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).