| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0310 |
| --- | --- |

| Title: | Non-CE5: Suggested text for CC-ALF padding process with raster scan slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Hongbin Liu, Li Zhang, Kai Zhang, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | wangyang.cs@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents suggested text changes for CC-ALF padding process with raster-scan slices. More specifically, it is proposed to align the luma sample padding process in CC-ALF to ALF for raster-scan slices. It is reported that no performance changes are observed in the common test conditions.

Introduction

The CC-ALF design in [1] was adopted as the CE5 common base in the 16th JVET meeting. With CC-ALF, a reconstructed chroma sample is further refined by the result of an adaptive linear filtering process applied to corresponding luma samples.

Meanwhile, padding method of unavailable samples outside of a raster-scan slice in the ALF process was adopted in the last JVET meeting [2]. In [2] , for a current CTU, if its above CTU and left CTU are available while its above-left CTU is unavailable, samples in the above CTU are used to pad the unavailable samples in the above-left CTU; if its below CTU and right CTU are available while its below-right CTU is unavailable, samples in the below CTU are used to pad the unavailable samples in the below-right CTU.

However, in both of the current CE5 common base software [3] and the corresponding specification changes [1], unavailable samples outside of a raster-scan slice in the CC-ALF process is not handled.

Proposed method

It is proposed to align the luma sample padding process in CC-ALF to ALF for raster-scan slices. With the proposed method, no additional memoery access patterns are required.

Proposed changes to working draft

The suggested specification changes on top of JVET-Q0058_CE5-Anchor_JVET-P2001-vE [1] is described as follows. Deleted text is marked with red strikethrough, and newly added text is highlighted in green.

- x.x.x.x Cross component filtering process for block of chroma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the luma adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPictureC,

- a chroma location ( xCtbC, yCtbC ) specifying the top-left sample of the current chroma coding tree block relative to the top left sample of the current picture,

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture

- a width ccAlfWidth of block of chroma samples

- a height ccAlfHeight of block of chroma samples

- cross component filter coefficients CcAlfCoeff[j], with j = 0..7

Output of this process is the modified filtered reconstructed chroma picture sample array ccAlfPicture.

For the derivation of the filtered reconstructed chroma samples ccAlfPicture[ xCtbC + x ][ yCtbC + y ], each reconstructed chroma sample inside the current chroma block of samples alfPictureC[ xCtbC + x ][ yCtbC + y ] with x = 0..ccAlfWidth − 1, y = 0..ccAlfHeight – 1, is filtered as follows:

  - The luma location (xL, yL) corresponding to the current chroma sample at chroma location ( xCtbC + x, yCtbC + y ) is set equal to ( (xCtbC + x ) * SubWidthC, ( yCtbC + y ) * SubHeightC )

  - The luma locations ( hxL + i, vyL + j ) with i = −1..1, j = −1..2 inside the array recPictureL are derived as follows:

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and xL − PpsVirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hxL + i = Clip3( PpsVirtualBoundariesPosX[ n ], pic_width_in_luma_samples − 1, xL + i )	(8-1229)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosX[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosX[ n ]− xL is greater than 0 and less than 4 for any n = 0..pps_num_ver_virtual_boundaries − 1, the following applies:

hx + i = Clip3( 0, PpsVirtualBoundariesPosX[ n ] − 1, xL + i )	(8-1230)

  - Otherwise, the following applies:

hx + i = Clip3( 0, pic_width_in_luma_samples − 1, xL + i )	(8-1231)

  - If pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and yL − PpsVirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( PpsVirtualBoundariesPosY[ n ], pic_height_in_luma_samples − 1, yL + j )	(8-1232)

  - Otherwise, if pps_loop_filter_across_virtual_boundaries_disabled_flag is equal to 1, and PpsVirtualBoundariesPosY[ n ] % CtbSizeY is not equal to 0, and PpsVirtualBoundariesPosY[ n ] − yL is greater than 0 and less than 4 for any n = 0..pps_num_hor_virtual_boundaries − 1, the following applies:

vy + j = Clip3( 0, PpsVirtualBoundariesPosY[ n ] − 1, yL + j )	(8-1233)

  - Otherwise, the following applies:

vy + j = Clip3( 0, pic_height_in_luma_samples − 1, yL + j )	(8-1234)

  - The variables clipLeftPos, clipRightPos, clipTopPos, and clipBottomPos, clipTopLeftFlag and clipBotRightFlag are derived by invoking the ALF boundary position derivation process as specified in clause 8.8.5.5 with ( xCtb, yCtb ) and ( xL – xCtb, yL – yCtb ) as inputs.

  - The variables hx + i and vy + j are modified by invoking the ALF sample padding process as specified in clause 8.8.5.6 with ( xCtb, yCtb ), ( hx + i, vy + j ), 0, clipLeftPos, clipRightPos, clipTopPos, clipBottomPos, clipTopLeftFlag and clipBotRightFlag as inputs.

The vertical sample position offsets yM1, yP1 and yP2 are specified in Table x-xx according to the vertical luma sample position yL, clipLeftPos and clipRightPos.

Reference

- K. Misra, F. Bossen, A. Segall, P. Cowan, N. Hu, et al., “CE5 Common Base: Cross Component Adaptive Loop Filter,” JVET-Q0058, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 17th Meeting: Brussels, BE, 7–17 January 2020.

- H. Liu, L. Zhang, K. Zhang, J. Xu, Y. Wang, N. Hu, et al., “Non-CE5: Suggested text changes for ALF padding process,” JVET-P1038, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019.

- https://vcgit.hhi.fraunhofer.de/jvet-p-ce5/VVCSoftware_VTM/tree/CE5-Anchor

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).