| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0311-v2 |
| --- | --- |

| Title: | Non-CE5: On CC-ALF padding for ALF virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Hongbin Liu, Li Zhang, Kai Zhang, Zhipin Deng, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | wangyang.cs@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In this contribution, two methods are proposed on CC-ALF padding for virtual boundaries. In method #1: filtering chroma samples between ALF virtual boundary and CTU bottom boundary are disabled. In method #2, a different symmetric padding method is proposed to replace existing CC-ALF padding method for ALF virtual boundaries. Compared with CE5 common base, {Y, U, V} BD-rate changes of these two methods are reportedly shown as follows:

Method #1: {0.00%, 0.22%, 0.19%}, {0.00%, 0.16%, 0.22%}, and {0.00%, 0.15%, 0.24%} for AI, RA, and LDB, respectively.

Method #2: {0.00%, 0.00%, 0.01%}, {0.00%, -0.01%, 0.00%}, and {0.01%, -0.07%, 0.11%} for AI, RA, and LDB, respectively.

Introduction

The CC-ALF design in [1] was adopted as the CE5 common base in the 16th JVET meeting. With CC-ALF, a reconstructed chroma sample is further refined by the result of an adaptive linear filtering process applied to the corresponding luma samples.

In the CE5 common base, boundary padding is performed in CC-ALF for the horizontal virtual boundaries, as shown in Figure 1. In Figure 1 (a) and (d), C7 needs to be padded, and it is set to C5. In Figure 1 (b) and (c), five samples need to be padded, denoted by C0, C4, C5, C6 and C7. More specifically, C0 is set to C2. C7 is set to C5 before C5 being padded. C4, C5, and C6 are set to C1, C2, and C3, respectively.

With the current design, the center line of the first 3 lines of the filter shape is regarded as the axis of symmetry, causing two issues as follows:

- Some available samples are padded unnecessarily. For example, C4, C5, and C6 in Figure 1 (c), and C7 in Figure 1 (d) are padded even though they are available.

- In the specification of CE5 common base, some unavailable samples are padded from samples that are also unavailable, which is incorrect. As shown in Figure 1 (b) and (c), C7 is padded from C5 which is unavailable.

(a)                 (b)                  (c)                  (d)

Figure 1. CC-ALF padding for virtual boundary in CE5 common base.

Proposed methods

Two methods are proposed to address the issues mentioned above.

Method #1

It is proposed to disable CC-ALF for chroma samples between the virtual boundary and CTU bottom boundary.

It is asserted that Method #1 could reduce the complexity overhead and simplify the operation logic of CC-ALF padding for ALF virtual boundaries, especially when the CTU bottom boundary is also a picture/subpicture/slice/tile boundary or 360-degree boundary.

Method #2

A different symmetric padding method is proposed to replace the existing CC-ALF padding method for virtual boundary.

Instead of using the center line of the first 3 lines of the filter shape as the axis of symmetry, the center line of the filter shape, e.g. the dotted yellow line between the 2nd line and 3rd line shown in Figure 2, is proposed to be regarded as the axis of symmetry. With the new axis of symmetry, the proposed symmetric padding method is shown in Figure 2. It is asserted that the sample lines to be padded are reduced in the proposed method, as shown in Figure 2 (c) and (d).

(a) and (c): C0 is set to C2, and C7 is set to C5. (b): C0 is set to C2. C4, C5, and C6 are set to C1, C2, and C3, respectively. C7 is set to C2.

(a)                 (b)                  (c)                  (d)

Figure 2 Proposed symmetric padding for virtual boundary in CC-ALF.

Simulation results

The proposed methods have been implemented on top of the CE5 common base [2]. Simulations are performed following the common test conditions [3]. The results of Method #1 and Method #2 are shown in Table 1 and Table 2 respectively, with the CE5 common base as the anchor.

Table 1: Coding performance of Method #1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.14% | 0.16% | 99% | 100% |
| Class A2 | 0.00% | 0.23% | 0.38% | 99% | 99% |
| Class B | 0.00% | 0.40% | 0.30% | 98% | 96% |
| Class C | 0.00% | 0.08% | 0.08% | 98% | 95% |
| Class E | 0.00% | 0.16% | 0.04% | 99% | 95% |
| Overall | 0.00% | 0.22% | 0.19% | 98% | 97% |
| Class D | 0.00% | 0.07% | 0.02% | 99% | 96% |
| Class F (optional) | 0.01% | 0.19% | 0.14% | 99% | 96% |
|  | Random Access Main10 |  |  |  |  |
|  | Over CE5 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.32% | 100% | 99% |
| Class A2 | 0.02% | 0.23% | 0.28% | 95% | 96% |
| Class B | 0.01% | 0.36% | 0.24% | 98% | 97% |
| Class C | -0.02% | 0.03% | 0.06% | 96% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.16% | 0.22% | 97% | 96% |
| Class D | -0.02% | 0.15% | 0.28% | 99% | 97% |
| Class F (optional) | 0.01% | 0.16% | 0.10% | 96% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.30% | 97% | 96% |
| Class C | 0.00% | 0.04% | 0.05% | 98% | 99% |
| Class E | 0.01% | 0.28% | 0.39% | 99% | 97% |
| Overall | 0.00% | 0.15% | 0.24% | 98% | 97% |
| Class D | -0.02% | -0.33% | 0.36% | 99% | 99% |
| Class F (optional) | 0.00% | 0.16% | 0.47% | 98% | 98% |

Table 2: Coding performance of Method #2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 101% | 101% |
| Class A2 | 0.00% | -0.01% | 0.01% | 106% | 103% |
| Class B | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.02% | -0.01% | 101% | 106% |
| Overall | 0.00% | 0.00% | 0.01% | 102% | 102% |
| Class D | 0.00% | 0.01% | -0.02% | 99% | 99% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  | Random Access Main10 |  |  |  |  |
|  | Over CE5 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.17% | 0.03% | 99% | 99% |
| Class A2 | -0.01% | 0.08% | 0.05% | 97% | 96% |
| Class B | 0.00% | 0.09% | -0.16% | 98% | 97% |
| Class C | -0.01% | -0.09% | 0.12% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% | 98% | 97% |
| Class D | 0.00% | 0.04% | 0.05% | 100% | 100% |
| Class F (optional) | -0.01% | 0.05% | -0.02% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE5 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.17% | 0.12% | 99% | 99% |
| Class C | -0.01% | 0.06% | -0.04% | 101% | 101% |
| Class E | 0.08% | -0.07% | 0.28% | 102% | 103% |
| Overall | 0.01% | -0.07% | 0.11% | 101% | 101% |
| Class D | -0.03% | -0.26% | -0.50% | 101% | 101% |
| Class F (optional) | 0.00% | 0.54% | 0.39% | 104% | 104% |

Conclusions

In this contribution, two methods are proposed on CC-ALF padding for virtual boundary. It is recommended adopting the proposed methods into the next version of VVC.

Proposed changes to working draft

The suggested specification changes on top of JVET-Q0058_CE5-Anchor_JVET-P2001-vE [1] is described as follows: deleted text is marked with red strikethrough, and newly added text are highlighted in green and grey for the first method and second method, respectively.

Method #1

- x.x.x.x Cross component filtering process for block of chroma samples

Inputs of this process are:

- a reconstructed luma picture sample array recPictureL prior to the luma adaptive loop filtering process,

- a filtered reconstructed chroma picture sample array alfPictureC,

- a chroma location ( xCtbC, yCtbC ) specifying the top-left sample of the current chroma coding tree block relative to the top left sample of the current picture,

- a luma location ( xCtb, yCtb ) specifying the top-left sample of the current luma coding tree block relative to the top left sample of the current picture

- a width ccAlfWidth of block of chroma samples

- a height ccAlfHeight of block of chroma samples

- cross component filter coefficients CcAlfCoeff[j], with j = 0..7

Output of this process is the modified filtered reconstructed chroma picture sample array ccAlfPicture.

For the derivation of the filtered reconstructed chroma samples ccAlfPicture[ xCtbC + x ][ yCtbC + y ], each reconstructed chroma sample inside the current chroma block of samples alfPictureC[ xCtbC + x ][ yCtbC + y ] with x = 0..ccAlfWidth − 1, y = 0..ccAlfHeight – 1, is filtered as follows:

  - If y * SubHeightC – ( CtbSizeY − 4 ) is greater than or equal to 0,

  - ccAlfPicture[ xCtbC + x ][ yCtbC + y ] = alfPictureC[ xCtbC + x, yCtbC + y ]

  - Otherwise, the following applies:

  - The luma location (xL, yL) corresponding to the current chroma sample at chroma location ( xCtbC + x, yCtbC + y ) is set equal to ( (xCtbC + x ) * SubWidthC, ( yCtbC + y ) * SubHeightC )

  - The luma locations ( hxL + i, vyL + j ) with i = −1..1, j = −1..2 inside the array recPictureL are derived as follows:

…

Method #2

Table x-xx – Specification of yM1, yP1 and yP2 according to the vertical luma sample position yL, clipTopPos and clipBottomPos

| Condition | yM1 | yP1 | yP2 |
| --- | --- | --- | --- |
| yL = = clipTopPos + 1 | −1 | 1 | 1 |
| yL = = clipTopPos | 0 | 0 | 1 |
| yL = = clipBottomPos − 1 | 0 | 0 | 1 |
| yL = = clipBottomPos − 2 | −1 | 1 | 1 |
| Otherwise | −1 | 1 | 2 |

| Condition | yM1 | yP1 | yP2 |
| --- | --- | --- | --- |
| ( y = = CtbSizeY − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 0 | 1 | 1 |
| ( y = = CtbSizeY − 5 ) && ( applyAlfLineBufBoundary = = 1 ) | 0 | 0 | 0 |
| ( y = = CtbSizeY − 6 ) && ( applyAlfLineBufBoundary = = 1 ) | 0 | 1 | 1 |
| Otherwise | −1 | 1 | 2 |

Reference

- K. Misra, F. Bossen, A. Segall, P. Cowan, N. Hu, et al., “CE5 Common Base: Cross Component Adaptive Loop Filter,” JVET-Q0058, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 17th Meeting: Brussels, BE, 7–17 January 2020.

- https://vcgit.hhi.fraunhofer.de/jvet-p-ce5/VVCSoftware_VTM/tree/CE5-Anchor.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).