| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0312 |
| --- | --- |

| Title: | CE4-related: On simplification for GEO weight derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yucheng Sun Fangdong Chen Li Wang | Tel: Email: | +86 18105755698 sunyucheng@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This contribution presents simplification based on GEO mode combined test, including CE4-1, CE4-2 and CE4-3. In this contribution, when calculation pixel-wise weight for each pixel position of GEO block, the look-up table and abs() process is no longer needed. The test results reportedly shows that only slight loss was found.

Compared to VTM7.0

•	RA: -0.27% (Y), -0.32% (U), -0.37% (V)

•	LD: -0.xx% (Y), -0.xx% (U), -0.xx% (V)

Proposed simplification

Current GEO mode weight calculation for each pixel:

g_geoFilterLookup[ 0 ][ Clip3(0, 26, abs(((x + A) >> B) + ((y + C) >> D) - E))]

or

g_geoFilterLookup[ 1 ][ Clip3(0, 26, abs(((x + A) >> B) + ((y + C) >> D) - E))]

The proposed simplification:

Clip3(0, 8, (y << A) + ((x << B) >> C) – D)

or

Clip3(0, 8, (y << A) - ((x << B) >> C) – D)

or

Clip3(0, 8, (x << A) + ((y << B) >> C) – D)

or

Clip3(0, 8, (x << A) - ((y << B) >> C) – D)

Performance results

The performance result is summarised in Table 1 and Table 2. The tests were performed using the JVET common test conditions with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 7.0.

Table 1 - Performance of RA.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.13% | -0.15% | -0.21% | 101% | 100% |
| Class A2 | -0.19% | -0.10% | -0.07% | 101% | 100% |
| Class B | -0.13% | -0.07% | -0.19% | 101% | 100% |
| Class C | -0.60% | -0.92% | -0.95% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.32% | -0.37% | 101% | 100% |
| Class D | -0.42% | -0.70% | -0.51% | 102% | 101% |
| Class F | -0.23% | -0.36% | -0.30% | 100% | 100% |

Table 2 - Performance of LD.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.67% | -1.18% | -0.78% | 102% | 99% |
| Class E | -0.69% | 0.18% | -0.50% | 99% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.71% | -0.22% | -1.26% | 102% | 99% |
| Class F |  |  |  |  |  |

Conclusion

In this contribution, weight derivation simplification on GEO is tested. Compare to VTM 7.0, only slight loss was found. It is recommended to adopt this modification.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).