| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0314-v1 |
| --- | --- |

| Title: | Non-CE: Retrained LFNST Matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kui Fan, Li Zhang, Kai Zhang, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-265-9942 kui.fan@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In this contribution, a different set of LFNST matrices are proposed to replace existing matrices. It is asserted that the complexity is kept unchanged while coding performance improvements could be achieved. Experimental results reportedly show that the proposed matrices provide -0.14% and -0.09% BD-rate under AI and RA configurations, respectively.

# Introduction

In the current design, eight 16×16 matrices and eight 16×48 matrices are defined for LFNST. These 16×16 matrices are used for the left-top 4×4 coefficients in 4×N or N×4 (N>=4) blocks. These 16×48 matrices are applied to the left-top 8×8 coefficients in 8×N or N×8 (N>=8) blocks.

# Proposed methods

In this contribution, a new set of LFNST transform matrices (kernels) are proposed to replace existing 16×16 and 16×48 matrices in the current LFNST design. The proposed matrices are trained from non-CTC sequences and QPs are from 22 to 27. The original LFNST design is kept unchanged except for the matrices.

# Experimental results

The experiments were carried out under “All Intra” and “Random Access” configurations as defined in the JVET Common Test Condition (CTC) [1] on top of VTM-7.0.

Table 1 shows the CTC results of the proposed method. Table 2 shows the results of high QPs (32, 37, 42, 47). More detailed information could be found in the attached spreadsheet.

Table 1. CTC test results of the proposed method on top of VTM-7.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.04% | 0.28% | 97% | 99% |
| Class A2 | -0.09% | 0.19% | 0.10% | 100% | 101% |
| Class B | -0.08% | 0.04% | 0.08% | 100% | 101% |
| Class C | -0.26% | -0.06% | -0.04% | 106% | 106% |
| Class E | -0.14% | -0.03% | 0.20% | 103% | 103% |
| Overall | -0.14% | 0.02% | 0.11% | 101% | 102% |
| Class D | -0.17% | 0.07% | -0.07% | 101% | 101% |
| Class F | -0.21% | -0.01% | -0.13% | 101% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.09% | -0.01% | 99% | 98% |
| Class A2 | -0.08% | 0.19% | 0.19% | 98% | 98% |
| Class B | -0.05% | 0.19% | 0.11% | 101% | 101% |
| Class C | -0.17% | 0.07% | 0.08% | 103% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.14% | 0.09% | 101% | 101% |
| Class D | -0.12% | -0.15% | 0.17% | 102% | 101% |
| Class F | -0.15% | 0.06% | -0.14% | 101% | 102% |

Table 2. High QP test results of the proposed method on top of VTM-7.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | -0.04% | 0.07% | 97% | 99% |
| Class A2 | -0.14% | 0.11% | 0.19% | 98% | 99% |
| Class B | -0.14% | 0.02% | 0.12% | 97% | 99% |
| Class C | -0.19% | 0.24% | 0.12% | 102% | 101% |
| Class E | -0.11% | -0.07% | 0.06% | 99% | 99% |
| Overall | -0.16% | 0.06% | 0.11% | 99% | 99% |
| Class D | -0.12% | 0.08% | 0.16% | 99% | 96% |
| Class F | -0.15% | 0.28% | -0.25% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | -0.12% | 97% | 97% |
| Class A2 | -0.09% | -0.10% | 0.09% | 99% | 99% |
| Class B | -0.10% | 0.27% | 0.12% | 100% | 100% |
| Class C | -0.12% | -0.07% | 0.20% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | 0.06% | 0.09% | 100% | 99% |
| Class D | -0.14% | 0.10% | -0.20% | 101% | 101% |
| Class F | -0.10% | -0.23% | -0.09% | 100% | 101% |

# Conclusion

In this contribution, a new set of LFNST matrices are proposed. The proposed method brings -0.14% and -0.09% BD-rates in the AI and RA configurations. The BD-rates for high QPs (32, 37, 42, 47) are -0.16% and -0.08% under AI and RA configurations. The consistent performance improvement verifies the effectiveness and the generalization capacity of the proposed LFNST matrices. It is recommended to adopt this method in the next VVC WD and VTM software.

# Reference

[1]	F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).