| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0317-v4 |
| --- | --- |

| Title: | AHG9: A cleanup on deblocking filtering between subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Ye-Kui Wang, Hongbin Liu, Jizheng Xu, Zhipin Deng (Bytedance) Hyeongmun Jang, Junghak Nam (LGE) | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com hm.jang@lge.com |
| Source: | Bytedance Inc., LG Electronics Inc. |  |  |

_____________________________

# Abstract

In the current VVC, whether to apply deblocking filter on a sample in one subpicture may depend on the loop_filter_across_subpic_enabled_flag[ ] associated with another subpicture, while whether to apply SAO and ALF filtering on a sample in one subpicture does not depend on the loop_filter_across_subpic_enabled_flag[ ] associated with another subpicture. It is asserted that the design intent was to have the same behavior herein for deblocking as SAO and ALF. In this contribution, modifications to the deblocking process are proposed. In solution #1, the loop_filter_across_subpic_enabled_flag[ ] of a sub-picture can only control whether to apply deblocking filtering on samples inside the subpicture. In solution #2, the spec is modified such that the deblocking filter is not applied on a boundary between two subpictures if loop_filter_across_subpic_enabled_flag[ ] of either of the two subpictures is equal to 0. It is asserted that no performance changes are observed under the common test conditions.

# Introduction

In VVC, loop_filter_across_subpic_enabled_flag[ SubPicIdx ] indicates whether loop-filtering, including SAO, ALF and deblocking filtering, is allowed across a boundary between subpictures. For SAO and ALF, loop_filter_across_subpic_enabled_flag[ SubPicIdx ] only controls whether to filter a sample inside the subpicture with index equal to SubPicIdx. However, the deblocking filtering process for a sample in one subpicture may be controlled by loop_filter_across_subpic_enabled_flag associated with another subpicture.

In JVET-P2001-v14, the deblocking process for a top or left boundary of a CTB is skipped if the boundary coincides with the boundaries of a subpicture for which loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0, wherein SubPicIdx is the index of the current subpicture. The purpose in the design of subpictures may be infringed in two cases as shown in Fig. 1.

1) loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 1 but loop_filter_across_subpic_enabled_flag[ SubPicIdx’ ] is equal to 0, where SubPicIdx’ is the subpicture index of a neighbouring subpicture left or above to the current subpicture. In this case, samples in the two subpictures around the subpicture boundary are filtered, breaking the independency of the neighbouring subpicture.

2) loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0 but loop_filter_across_subpic_enabled_flag[ SubPicIdx’ ] is equal to 1, where SubPicIdx’ is the subpicture index of a neighbouring subpicture left or above to the current subpicture. In this case, samples in the two subpictures around the subpicture boundary are not filtered. Although the independency of the current subpicture is not broken, the filtering on samples in the neighbouring subpicutre is uncessarily forbiden, which may deterorite the coding efficiency of the neighbouring subpicutre.

(a)

(b)

Fig. 1 Deblocking filtering controlling for sub-picture boundaries in case 1(a) and case 2 (b) in VVC.

# Proposed methods

In this contribution, two solutions are poposed.

## Solution #1

In solution #1, it is proposed the loop_filter_across_subpic_enabled_flag of a subpicture can only control whether to apply deblocking filtering on samples inside the subpicture. In this way, SAO, ALF and deblocking filtering will follow a consistent rule on boundaries between subpictures. Fig. 2 shows the proposed controlling rules in two cases.

(a)

(b)

Fig. 2 Deblocking filtering controlling for sub-picture boundaries in case 1(a) and case 2 (b) as proposed.

The working draft changes of solution #1 based on JVET-P2001-v14 are as follows, with the deleted text and newly added text highlighted in turquois and yellow, respectively.

### 8.8.3 Deblocking filter process

#### 8.8.3.1 General

Inputs to this process are the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

…

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

- Edges that are at the boundary of the picture,

- Edges that coincide with the boundaries of a subpicture for which loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0,

- Edges that coincide with the virtual boundaries of the picture when VirtualBoundariesDisabledFlag is equal to 1,

- …

#### 8.8.3.2 Deblocking filter process for one direction

Inputs to this process are:

- the variable treeType specifying whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

…

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left or right boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- …

- Otherwise, if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top or bottom boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- …

#### 8.8.3.6.6 Filtering process for a luma sample using short filters

…

When nDp is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample p0 is equal to 1, nDp is set equal to 0

When nDq is greater than 0 and pred_mode_plt_flag of the coding unit that includes the coding block containing the sample q0 is equal to 1, nDq is set equal to 0:

When all the conditions below are satisfied:

nDp is greater than 0

subPicIdxP  is not equal to subPicIdxQ

loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0

nDp is set equal to 0, wherein subPicIdxP is the subpicture index of the subpicture containing the sample p0 and subPicIdxQ is the subpicture index of the subpicture containing the sample q0.

When all the conditions below are satisfied:

nDq is greater than 0

subPicIdxP  is not equal to subPicIdxQ

loop_filter_across_subpic_enabled_flag[ subPicIdxQ ] is equal to 0

nDq is set equal to 0, wherein subPicIdxP is the subpicture index of the subpicture containing the sample p0 and subPicIdxQ is the subpicture index of the subpicture containing the sample q0.

#### 8.8.3.6.7 Filtering process for a luma sample using long filters

…

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1, the filtered sample value, qi′ is substituted by the corresponding input sample value qj with j = 0..maxFilterLengthQ − 1.

When subPicIdxP  is not equal to subPicIdxQ and loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index of the subpicture containing the sample p0 and subPicIdxQ is the subpicture index of the subpicture containing the sample q0, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When subPicIdxP  is not equal to subPicIdxQ and loop_filter_across_subpic_enabled_flag[ subPicIdxQ ] is equal to 0, wherein subPicIdxP is the subpicture index of the subpicture containing the sample p0 and subPicIdxQ is the subpicture index of the subpicture containing the sample q0, the filtered sample value, qi′ is substituted by the corresponding input sample value qj with j = 0..maxFilterLengthQ − 1.

#### 8.8.3.6.9 Filtering process for a chroma sample

…

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample pi is equal to 1, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When pred_mode_plt_flag of the coding unit that includes the coding block containing the sample qi is equal to 1, the filtered sample value, qi′ is substituted by the corresponding input sample value qi with i = 0..maxFilterLengthQ − 1:

When subPicIdxP  is not equal to subPicIdxQ and loop_filter_across_subpic_enabled_flag[ subPicIdxP ] is equal to 0, wherein subPicIdxP is the subpicture index of the subpicture containing the sample p0 and subPicIdxQ is the subpicture index of the subpicture containing the sample q0, the filtered sample value, pi′ is substituted by the corresponding input sample value pi with i = 0..maxFilterLengthP − 1.

When subPicIdxP  is not equal to subPicIdxQ and loop_filter_across_subpic_enabled_flag[ subPicIdxQ ] is equal to 0, wherein subPicIdxP is the subpicture index of the subpicture containing the sample p0 and subPicIdxQ is the subpicture index of the subpicture containing the sample q0, the filtered sample value, qi′ is substituted by the corresponding input sample value qi with i = 0..maxFilterLengthQ − 1:

## Solution #2

In solution #2, the spec is modified such that the deblocking filter is not applied on a boundary between two subpictures if loop_filter_across_subpic_enabled_flag[ ] of either of the two subpictures is equal to 0.

The working draft changes of solution #2 based on JVET-P2001-v14 are as follows, with the deleted text and newly added text highlighted in turquois and yellow, respectively.

### 8.8.3 Deblocking filter process

#### 8.8.3.1 General

Inputs to this process are the reconstructed picture prior to deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

Outputs of this process are the modified reconstructed picture after deblocking, i.e., the array recPictureL and, when ChromaArrayType is not equal to 0, the arrays recPictureCb and recPictureCr.

…

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

- Edges that are at the boundary of the picture,

- Edges that coincide with the boundaries of a subpicture for which loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0,

- Edges that coincide with the boundaries of a subpicture X and loop_filter_across_subpic_enabled_flag[ SubPicXIdx ] is equal to 0, wherein SubPicXidx is the subpicture index of the subpicture X.

- Edges that coincide with the virtual boundaries of the picture when VirtualBoundariesDisabledFlag is equal to 1,

- …

#### 8.8.3.2 Deblocking filter process for one direction

Inputs to this process are:

- the variable treeType specifying whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

…

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left or right boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The left boundary of the current coding block concides with a left or a right boundary of a subpicture X and loop_filter_across_subpic_enabled_flag[ SubPicXIdx ] is equal to 0, wherein SubPicXidx is the subpicture index of the subpicture X.

- …

- Otherwise, if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top or bottom boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The top boundary of the current coding block concides with a top or a bottom boundary of a subpicture X and loop_filter_across_subpic_enabled_flag[ SubPicXIdx ] is equal to 0, wherein SubPicXidx is the subpicture index of the subpicture X.

- …

# Conclusions

Since the proposed method #1 makes the concept of subpicture more consistent, it is suggested adopting solution #1 in this proposal to VVC.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).