| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0319-v1 |
| --- | --- |

| Title: | Non-CE5: On SEI for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Hai Wei Sun Chong Soon Lim | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com haiwei.sun@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes a use of ALF module. This is a further follow-up on JVET-P0179. In the case when ALF module is disabled through bitstream header, an adaptive loop filter control SEI is decoded. The proposed SEI is used to turn on the ALF module to filter the reconstructed image for output using the accompanied ALF filter flags and filter coefficients.

Using the proposed SEI, it is possible to customize the picture quality. In an example, customized sets of sharpening filter coefficients combined with low-pass filter coefficients can be used in ALF module to sharpen the output YUV to enhance the edges and smoothen the unwanted noise of an image. A use-case application is in post-processing.

# Introduction

In the VTM7, an Adaptive Loop Filter (ALF) with block-based filter adaption is applied. For the luma component, one among 25 filters is selected for each 4×4 block, based on the direction and activity of local gradients. The ALF filter parameters are signalled in Adaptation Parameter Set (APS). In one APS, up to 25 sets of luminance filter coefficients and clipping value indexes, and up to eight sets of chrominance filter coefficients and clipping value indexes could be signalled.

The ALF filter coefficients are calculated in the encoder and the filter sets which have the lowest rate distortion costs are used and signalled. Figure 1 shows VTM7 decoder block diagram and location of ALF.

Figure 1 – VTM7 decoder block diagram

In the case when ALF module is disabled in the bitstream header, the decoder block diagram becomes Figure 2.

Figure 2 – VTM7 decoder block diagram when ALF is disabled

# Classification of ALF Luma

In the ALF module, the whole luma frame is classified into 25 classifications. Because of these classifications, it is possible to use different filters for each classification.

For example, Spincalendar 1280x720@50p, RA QP32 coded bitstream with ALF off. Some selected classifications and their respective luma pixels.

Class 0

Class 4

Class 22

Class 23

Class 4 contains most of the numbers and characters of the calendar. Other features are scattered in other classes. It is possible to customize picture quality for different features of the frame by using different kind of filters for each class. For example, apply sharpening filter to class 4 to enhance the numbers and characters and apply other types of filters to other classes to create a contrast look to the frame.

# Proposal

We propose an adaptive loop filter control SEI which can be used to enable ALF module for processing output pictures in the case when ALF module is disabled in the bitstream (Figure 3).

Figure 3 – VTM7 decoder block diagram when ALF is disabled with proposal

The picture level ALF filter on/off flags for luminance and Cb/Cr chrominance and their respective filter coefficients are signalled in the same SEI as shown in Aspect 1.

Aspect 1: adaptive loop filter control SEI

| adaptive_loop_filter_control( payloadSize ) { | Descriptor |
| --- | --- |
| alf_enabled_flag | u(1) |
| if( alf_enabled_flag ) { |  |
| alf_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| alf_chroma_idc | u(2) |
| alf_data( ) |  |
| } |  |
| } |  |

alf_data( ): Adaptive loop filter data syntax from JVET-Q0041-v2_DraftText.docx

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| if( alf_luma_filter_signal_flag ) { |  |
| alf_luma_clip_flag | u(1) |
| alf_luma_num_filters_signalled_minus1 | ue(v) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | u(v) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| for( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| if( alf_luma_clip_flag ) |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| for( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_num_alt_filters_minus1 | ue(v) |
| for( altIdx = 0; altIdx <= alf_chroma_num_alt_filters_minus1; altIdx++ ) { |  |
| alf_chroma_clip_flag[ altIdx ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ altIdx ][ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ altIdx ][ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |
| } |  |
| if( alf_chroma_clip_flag[ altIdx ] ) |  |
| for( j = 0; j < 6; j++ ) |  |
| alf_chroma_clip_idx[ altIdx ][ j ] | u(2) |
| } |  |
| } |  |
| } |  |

The CTB level on/off ALF filter flags can also be signalled additionally as shown in Aspect 2.

Aspect 2: adaptive loop filter control SEI with CTB level mask

| adaptive_loop_filter_control( payloadSize ) { | Descriptor |
| --- | --- |
| alf_enabled_flag | u(1) |
| if( alf_enabled_flag ) |  |
| alf_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| alf_chroma_idc | u(2) |
| if ( alf_luma_flag ) { |  |
| alf_luma_ctb_enable_flag | u(1) |
| if( alf_luma_ctb_enable_flag ) { |  |
| for( ctbAddrRs = 0; ctbAddrRs < PicSizeMaxInCtbsY; ctbAddrRs++ ) { |  |
| CtbAddrX = ( ctbAddrRs % PicWidthMaxInCtbsY ) |  |
| CtbAddrY = ( ctbAddrRs / PicWidthMaxInCtbsY ) |  |
| alf_ctb_flag [0] [CtbAddrX][ CtbAddrY] | u(1) |
| } |  |
| } |  |
| } |  |
| if ( alf_chroma_idc ) { |  |
| alf_chroma_ctb_enable_flag | u(1) |
| if( alf_chroma_ctb_enable_flag ) { |  |
| for( ctbAddrRs = 0; ctbAddrRs < PicSizeMaxInCtbsY; ctbAddrRs++ ) { |  |
| CtbAddrX = ( ctbAddrRs % PicWidthMaxInCtbsY ) |  |
| CtbAddrY = ( ctbAddrRs / PicWidthMaxInCtbsY ) |  |
| if ( alf_chroma_idc == 1 ¦¦ alf_chroma_idc == 3 ) |  |
| alf_ctb_flag [1] [CtbAddrX][ CtbAddrY] | u(1) |
| if ( alf_chroma_idc == 2 ¦¦ alf_chroma_idc == 3 ) |  |
| alf_ctb_flag [2] [CtbAddrX][ CtbAddrY] | u(1) |
| } |  |
| } |  |
| } |  |
| alf_data( ) |  |
| } |  |
| } |  |

The variables PicWidthMaxInCtbsY, PicHeightMaxInCtbsY and PicSizeMaxInCtbsY are derived as follows:

PicWidthMaxInCtbsY = Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY )

PicHeightMaxInCtbsY = Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY )

PicSizeMaxInCtbsY = PicWidthMaxInCtbsY * PicHeightMaxInCtbsY

If CC_ALF is adopted in the VVC specification, the proposed SEI is shown in Aspect 3 (Cb and Cr are separately enabled) or Aspect 4 (Cb and Cr are jointly enabled).

Aspect 3: adaptive loop filter control SEI with CC-ALF separate Cb/Cr enabled flag

| adaptive_loop_filter_control( payloadSize ) { | Descriptor |
| --- | --- |
| alf_enabled_flag | u(1) |
| if( alf_enabled_flag ) { |  |
| alf_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| alf_chroma_idc | u(2) |
| alf_cross_component_cb_flag | u(1) |
| alf_cross_component_cr_flag | u(1) |
| alf_data( ) |  |
| } |  |
| } |  |

Aspect 4: adaptive loop filter control SEI with CC-ALF combined Cb/Cr enabled flag

| adaptive_loop_filter_control( payloadSize ) { | Descriptor |
| --- | --- |
| alf_enabled_flag | u(1) |
| if( alf_enabled_flag ) { |  |
| alf_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| alf_chroma_idc | u(2) |
| alf_cross_component_cb_cr_flag | u(1) |
| alf_data( ) |  |
| } |  |
| } |  |

The semantics of adaptive loop filter control SEI are as follows:

The adaptive loop filter control SEI message provides information to use the adaptive loop filter to modify the pictures for output. When sps_alf_enabled_flag of the bitstream is equal to 1, there shall be no adaptive loop filter control SEI messages present in the CLVS.

Use of this SEI message requires the definition of the following function:

–	A function alf_data( ) that parses the adaptive loop filter data syntax.

alf_enabled_flag equal to 1 indicates that the adaptive loop filter is enabled.

alf_luma_flag equal to 1 specifies that adaptive loop filter is applied to the luminance component for all slices associated with the PH. alf_luma_flag equal to 0 specifies that adaptive loop filter is not applied to the luminance component for all slices associated with the PH.

When alf_luma_flag is not present, it is inferred to be equal to 0.

alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components.

When alf_chroma_idc is not present, it is inferred to be equal to 0.

alf_luma_ctb_enable_flag equal to 1 specifies that CTB level enabling flags for lumimance component are present. alf_ctb_enable_flag equal to 0 specifies that CTB level enabling flags for lumimance component are not present.

When alf_luma_ctb_enable_flag is not present, it is inferred to be equal to 0.

alf_chroma_ctb_enable_flag equal to 1 specifies that CTB level enabling flags for chrominance component Cb and Cr are present. alf_ctb_enable_flag equal to 0 specifies that CTB level enabling flags for chrominance component Cb and Cr are not present.

When alf_chroma_ctb_enable_flag is not present, it is inferred to be equal to 0.

alf_ctb_flag[ cIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 1 specifies that the adaptive loop filter is applied to the coding tree block of the colour component indicated by cIdx of the coding tree unit at luma location ( xCtb, yCtb ). alf_ctb_flag[ cIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] equal to 0 specifies that the adaptive loop filter is not applied to the coding tree block of the colour component indicated by cIdx of the coding tree unit at luma location ( xCtb, yCtb ).

When alf_ctb_flag[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to alf_luma_flag.

When alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to (alf_chroma_idc == 1 || alf_chroma_idc == 3).

When alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is inferred to be equal to (alf_chroma_idc == 2 || alf_chroma_idc == 3).

alf_cross_component_cb_flag equal to 1 specifies that adaptive loop filter is applied to the Cb color component. alf_cross_component_cb_flag equal to 0 specifies that adaptive loop filter is not applied to the Cb color component.

When alf_cross_component_cb_flag is not present, it is inferred to be equal to 0.

alf_cross_component_cr_flag equal to 1 specifies that adaptive loop filter is applied to the Cr color component. alf_cross_component_cr_flag equal to 0 specifies that adaptive loop filter is not applied to the Cr color component.

When alf_cross_component_cr_flag is not present, it is inferred to be equal to 0.

alf_cross_component_cb_cr_flag equal to 1 specifies that adaptive loop filter is applied to the Cb and Cr color components. alf_cross_component_cb_cr_flag equal to 0 specifies that adaptive loop filter is not applied to the Cb and Cr color components.

When alf_cross_component_cb_cr_flag is not present, it is inferred to be equal to 0.

# Encoder/Decoder implementation

For effective use of adaptive loop filter control SEI, one implementation method is suggested. In VTM7, maxDpbPicBuf size is 8. Figure 4 shows an example of VTM7 DPB buffer implementation where each buffer space consists of a flag to indicate “needed/not needed for output” and a flag to indicate “used/unused for short/long-term reference”.

Figure 4 – VTM7 DPB buffer implementation

Figure 5 shows an example of the proposed DPB buffer implementation. Each buffer space consists of:

- A set of flag and memory space to indicate “needed/not needed for output” and to store reconstructed image (before ALF), and

- A set of flag and memory space to indicate “used/unused for short/long-term reference” and to store images for output/display (e.g. ALF filtered reconstructed image).

Figure 5 – Proposed DPB buffer implementation

# Use case application

In the use-case of production studio, the content provider can customize the desired subjective quality and deliver to the consumer the same subjective quality by using VVC. Consumer can easily get the same subjective quality by just using VVC without the need of additional filtering modules.

In another use-case, an individual can use a production tool to customize the filters and classifications that he/she want to apply to and add the proposed SEI to the bitstream. The desired subjective quality can be embedded into the bitstream easily and uploaded for media sharing.

Figure 6 shows an example of what a production tool interface may look like.

Figure 6 – Example of production tool interface on tablet/mobile device

# Summary

The adaptive loop filter control SEI introduces new application uses for VVC. We propose to adopt at least one of the aspects of adaptive loop filter control SEI into VVC specification to expand the use case of VVC.

# References

[1] H.-B Teo, H.-W Sun, C.-S Lim, “Non-CE5: ALF as a post-filter”, JVET-P0179, JVET 16th Meeting: Geneva, CH, 1–11 October 2019.

[2] J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 7 (VTM 7)”, JVET-P2002, JVET 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).