| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0321-v2 |
| --- | --- |

| Title: | Non-CE1: Deblocking filter decision simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Chutong Wang Chong Soon Lim | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com chutong.wang@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposed to simplify the decision processes which selects normal or strong filtering in the deblocking filter. The proposed technique removes the multiplication steps in the decision process. The BDR impact versus VTM-7.0 is 0.00%, 0.00%, 0.00%, 0.01% for AI/RA/LDB/LDP.

# Introduction

This contribution proposed to simplify the decision processes which selects normal or strong filtering in the deblocking filter. Whether to apply strong or normal deblocking is determined based on the first and the fourth lines across the block boundary of four samples.

In VVC draft 7 [1], three checks on luma and chroma samples for deblocking filter decision are shown as below:

8.8.3.6.5	Decision process for a luma sample

- dpq is less than ( β  >>  2 ),

- sp + sq is less than sThr,

- spq is less than ( 5 * tC + 1 )  >>  1

8.8.3.6.8	Decision process for a chroma sample

- dpq is less than ( β  >>  2 ),

- Abs( p3 − p0 ) + Abs( q0 − q3 ) is less than ( β  >>  3 ),

- Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1.

wherein,

- The variable dpq is set equal to

    - 2 * dpq0L, 2 * dpq3L, 2 * dpq0, or 2 * dpq3 for luma

    - 2 * dpq0, or 2 * dpq1 for chroma

Section 2 introduces details on simplification on checks. Results including visual examples are shown in section 3 and conclusions in section 4.

# Proposed simplification on checks

In the decision process for luma/chroma sample, many operations and checks are used. Based on our investigations, the multiplication step to calculate dpq looks unnecessary and can be avoided by adjusting β. The decision process can be adjusted as below:

8.8.3.6.5	Decision process for a luma sample

- dpq is less than ( β  >>  2 3 ),

- sp + sq is less than sThr,

- spq is less than ( 5 * tC + 1 )  >>  1

8.8.3.6.8	Decision process for a chroma sample

- dpq is less than ( β  >>  2 3 ),

- Abs( p3 − p0 ) + Abs( q0 − q3 ) is less than ( β  >>  3 ),

- Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1.

wherein,

- The variable dpq is set equal to

    - 2 * dpq0L, 2 * dpq3L, 2 * dpq0, or 2 * dpq3 for luma

    - 2 * dpq0, or 2 * dpq1 for chroma

For detailed spec change locations, please refer the attached spec document.

# Results (Proposal)

The objective performance for the proposal is shown below in comparison to VTM-7.0 [2] using CTC [3]

|  | All intra Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 98% | 97% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | -0.01% | 103% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 108% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 105% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 106% | 103% |

|  | Random access Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.02% | 107% | 99% |
| Class A2 | 0.00% | 0.01% | 0.02% | 101% | 96% |
| Class B | 0.00% | 0.03% | 0.00% | 97% | 101% |
| Class C | 0.01% | -0.02% | -0.01% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 100% | 99% |
| Class D | 0.00% | 0.01% | -0.01% | 101% | 99% |
| Class F | 0.01% | -0.02% | -0.01% | 86% | 100% |

|  | Low-delay B Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.06% | -0.06% | 106% | 95% |
| Class C | 0.01% | -0.01% | 0.00% | 102% | 101% |
| Class E | 0.01% | 0.08% | -0.33% | 104% | 97% |
| Overall | 0.00% | 0.04% | -0.11% | 104% | 97% |
| Class D | 0.00% | 0.04% | 0.03% | 104% | 99% |
| Class F | -0.06% | -0.01% | 0.01% | 100% | 98% |

|  | Low-delay P Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.06% | -0.13% | 114% | 118% |
| Class C | 0.02% | -0.01% | -0.19% | 118% | 116% |
| Class E | 0.01% | -0.04% | 0.14% | 115% | 129% |
| Overall | 0.01% | 0.01% | -0.08% | 116% | 120% |
| Class D | 0.01% | -0.01% | 0.02% | 112% | 103% |
| Class F | -0.04% | -0.01% | 0.05% | 110% | 121% |

# Conclusions

We recommend adoption of this simplification on the decision process for deblocking filter to the VVC draft. This reduces unnecessary computation with negligible BDR impact.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 7)", Document of Joint Video Experts Team, JVET-Q0041.

- VTM-7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).