| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0322-v1 |
| --- | --- |

| Title: | CE1-related: Long luma deblocking filter decision modification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Boon Teo Chutong Wang Chong Soon Lim | Tel: Email: | +65-6550-5417 hanboon.teo@sg.panasonic.com chutong.wang@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

It is claimed that sometimes filtering for long luma deblocking is applied, although it should not. The wrong decision sometime causes visual artifacts. This contribution proposed to fix the problem by changing the position of the samples used for the gradient checks when max filter length equal to 7.

The BDR impact versus VTM-7.0 is 0.00%, 0.01%, -0.02%, 0.03% for AI/RA/LDB/LDP.

# Introduction

Block artifacts from boundaries between large luma blocks can be dramatically suppressed by long luma deblocking filter in VTM-7.0. There are three checks on long luma deblocking filter decision, and each check has its own purpose.

Consider a line of samples across the boundary between block P and Q, with 8 samples in block P and 8 samples in block Q as follows:

Samples:       p7 p6 p5 p4 p3 p2 p1 p0 | q0 q1 q2 q3 q4 q5 q6 q7

In VVC draft 7 [1], three checks for long luma deblocking filter decision are shown below:

CHECK1: checks that there is a low spatial activity on the side of block boundary.

- dpq is less than ( β  >>  2),

wherein dpq = p2 -2*p1 + p0+ p5-2*p4 + p3+ 1≫1

+ q2 -2*q1 + q0+ q5-2*q4 + q3+ 1≫1].

CHECK2: checks that the signal on the sides of the block boundary is flat.

- sp + sq is less than sThr,

wherein sp = p3 – p0+ p7- p3 + 1≫1;

sq = (|q3-q0|+|q7- q3)|+1)≫1.

CHECK3: checks that the difference in intensities of the immediate samples on two sides of the block does not exceed the threshold.

- spq is less than ( 5 * tC + 1 )  >>  1

wherein spq = | p0- q0|.

Only when all three checks are true, long luma deblocking filter can be applied. One of the three checks, gradient check (CHECK1), doesn’t cover all the necessary samples. The used samples are p0, p1, p2, p3, p4, p5, q0, q1, q2, q3, q4, q5, but p6, p7, q6, q7 are not involved.

CE1-1.2 proposed to include p6, p7, q6, q7 in CHECK2 with additional gradient calculations, while we suggest modifying the gradient check (CHECK1) without any changes on other checks (CHECK2 and CHECK3) and corresponding thresholds.

When its CHECK1 gradient range is extended, more samples can be naturally involved in this check which avoids wrong decisions to use the long luma deblocking filter. Section 2 proposes modified gradient calculations to extend check length and cover all samples.

Results including visual examples are shown in section 3 and conclusions in section 4.

# Modification on gradient check

Compared to HEVC, the three checks cover more samples to support long deblocking filter decision, but CHECK1 is still incomplete due to limited check range. When detecting spatial activity, neighbor samples are used for calculation on the centre point. (For example, |p2 -2*p1 + p0| can detect whether p1, centre point, is a peak or not).

CHECK1 includes 4 detections for 4 centre points (p1, q1, p4, q4), which is not enough when max filter length is 7. To allow more neighbor samples involve in the checking, the interval of gradient is extended from 1 to 2.

For example:

Before:			p2 - 2*p1 + p0 =|p2-p1-p1-p0|

After modified:	p4 -p3-p2+ p1=|p4-p2-p3-p1|

In HEVC, the filter length is half of that in VTM7.0, such difference may be not obvious to notice in subjective quality. But now it supports long deblocking filter in VTM7.0, the impact of incomplete gradient check can be seen clearly in results (Section 3).

To combat this issue, modified gradient calculations are added to the long luma filter decision when maxFilterLengthP or maxFilterLengthQ is equal to 7 as high-lighted in yellow on top of VVC draft 7[1]:

..

- When sidePisLargeBlk or sideQisLargeBlk is greater than 0, the following applies:

- The variables dp0L, dp3L are derived and maxFilterLengthP is modified as follows:

- If sidePisLargeBlk is equal to 1, the following applies:

dp0L = maxFilterLengthP==7 ? ( Abs( p6,0 – p5,0 – p4,0 + p3,0 ) + Abs( p4,0 – p3,0 – p2,0 + p1,0 ) + 1 ) >> 1:  ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1	 (1276)

dp3L = maxFilterLengthP==7 ? ( Abs( p6,3 – p5,3 – p4,3 + p3,3 ) + Abs( p4,3 – p3,3 – p2,3 + p1,3 ) + 1 ) >> 1: ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1	 (1277)

- Otherwise, the following applies:

dp0L = dp0							(1278)

dp3L = dp3							(1279)

maxFilterLengthP = 3					(1280)

..

# Results

## Visual examples

Figure 1: POC 97 DayStreet at QP 37 ALF ON, Left: Anchor, Right: Proposed fix with modified gradient check in VTM-7.0.

Figure 2: POC 97 DayStreet at QP 37 ALF OFF, Left: Anchor, Right: Proposed fix with modified gradient check in VTM-7.0.

## Objective performance SDR

The objective performance for the proposal is shown below in comparison to VTM-7.0 [2] using CTC [3]. It is noted that encoding and decoding timings are not accurate due to different PC clusters being used for the simulations. But encoding and decoding timing is expected to be similar to anchor.

### Proposed fixes for long luma filter decision vs VTM-7.0

|  | All intra Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 107% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 104% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 99% |

|  | Random access Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.00% | 97% | 98% |
| Class A2 | -0.01% | -0.02% | -0.01% | 98% | 92% |
| Class B | 0.01% | -0.01% | -0.06% | 99% | 101% |
| Class C | 0.01% | -0.04% | 0.01% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.02% | 99% | 98% |
| Class D | -0.01% | 0.01% | -0.01% | 102% | 99% |
| Class F | 0.01% | 0.06% | -0.03% | 85% | 99% |

|  | Low-delay B Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | -0.38% | -0.06% | 99% | 97% |
| Class C | -0.03% | -0.20% | -0.28% | 108% | 96% |
| Class E | -0.13% | 0.06% | -0.48% | 98% | 97% |
| Overall | -0.02% | -0.21% | -0.24% | 102% | 97% |
| Class D | -0.08% | 0.44% | 0.74% | 108% | 98% |
| Class F | 0.02% | -0.71% | 0.66% | 107% | 101% |

|  | Low-delay P Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.28% | -0.10% | 117% | 119% |
| Class C | -0.01% | 0.14% | -0.11% | 113% | 111% |
| Class E | 0.14% | 0.16% | -0.28% | 109% | 125% |
| Overall | 0.03% | 0.20% | -0.15% | 114% | 118% |
| Class D | 0.00% | 0.22% | 0.30% | 114% | 108% |
| Class F | 0.05% | -0.04% | 0.00% | 112% | 121% |

# Conclusions

We recommend adoption of this modification for long luma deblocking filter decision to the VVC draft. This improves the decision-making process for long luma deblocking filter, suppresses block artifacts and does not introduce additional complexity at the same time.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 7)", Document of Joint Video Experts Team, JVET-Q0041.

- VTM-7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).