| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0323-v1 |
| --- | --- |

| Title: | Non-CE3: Encoder optimization for chroma BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chi-Chang Kuo, Sheng-Po Wang, Ching-Chieh Lin1, Chun-Lung Lin2 | Tel: Email: | +886-3-5915758 JackLin@itri.com1 chunlung@itri.com2 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

Chroma BDPCM can provide benefit on coding efficiency especially for lossless test condition. This contribution proposed 2 methods of encoder optimization for chroma BDPCM. Both methods were implemented on top of CE3-2.2 and CE3-2.4a source codes and compared over the results of them.

When using CE3-2.2 as anchor, the overall bit-rate savings for the proposed method 1 are 0.87% and 0.24%, with encoding time savings -11% and -2% for AI and RA, respectively. For the proposed method 2, the overall bit-rate savings are 0.36% and 0.20%, with encoding time savings -9% and -1% for AI and RA, respectively.

When using CE3-2.4a as anchor, the overall bit-rate savings for the proposed method 1 are 0.84% and 0.17%, with -8% and -1% encoding time saving for AI and RA, respectively. For the proposed method 2, the overall bit-rate savings are 0.38% and 0.14%, with encoding time savings -5% and -1% for AI and RA, respectively.

Introduction

In current VTM-7.0 [1] software, the encoding search for chroma intra mode is implemented by SATD based fast search in the first round. In the second round, 5 candidates with best SATD costs will be further examined with full RD searches to determine the final intra mode. When chroma BDPCM is enabled, there are 2 more candidates included for full RD searches in the second round.

Therefore, this contribution proposes a fast mode decision method for chroma BDPCM by inferring the coding mode selection of luma BDPCM to reduce the number of SATD calculations and full RD searches for chroma BDPCM and reduce the encoding time.

Proposed Methods

Variables for the poposed process are described as follows:

- luma location ( x0, y0 ) specifying the top-left sample of the current chroma coding block relative to the top left luma sample of the current picture,

- cbWidth and cbHeight specifying the width and height of the current coding block in luma samples,

- luma location ( xRef, yRef ) specifying the reference position and it is set equal to ( x0 + cbWidth / 2, y0 + cbHeight / 2 ),

- lumaBdpcmFlag and lumaBdpcmDirFlag are set equal to the syntax elements intra_bdpcm_luma_flag[ xRef ][ yRef ] and intra_bdpcm_luma_dir_flag[ xRef ][ yRef ], respectively,

- fastChromaBdpcm equal to 1 specifies that the proposed fast BDPCM mode decision is applied to the current chroma coding block.

The function estIntraPredChromaQT( ) of encoder source code is modified by the proposed process. When variable fastChromaBdpcm is equal to 1, the following applies:

- skip SATD based fast search in the first round,

- set coding mode of the current chroma block to BDPCM (i.e., intra_bdpcm_chroma_flag[ x0 ][ y0 ] = 1),

- set BDPCM prediction direction of the current chroma block to lumaBdpcmDirFlag (i.e., intra_bdpcm_chroma_dir_flag[ x0 ][ y0 ] = lumaBdpcmDirFlag),

- for the full RD search in the second round, only 1 candidate with BDPCM mode is evaluated.

There are 2 methods to derive variable fastChromaBdpcm:

- Method 1:

  - If all the following conditions are true, fastChromaBdpcm is set equal to 1:

    - Test of chroma BDPCM is allowed.

    - lumaBdpcmFlag is equal to 1.

  - Otherwise, fastChromaBdpcm is set equal to 0.

- Method 2:

  - If all the following conditions are true, fastChromaBdpcm is set equal to 1:

    - Test of chroma BDPCM is allowed.

    - lumaBdpcmFlag is equal to 1.

    - (width * height) in chroma samples is smaller than or equal to 64.

  - Otherwise, fastChromaBdpcm is set equal to 0.

Simulation Results

The tests were conducted under CE3 test condition [2]. The implementations were on top of the CE3-2.2 [3] and CE3-2.4a [4] source codes. There are 4 tests for the proposed methods described as follows:

- Test 1: method 1 on top of the CE3-2.2 source code.

- Test 2: method 2 on top of the CE3-2.2 source code.

- Test 3: method 1 on top of the CE3-2.4a source code.

- Test 4: method 2 on top of the CE3-2.4a source code.

Table 3-1 shows the results of Test 1, when use VTM7.0 as anchor.

Table 3-2 shows the results of Test 2, when use VTM7.0 as anchor.

Table 3-3 shows the results of Test 3, when use VTM7.0 as anchor.

Table 3-4 shows the results of Test 4, when use VTM7.0 as anchor.

Table 3-5 shows the results of Test 1, when use CE3-2.2 as anchor.

Table 3-6 shows the results of Test 2, when use CE3-2.2 as anchor.

Table 3-7 shows the results of Test 3, when use CE3-2.4a as anchor.

Table 3-8 shows the results of Test 4, when use CE3-2.4a as anchor.

Table 3-9 shows the overall results for CE3-2.2, CE3-2.4a, and Tests 1~4 for comparison.

Table 3-1: Results of Test 1 (method 1 on top of the CE3-2.2 source code)

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test 1 |  | VTM7 | Test 1 |  |
| Class A1 | 2.2 | 2.3 | -0.76% | 2.2 | 2.3 | -0.41% |
| Class A2 | 1.6 | 1.6 | -1.27% | 1.7 | 1.7 | -0.30% |
| Class B | 2.2 | 2.2 | -2.27% | 2.3 | 2.4 | -0.69% |
| Class C | 1.9 | 2.0 | -2.08% | 2.4 | 2.4 | -0.55% |
| Class D | 1.9 | 2.0 | -4.15% | 2.8 | 2.8 | -0.80% |
| Class E | 2.8 | 2.9 | -2.73% |  |  |  |
| Class F | 5.3 | 5.6 | -4.58% | 33.7 | 34.7 | -1.93% |
| TGM | 11.8 | 12.1 | -2.08% | 107.1 | 108.2 | -1.07% |
| Overall | 2.1 | 2.2 | -1.89% | 2.2 | 2.2 | -0.52% |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 89% |  |  | 97% |  |  |

Table 3-2: Results of Test 2 (method 2 on top of the CE3-2.2 source code)

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test 2 |  | VTM7 | Test 2 |  |
| Class A1 | 2.2 | 2.3 | -1.13% | 2.2 | 2.3 | -0.48% |
| Class A2 | 1.6 | 1.6 | -1.31% | 1.7 | 1.7 | -0.30% |
| Class B | 2.2 | 2.2 | -2.43% | 2.3 | 2.4 | -0.72% |
| Class C | 1.9 | 2.0 | -2.26% | 2.4 | 2.4 | -0.60% |
| Class D | 1.9 | 2.0 | -4.29% | 2.8 | 2.8 | -0.81% |
| Class E | 2.8 | 2.9 | -4.79% |  |  |  |
| Class F | 5.3 | 5.7 | -4.72% | 33.7 | 34.7 | -1.96% |
| TGM | 11.8 | 12.2 | -2.77% | 107.1 | 108.4 | -1.19% |
| Overall | 2.1 | 2.2 | -2.38% | 2.2 | 2.2 | -0.55% |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 87% |  |  | 97% |  |  |

Table 3-3: Results of Test 3 (method 1 on top of the CE3-2.4a source code)

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test 3 |  | VTM7 | Test 3 |  |
| Class A1 | 2.2 | 2.4 | -5.98% | 2.2 | 2.4 | -6.64% |
| Class A2 | 1.6 | 1.8 | -10.08% | 1.7 | 1.9 | -8.40% |
| Class B | 2.2 | 2.3 | -5.87% | 2.3 | 2.5 | -4.44% |
| Class C | 1.9 | 2.1 | -6.57% | 2.4 | 2.6 | -5.85% |
| Class D | 1.9 | 2.1 | -8.34% | 2.8 | 2.9 | -6.00% |
| Class E | 2.8 | 3.0 | -6.97% |  |  |  |
| Class F | 5.3 | 5.7 | -6.66% | 33.7 | 35.1 | -4.85% |
| TGM | 11.8 | 12.0 | -1.76% | 107.1 | 107.9 | -0.94% |
| Overall | 2.1 | 2.3 | -6.93% | 2.2 | 2.4 | -6.05% |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 92% |  |  | 97% |  |  |

Table 3-4: Results of Test 4 (method 2 on top of the CE3-2.4a source code)

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test 4 |  | VTM7 | Test 4 |  |
| Class A1 | 2.2 | 2.4 | -6.23% | 2.2 | 2.4 | -6.68% |
| Class A2 | 1.6 | 1.8 | -10.23% | 1.7 | 1.9 | -8.41% |
| Class B | 2.2 | 2.3 | -5.95% | 2.3 | 2.5 | -4.47% |
| Class C | 1.9 | 2.1 | -6.70% | 2.4 | 2.6 | -5.88% |
| Class D | 1.9 | 2.1 | -8.42% | 2.8 | 2.9 | -6.02% |
| Class E | 2.8 | 3.0 | -8.78% |  |  |  |
| Class F | 5.3 | 5.7 | -6.79% | 33.7 | 35.1 | -4.92% |
| TGM | 11.8 | 12.1 | -2.34% | 107.1 | 108.0 | -1.04% |
| Overall | 2.1 | 2.3 | -7.35% | 2.2 | 2.4 | -6.08% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 90% |  |  | 97% |  |  |

Table 3-5: Results of Test 1 when use CE3-2.2 as anchor

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE3-2.2 | Test 1 |  | CE3-2.2 | Test 1 |  |
| Class A1 | 2.3 | 2.3 | 0.57% | 2.3 | 2.3 | 0.56% |
| Class A2 | 1.6 | 1.6 | 0.73% | 1.7 | 1.7 | 0.19% |
| Class B | 2.2 | 2.2 | 0.42% | 2.4 | 2.4 | 0.13% |
| Class C | 2.0 | 2.0 | 0.59% | 2.4 | 2.4 | 0.17% |
| Class D | 2.0 | 2.0 | 0.59% | 2.8 | 2.8 | 0.07% |
| Class E | 2.9 | 2.9 | 2.45% |  |  |  |
| Class F | 5.7 | 5.6 | 0.90% | 34.9 | 34.7 | 0.32% |
| TGM | 12.5 | 12.1 | 3.07% | 109.4 | 108.2 | 0.98% |
| Overall | 2.2 | 2.2 | 0.87% | 2.2 | 2.2 | 0.24% |
| Enc Time[%] | 89% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3-6: Results of Test 2 when use CE3-2.2 as anchor

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE3-2.2 | Test 2 |  | CE3-2.2 | Test 2 |  |
| Class A1 | 2.3 | 2.3 | 0.19% | 2.3 | 2.3 | 0.49% |
| Class A2 | 1.6 | 1.6 | 0.69% | 1.7 | 1.7 | 0.18% |
| Class B | 2.2 | 2.2 | 0.26% | 2.4 | 2.4 | 0.10% |
| Class C | 2.0 | 2.0 | 0.41% | 2.4 | 2.4 | 0.12% |
| Class D | 2.0 | 2.0 | 0.45% | 2.8 | 2.8 | 0.06% |
| Class E | 2.9 | 2.9 | 0.29% |  |  |  |
| Class F | 5.7 | 5.7 | 0.74% | 34.9 | 34.7 | 0.29% |
| TGM | 12.5 | 12.2 | 2.34% | 109.4 | 108.4 | 0.85% |
| Overall | 2.2 | 2.2 | 0.36% | 2.2 | 2.2 | 0.20% |
| Enc Time[%] | 91% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

Table 3-7: Results of Test 3 when use CE3-2.4a as anchor

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE3-2.4a | Test 3 |  | CE3-2.4a | Test 3 |  |
| Class A1 | 2.4 | 2.4 | 0.67% | 2.4 | 2.4 | 0.42% |
| Class A2 | 1.8 | 1.8 | 0.39% | 1.9 | 1.9 | 0.08% |
| Class B | 2.3 | 2.3 | 0.47% | 2.5 | 2.5 | 0.10% |
| Class C | 2.1 | 2.1 | 0.66% | 2.6 | 2.6 | 0.15% |
| Class D | 2.1 | 2.1 | 0.68% | 2.9 | 2.9 | 0.06% |
| Class E | 3.1 | 3.0 | 2.33% |  |  |  |
| Class F | 5.8 | 5.7 | 1.02% | 35.3 | 35.1 | 0.38% |
| TGM | 12.4 | 12.0 | 3.17% | 109.1 | 107.9 | 0.98% |
| Overall | 2.3 | 2.3 | 0.84% | 2.4 | 2.4 | 0.17% |
| Enc Time[%] | 92% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

Table 3-8: Results of Test 4 when use CE3-2.4a as anchor

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE3-2.4a | Test 4 |  | CE3-2.4a | Test 4 |  |
| Class A1 | 2.4 | 2.4 | 0.40% | 2.4 | 2.4 | 0.37% |
| Class A2 | 1.8 | 1.8 | 0.23% | 1.9 | 1.9 | 0.06% |
| Class B | 2.3 | 2.3 | 0.38% | 2.5 | 2.5 | 0.07% |
| Class C | 2.1 | 2.1 | 0.52% | 2.6 | 2.6 | 0.12% |
| Class D | 2.1 | 2.1 | 0.58% | 2.9 | 2.9 | 0.04% |
| Class E | 3.1 | 3.0 | 0.34% |  |  |  |
| Class F | 5.8 | 5.7 | 0.88% | 35.3 | 35.1 | 0.31% |
| TGM | 12.4 | 12.1 | 2.57% | 109.1 | 108.0 | 0.88% |
| Overall | 2.3 | 2.3 | 0.38% | 2.4 | 2.4 | 0.14% |
| Enc Time[%] | 95% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 100% |  |  |

Table 3-9: Comparison of CE3-2.2, CE3-2.4a, and Tests 1~4

|  | Over VTM-7.0, Lossless, AI |  |  | Over VTM-7.0, Lossless, RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | bit-rate savings | EncT | DecT | bit-rate savings | EncT | DecT |
| CE3-2.2 | -2.73% | 116% | 90% | -0.76% | 106% | 97% |
| Test 1 (method 1 on top of CE3-2.2) | -1.89% | 102% | 89% | -0.52% | 104% | 97% |
| Test 2 (method 2 on top of CE3-2.2) | -2.38% | 106% | 87% | -0.55% | 105% | 97% |
| CE3-2.4a | -7.70% | 107% | 94% | -6.21% | 102% | 97% |
| Test 3 (method 1 on top of CE3-2.4a) | -6.93% | 99% | 92% | -6.05% | 101% | 97% |
| Test 4 (method 2 on top of CE3-2.4a) | -7.35% | 102% | 90% | -6.08% | 102% | 97% |

Conclusions

This contribution proposed two methods to reduce encoding time for chroma BDPCM. Simulation results show that the encoding time of proposed method 2 were 91% and 99% for AI and RA respectively when compared to CE3-2.2. It is suggested to adopt the proposed method 2 into VTM software.

References

- VTM-7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

- T. Nguyen, T. C. Ma, and A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding,” JVET-P2023, October 2019.

- A. Nalci, H. Wang, M. Coban, M. Karczewicz, J. Choi, H. Jang, S. Yoo, J. Heo, J. Lim, S. Kim, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE3-2.1 and CE3-2.2: Luma/Chroma BDPCM for Lossless Coding with Transform Skip Residual Coding”, JVET-Q0086, January 2020.

- A. Nalci, H. Wang, M. Coban, M. Karczewicz, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding”, JVET-Q0089, January 2020.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).