| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0325-v1 |
| --- | --- |

| Title: | CE1-related: Deblocking for GEO (Geometric Partition Mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, SZ DJI Technology Co., Ltd.2 | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

In this contribution, deblocking filter for GEO (Geometric Partition Mode) CU boundaries is proposed. This contribution addresses the potential blocking artifacts of the current deblocking design for GEO. The potential discontinuity of block boundary is mainly caused by the discrepancy of the weights in GEO. To resolve this problem, this contribution proposes to conduct deblocking filtering on these overlooked boundaries between GEO blending area and another CU in the current VVC.

BD-rate are summarized as follows,

Anchor: VTM7.0 CE4 common base

Test: Anchor + deblocking for GEO

RA:   0.00% (Y), 0.00% (Cb), 0.00% (Cr)
LDB:  -0.01% (Y), 0.04% (Cb), -0.33% (Cr)

Anchor: VTM7.0 CE4 common base with ALF OFF

Test: Anchor + deblocking for GEO

RA:   0.00% (Y), 0.00% (Cb), 0.02% (Cr)
LDB:  -0.01% (Y), 0.04% (Cb), -0.33% (Cr)

Introduction

The deblocking for triangle partition merge (TPM) CU boundaries was proposed in JVET-P0269[4] to improve visual quality. GEO (Geometric partitioning mode), which introduces more partition candidates compared to TPM, is investigated in CE4[5]. GEO uses the similar blending method and motion vector storage strategy as TPM. Hence, the potential discontinuity of GEO block boundaries is also an issue which need to be fixed.

Figure 1.  Blending process of GEO

Proposed Method

To resolve these problems mentioned above, the proposed method is as follows,

Deblocking for GEO

The spec changes are shown bellow to elaborate the proposed method.

–	Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

–	The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

–	The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

–	The coding subblock containing the sample p0 is geometric mode, and the coding subblock containing the sample p0 is different from that of the sample q0, and sample is in the blending area of corresponding coding subblock.

–	The coding subblock containing the sample q0 is geometric mode, and the coding subblock containing the sample p0 is different from that of the sample q0, and sample is in the blending area of corresponding coding subblock.

Simulation results

The experiments were performed based on CE4 common base software [5] with VTM configuration files under JVET common test condition [3]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

The experimental results of the proposed method compared to CE4 common based are summarized in Table 1. The average BD-rates of 0.00%, and -0.xx% are observed for RA, and LDB cases, respectively.

The experimental results of the proposed method compared to CE4 common based with ALF OFF are summarized in Table 2. The average BD-rates of 0.xx%, and 0.xx% are observed for RA, and LDB cases, respectively.

Table 1. Experimental result of the proposed method(Anchor: CE4 common base [5])

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 89% | 82% |
| Class A2 | 0.02% | -0.04% | 0.00% | 97% | 88% |
| Class B | -0.01% | 0.02% | -0.06% | 85% | 74% |
| Class C | 0.00% | 0.01% | 0.07% | 84% | 74% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 88% | 78% |
| Class D | 0.02% | 0.03% | -0.02% | 84% | 74% |
| Class F | 0.00% | 0.02% | 0.02% | 84% | 74% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.02% | 0.01% | -0.10% | 69% | 61% |
| Class E | 0.19% | -0.26% | -0.15% | 74% | 64% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.32% | 0.24% | 73% | 61% |
| Class F | 0.02% | 0.31% | -0.06% | 73% | 64% |

Table 2. Experimental result of the proposed method(Anchor: CE4 common base with ALF OFF)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.02% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.00% | 0.00% | -0.02% | 101% | 99% |
| Class C | 0.01% | -0.02% | -0.01% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.00% | -0.02% | 83% | 94% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 0.02% | 0.19% | -0.22% | 101% | 105% |
| Class E | 0.06% | -0.66% | -0.53% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.03% | 0.71% | 0.33% | 100% | 99% |
| Class F | 0.08% | -0.11% | 0.23% | 99% | 100% |

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

- X.W. Meng (PKU), X. Zheng (DJI), S.S. Wang, S.W. Ma (PKU), “AHG18 / non-CE5: Deblocking for TPM and BCW”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P0269, Oct. 2019

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei), K. Reuze, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), R.-L. Liao, J. Chen, Y. Ye, J. Luo (Alibaba), M. Blaser, J. Sauer (RWTH Aachen), “CE4 Common Base: Geometric inter prediction”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q0079, Jan. 2020

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).