| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0326-v1 |
| --- | --- |

| Title: | CE5-related: On CC-ALF slice and picture header syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, SZ DJI Technology Co., Ltd.2 | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

In this contribution, a cross component adaptive loop filter (CC-ALF) slice and picture header syntax cleanup method is proposed. Specifically, it is proposed to remove the following syntax: pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_cr_filters_signalled_minus1, slice_cross_component_cb_filters_signalled_minus1 and slice_cross_component_cr_filters_signalled_minus1, since they are unnecessary.

Introduction

In CE5, a Cross Component Adaptive Loop Filter (CC-ALF) is investigated[2] to refine the chroma components by using luma samples. CC-ALF is part of the ALF process. Separate side information is signalled in the bitstream to control CC-ALF. In the CE5 common base description, the number of filters and filter coefficients are signalled in ALF APS. However, the number of CC-ALF filters is also signalled in PH and SH, which is unnecessary.

Proposed Method

To resolve these problems mentioned above, this contribution proposes to remove the PH and SH syntax related to CCALF filter number, pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_cr_filters_signalled_minus1, slice_cross_component_cb_filters_signalled_minus1 and slice_cross_component_cr_filters_signalled_minus1.

The modifactions of Spec are as follows,

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| …… | u(1) |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

pic_alf_enabled_present_flag equal to 1 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, pic_alf_aps_id_chroma, pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id,  pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id, and pic_cross_component_cr_filters_signalled_minus1 are present in the PH. pic_alf_enabled_present_flag equal to 0 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, pic_alf_aps_id_chroma, pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id,  pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id, and pic_cross_component_cr_filters_signalled_minus1 are not present in the PH. When pic_alf_enabled_present_flag is not present, it is inferred to be equal to 0.

pic_cross_component_cb_filters_signalled_minus1 plus 1 specifies the number of cross component Cb filters. The value of pic_cross_component_cb_filters_signalled_minus1 shall be in the range 0 to 3

When pic_cross_component_alf_cb_enabled_flag equal to 1, it is a requirement of bitstream conformance that pic_cross_component_cb_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cb_filters_signalled_minus1 in the referenced ALF APS referred to by pic_cross_component_alf_cb_aps_id.

pic_cross_component_cr_filters_signalled_minus1 plus 1 specifies the number of cross component Cr filters. The value of pic_cross_component_cr_filters_signalled_minus1 shall be in the range 0 to 3

When pic_cross_component_alf_cr_enabled_flag equal to 1, it is a requirement of bitstream conformance that pic_cross_component_cr_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cr_filters_signalled_minus1 in the referenced ALF APS referred to by pic_cross_component_alf_cr_aps_id.

7.3.7	Slice header syntax

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … | u(v) |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

slice_cross_component_cb_filters_signalled_minus1 plus 1 specifies the number of cross component Cb filters. The value of slice_cross_component_cb_filters_signalled_minus1 shall be in the range 0 to 3. When slice_cross_component_alf_cb_enabled_flag is equal to 1 and slice_cross_component_cb_filters_signalled_minus1 is not present, the value of slice_cross_component_cb_filters_signalled_minus1 is inferred to be equal to the value of pic_cross_component_cb_filters_signalled_minus1.

When slice_cross_component_alf_cb_enabled_flag equal to 1, it is a requirement of bitstream conformance that slice_cross_component_cb_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cb_filters_signalled_minus1 in the ALF APS referred to by slice_cross_component_alf_cb_aps_id of current slice.

slice_cross_component_cr_filters_signalled_minus1 plus 1 specifies the number of cross component Cr filters. The value of slice_cross_component_cr_filters_signalled_minus1 shall be in the range 0 to 3. When slice_cross_component_alf_cr_enabled_flag is equal to 1 and slice_cross_component_cr_filters_signalled_minus1 is not present, the value of slice_cross_component_cr_filters_signalled_minus1 is inferred to be equal to the value of pic_cross_component_cr_filters_signalled_minus1.

When slice_cross_component_alf_cr_enabled_flag equal to 1, it is a requirement of bitstream conformance that slice_cross_component_cr_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cr_filters_signalled_minus1 in the referenced ALF APS referred to by slice_cross_component_alf_cr_aps_id of current slice.

| alf_ctb_cross_component_cb_idc[ ][ ] | TR | cMax = ( alf_cross_component_cb_filters_signalled_minus1 slice_cross_component_cb_filters_signalled_minus1 + 1 ), cRiceParam = 0 |
| --- | --- | --- |
| alf_ctb_cross_component_cr_idc[ ][ ] | TR | cMax = ( alf_cross_component_cr_filters_signalled_minus1 slice_cross_component_cr_filters_signalled_minus1 + 1 ), cRiceParam = 0 |

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, Oct. 2019

- K. Misra, F. Bossen, A. Segall, et.al, “CE5-related: On the design of CC-ALF,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P1008, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).