| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0329-v1 |
| --- | --- |

| Title: | AHG9: Syntax cleanup of chroma coding tools in 444 color format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Jie Chen, Yan Ye, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In VVC draft 7, for a sequence with 4:4:4 color format, the three components can be separately processed as monochrome pictures by setting a SPS flag separate_colour_plane_flag to true. In this case, there is no chroma components for the sequence. However, the JCCR, chroma BDPCM, ACT flag are still signaled in SPS, which is redundant. Besides, the palette mode can be enabled for 4:4:4 color format with separate_colour_plane_flag equal to true but cannot be enabled for monochrome color format. In this contribution, it is proposed to signal the JCCR, chroma BDPCM, ACT and palette flag only when chroma components exist.

Introduction

In VVC draft 7, when a sequence is 4:4:4 color format, a SPS flag separate_colour_plane_flag is signaled to indicate whether the three color components are coded separately. When this SPS flag is equal to 1, three components are coded as monochrome. That means, the variable ChromaArrayType is set equal to 0 according to the following semantics:

Depending on the value of separate_colour_plane_flag, the value of the variable ChromaArrayType is assigned as follows:

–	If separate_colour_plane_flag is equal to 0, ChromaArrayType is set equal to chroma_format_idc.

–	Otherwise (separate_colour_plane_flag is equal to 1), ChromaArrayType is set equal to 0.

There are three problems in current syntax design when separate_colour_plane_flag is equal to 1:

- There is syntax redundancy in SPS. The enabled flag for JCCR and chroma BDPCM are signaled regardless of ChromaArrayType.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| sps_joint_cbcr_enabled_flag | u(1) |
| … |  |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| if( sps_bdpcm_enabled_flag && chroma_format_idc = = 3 ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| … |  |
| } |  |

- The adaptive color transform, ACT, may be applied to a block even though there is no chroma components in this block. According to the specification, the ACT SPS flag can be true for 4:4:4 color format with separate_colour_plane_flag equal to 1. In this case, a CU level flag is signaled for each CU to indicate whether the ACT is applied to the CU.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( chroma_format_idc = = 3 ) { |  |
| sps_palette_enabled_flag | u(1) |
| sps_act_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_act_enabled_flag && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| … |  |
| } |  |

- There is inconsistency in palette mode design. The palette mode cannot be enabled for monochrome color format but can be enabled for 4:4:4 chroma format with separate_colour_plane_flag equal to true. It is better to clarify whether the palette mode is applied to monochrome color format.

Proposed methods

In this contribution, it is proposed to signal the JCCR, chroma BDPCM, ACT and palette flag only when ChromaArrayType is not equal to 0. The changes to VVC draft 7 are highlighted in yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| sps_joint_cbcr_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| sps_sao_enabled_flag | u(1) |
| sps_alf_enabled_flag | u(1) |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| if( sps_bdpcm_enabled_flag && chroma_format_idc = = 3 && ChromaArrayType != 0 ) |  |
| sps_bdpcm_chroma_enabled_flag | u(1) |
| sps_ref_wraparound_enabled_flag | u(1) |
| … |  |
| if( chroma_format_idc = = 3 && ChromaArrayType != 0 ) { |  |
| sps_palette_enabled_flag | u(1) |
| sps_act_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

Conclusion

This contribution proposes to clean up the syntax design for chroma coding tool in 4:4:4 color format with separate_colour_plane_flag equal to true. It is suggested to adopt this contribution to VVC.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).