| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0330-v1 |
| --- | --- |

| Title: | On block partitioning at picture boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ru-Ling Liao, Jie Chen, Yan Ye, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | ruling.lrl@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In VVC draft 7, a block that exceed the picture boundary is forced to be split until all the samples are located inside the picture boundary. However, none of splitting methods is allowed to split the block in the following two cases:

- A CTU block exceeds picture boundary and both the CTU size and min QT size are set to 128;

- A block exceeds picture boundary and its size is not larger than min QT size, and min QT size is larger than the min CU size and the max BT/TT depth is equal to 0.

In this contribution, it is proposed to use QT splitting in these two conditions.

Introduction

In VVC draft 7, when a portion of a tree node block exceeds the bottom or right picture boundary, the tree node block is forced to be split until all the samples of every coded block are located inside the picture boundaries. However, there are two cases that none of splitting is allowed for a block located at picture boundary and contained samples exceeded the picture boundary. More specifically, all the variables allowSplitQt, allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, and allowSplitTtHor are set equal to false in these two cases.

In the first case, when both the CTU size and min QT size are set to 128, the aforementioned variables are set equal to false for a CTU block exceeding the picture boundary. According to VVC draft 7 vE, the allowSplitQt is set equal to false due to the following restrictions in section 6.4.1:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize / SubWidthC is less than or equal to MinQtSizeC

The allowSplitBtVer and allowSplitBtHor are set equal to false due to the following restrictions in section 6.4.2:

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_VER

- y0 + cbHeight is greater than pic_height_in_luma_samples

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_VER

- cbHeight is greater than 64

- x0 + cbWidth is greater than pic_width_in_luma_samples

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_HOR

- cbWidth is greater than 64

- y0 + cbHeight is greater than pic_height_in_luma_samples

- Otherwise, if all of the following conditions are true, allowBtSplit is set equal to FALSE

- btSplit is equal to SPLIT_BT_HOR

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is less than or equal to pic_height_in_luma_samples

The allowSplitTtVer and allowSplitTtHor are set equal to fasle due to the following restrictions in section 6.4.3:

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

When all the variables (i.e allowSplitQt, allowSplitBtVer, allowSplitBtHor, allowSplitTtVer, and allowSplitTtHor) are equal to false, the block is split using the TT vertical since split_cu_flag is inferred to be 1, split_qt_flag is inferred to be 0, mtt_split_cu_vertical_flag is inferred to be 1 and mtt_split_cu_binary_flag is inferred to be 0. The TT vertical splitting violates the VPDU constraints that a block whose width or height is larger than 64 cannot used the TT splitting.

In the second case, when min QT size is larger than the min CU size and the max BT/TT depth is equal to 0, the aforementioned variables are set equal to false for a block whose width or height is smaller than or equal to min QT size even though the block exceeds the picture boundary according to the following restrictions in VVC draft 7 vE:

- If one or more of the following conditions are true, allowSplitQt is set equal to FALSE:

- treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA and cbSize is less than or equal to MinQtSizeY

- treeType is equal to DUAL_TREE_CHROMA and cbSize / SubWidthC is less than or equal to MinQtSizeC

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- mttDepth is greater than or equal to maxMttDepth

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- mttDepth is greater than or equal to maxMttDepth

Proposed methods

To solve the aforementioned issues, it is proposed to use QT splitting for a block that exceeds picture boundary when all the variables (allowSplitQt, allowSplitBtHor, allowSplitBtVer, allowSplitTtHor and allowSplitTtVer) are equal to false. The changes to VVC draft 7 vE are highlighted in yellow.

split_qt_flag specifies whether a coding unit is split into coding units with half horizontal and vertical size.

When split_qt_flag is not present, the following applies:

- If all of the following conditions are true, split_qt_flag is inferred to be equal to 1:

  - split_cu_flag is equal to 1

  - allowSplitQt, allowSplitBtHor, allowSplitBtVer, allowSplitTtHor and allowSplitTtVer are eqaul to FALSE.

- Otherwise, iIf allowSplitQt is equal to TRUE, the value of split_qt_flag is inferred to be equal to 1.

- Otherwise, the value of split_qt_flag is inferred to be equal to 0.

Conclusion

This contribution proposes a simple solution to fix the block partitioning at picture boundary. It is suggested to adopt this contribution to the next version of VVC specification.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).