| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0332r2 |
| --- | --- |

| Title: | AhG12: On the slice number signaling dependent on subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal would like to replace the signaling of the number of slices in picture with the signaling of the delta between the number of subpictures in picture and the number of slices in picture. The evaluation of the bit comparison is also provided when enforcing pps_num_subpics_minus1 to be signaled as proposed in JVET-Q0333, Q0334 and Q0335.

# Problem Statement

The syntax element num_slices_in_pic_minus1 is signaled in PPS to derive the rectangular slices. It is defined as follows:

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0.

The number of subpictures in picture shall be smaller than or equal to the number of slices in picture, i.e., sps_num_subpics_minus1 <= num_slices_in_pic_minus1. This disallows a slice contains multiple subpictures. Since the number of subpicture could be up to 255. In such case, it would cost at least 15 bits to signal the value of num_slices_in_pic_minus1. This proposal would like to improve the design by signaling the delta between the number of subpictures in picture and the number of slices in picture.

# Proposed changes based on sps_num_subpics_minus1

The syntax element num_slices_in_pic_minus_num_subpics is proposed to replace num_slices_in_pic_minus1. The current VVC specification is modified as follows (the yellow mark shows the changes compared to the current VVC specification):

| if( rect_slice_flag ) |  |
| --- | --- |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus_num_subpics | ue(v) |

num_slices_in_pic_minus_num_subpics plus ( sps_num_subpics_minus1 + 1 ) specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus_num_subpics shall be in the range of 0 to MaxSlicesPerPicture – ( sps_num_subpics_minus1 + 1 ), inclusive, where MaxSlicesPerPicture is specified in Annex A.

NumSlicesInPic = num_slices_in_pic_minus_num_subpics + sps_num_subpics_minus1 + 1

There are two inferences:

- When single_slice_per_subpic_flag is equal to 1, num_slices_in_pic_minus_num_subpics is inferred to be equal to 0.

- When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus_num_subpics is inferred to be equal to 0.

# Proposed changes based on pps_num_subpics_minus1

It is proposed to enforce PPS to signal pps_num_subpics_minus1 (with no condition on pps_subpics_id_signalling_present_flag). This is as proposed in JVET-Q0333, Q0334, and Q0335. The syntax element num_slices_in_pic_minus_num_subpics is proposed to replace num_slices_in_pic_minus1. The current VVC specification is modified as follows (the yellow mark shows the changes compared to the current VVC specification):

| pps_num_subpics_minus1 | ue(v) |
| --- | --- |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| … |  |
| } |  |
| … |  |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus_num_subpics | ue(v) |

num_slices_in_pic_minus_num_subpics plus ( pps_num_subpics_minus1 + 1 ) specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus_num_subpics shall be in the range of 0 to MaxSlicesPerPicture – ( pps_num_subpics_minus1 + 1 ), inclusive, where MaxSlicesPerPicture is specified in Annex A.

NumSlicesInPic = num_slices_in_pic_minus_num_subpics + pps_num_subpics_minus1 + 1

There are two inferences:

- When single_slice_per_subpic_flag is equal to 1, num_slices_in_pic_minus_num_subpics is inferred to be equal to 0.

- When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus_num_subpics is inferred to be equal to 0.

# Bit comparison

In Q0333, Q0334, and Q0335, pps_num_subpics_minus1 is signaled without the condition of the pps_subpics_id_signalling_present_flag. This is used to evaluate the bit comparison of the proposed method with the current VVC 7 as follows:

VVC 7: >=30 bits

- pps_num_subpics_minus1

  - Condition on pps_subpics_id_signalling_present_flag

  - The range of the values: 0 ~ 255, inclusive

  - 15 bits in worst case

- num_slices_in_pic_minus1

  - >= 15 bits

Suggested change (combined with Q0333/334/335): >=16 bits

- pps_num_subpics_minus1

  - Condition on pps_subpics_id_signalling_present_flag

  - The range of the values: 0 ~ 255, inclusive

  - 15 bits in worst case

- num_slices_in_pic_minus1

  - >= 1 bits

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).