| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0333 |
| --- | --- |

| Title: | AhG12: On the subpicture-based scaling process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal would like to support a subpicture-based scaling process to allow the current subpicture to refer to a reference subpicture using the same subpicture ID if the current subpicture is treated as a picture. The modifications include:

- Each subpicture signals a scaling window flag. If the scaling window flag is enabled for the i-th subpicture, a scaling window is cropped from the i-th subpicture.

  - The parameters of subpicture scaling window are signaled in both PPS and PH. The scaling window for the i-th subpicture derived from PPS can be overridden by the scaling window derived from PH.

- The scaling ratio is derived from the scaling windows cropped from the reference subpicture and the current subpicture, both of which uses the same subpicture ID.

# Problem Statement

In current VVC, the subpicture ID may change within a CLVS. A video source may stream in a specific subpicture using the same subpicture ID. As shown in Figure 1, 3 separate source contents may stream in 3 respective subpictures. If a current subpicture is treated as a picture, the current subpicture would treat the other subpictures as not existed. In such case, the current subpicture should refer to a reference subpicture using the same subpicture ID, instead of the current VVC design which refers to a reference subpicture using a different subpicture ID. Besides, there is no reference subpicture scaling process in the current VVC design to scale a reference subpicture to a current subpicture if the subpictures are in different sizes. This proposal investigates several aspects, such as scaling windows, derivation of scaling ratios, constraints on the scaling ratios, and integer positions for interpolation filtering, to handle the subpicture scaling ratios process when the subpicture ID changes within a CLVS. Note that in this proposal the reference picture resampling (RPR) is disabled, i.e., ref_pic_resampling_enabled_flag is set equal to 0, if the subpicture is enabled.

Figure 1. The subpicture sizes on the same subpicture ID between the reference picture and the current picture could be different, e.g. subpicture ID 1 and subpicture ID 2, or could be identical, e.g. subpicture ID 3.

# Subpicture scaling windows

The scaling window is used to avoid the location misalignment in the reference picture scaling process when the reference picture size and the current picture size are different. The picture scaling ratios are derived from the scaling window directly, and the formula is extracted from VVC WD clause 8.3.2 as follows:

RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL  >>  1 ) ) / PicOutputWidthL
RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL  >>  1 ) ) / PicOutputHeightL

where fRefWidth and fRefHeight are set equal to PicOutputWidthL and PicOutputHeightL of the reference picture RefPicList[ i ][ j ] in luma samples, respectively. PicOutputWidthL and PicOutputHeightL are the scaling window width and height as shown in Figure 2, and are cropped from the picture width and height as shown below:

PicOutputWidthL = pic_width_in_luma_samples − ( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples − ( scaling_win_bottom_offset + scaling_win_top_offset )

Current picture

Reference picture

Reference scaling window

Current scaling window

Figure 2. The scaling windows inside the reference picture and the current picture

PicOutputWidthL[ SubPicIdx ]

Current scaling

window

Reference scaling

window

fRefWidth

fRefHeight

PicOutputHeightL[ SubPicIdx ]

Figure 3. The subpicture scaling window with fRefWidth and fRefHeight for the reference subpicture 1, and the subpicture scaling window with PicOutputWidthL[ 1 ] and PicOutputHeightL[ 1 ] for the current subpicture. Note that RefSubPicIdx and SubPicIdx are subpicture layout indexes, and are respectively equal to 0 and 1 for subpicture using ID = 1

To apply the same picture scaling process to the subpictures using the same ID, the scaling window should be enabled as well for the current subpicture if the current subpicture is treated as a picture. Therefore, it is proposed to do the following modifications on top of the current VVC:

- Each subpicture signals a scaling window flag, e.g. pps_scaling_window_flag[ i ]. If the i-th flag pps_scaling_window_flag[ i ] is equal to 1, the scaling window offsets are signaled for the i-th subpicture. The scaling window for the i-th subpicture are cropped from the i-th subpicture as shown in Figure 3.

- The number of subpictures in the CLVS shall be first signaled to loop a group of the scaling window offsets.

- Subpicture scaling window parameters are signaled in both PPS and PH. The scaling window for the i-th subpicture derived from PPS can be overridden by the scaling window derived from PH.

- The value of pps_scaling_window_flag[ i ] shall be inferred to be 0 to avoid unexpected activation of the picture scaling process if at least one of the following conditions is true

  - subpicture functionality is enabled and the i-th subpicture is not treated as a picture

  - RPR functionality is disabled and subpicture ID list is not reordered in the CLVS

  - RPR functionality and subpicture functionality are both disabled

  - RPR functionality and subpicture functionality are both enabled at the same time

The proposed syntax change in VVC Draft 7 version 14 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 7 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. pic_width_in_luma_samples; ue(v). pic_height_in_luma_samples; ue(v). …. pps_num_subpics_minus1; ue(v). for( i = 0; i <= pps_num_subpics_minus1; i++ ) {. pps_scaling_window_flag[ i ]; u(1). if( pps_scaling_window_flag[ i ] ) {. pps_scaling_win_left_offset[ i ]; ue(v). pps_scaling_win_right_offset[ i ]; ue(v). pps_scaling_win_top_offset[ i ]; ue(v). pps_scaling_win_bottom_offset[ i ]; ue(v). }. }. …. pps_subpic_id_signalling_present_flag; u(1). if( pps_subpics_id_signalling_present_flag ) {. pps_num_subpics_minus1; ue(v). pps_subpic_id_len_minus1; ue(v). for( i = 0; i <= pps_num_subpic_minus1; i++ ). pps_subpic_id[ i ]; u(v). }. …. Section 7.4.3.4 in VVC Draft 7 version 14: pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples. When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples. pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples. When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples. pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS. It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to sps_num_subpics_minus1. pps_scaling_window_flag[ i ] equal to 1 specifies that the scaling window offset parameters for the i-th subpicture are present in the PPS. pps_scaling_window_flag[ i ] equal to 0 specifies that the scaling window offset parameters are not present for the i-th subpicture in the PPS. When ref_pic_resampling_enabled_flag is equal to 0, the value of scaling_window_flag shall be equal to 0. The value of pps_scaling_window_flag[ i ] shall be inferred to be 0 if one of the following conditions is true: - ref_pic_resampling_enabled_flag is equal to 0 and subpics_present_flag is equal to 0 - ref_pic_resampling_enabled_flag is equal to 0 and sps_subpic_id_present_flag is equal to 0 - ref_pic_resampling_enabled_flag is equal to 0 and sps_subpic_id_signalling_present_flag is equal to 1 - subpics_present_flag is equal to 1 and subpic_treated_as_pic_flag[ i ] is equal to 0 - ref_pic_resampling_enabled_flag is equal to 1 and subpics_present_flag is equal to 1 pps_scaling_win_left_offset[ i ], pps_scaling_win_right_offset[ i ], pps_scaling_win_top_offset[ i ], and pps_scaling_win_bottom_offset[ i ] specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation of the i-th subpicture. When pps_scaling_window_flag[ i ] is equal to 0, the values of pps_scaling_win_left_offset[ i ], pps_scaling_win_right_offset[ i ], pps_scaling_win_top_offset[ i ], and pps_scaling_win_bottom_offset[ i ] are inferred to be equal to 0. The width and height of the i-th subpicture is derived as follows: subpic_width_in_luma_samples[ i ] = Min( pic_width_in_luma_samples, ( subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1 ) * CtbSizeY ) – subpic_ctu_top_left_x[ i ] * CtbSizeY subpic_height_in_luma_samples[ i ] = Min( pic_height_in_luma_samples, ( subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) * CtbSizeY ) – subpic_ctu_top_left_y[ i ] * CtbSizeY The value of pps_scaling_win_left_offset[ i ] + pps_scaling_win_right_offset[ i ] shall be less than pic_width_in_luma_samples subpic_width_in_luma_samples[ i ], and the value of pps_scaling_win_top_offset[ i ] + pps_scaling_win_bottom_offset[ i ] shall be less than pic_height_in_luma_samples subpic_height_in_luma_samples[ i ]. Section 7.3.2.6 in VVC Draft 7 version 14: Picture_header_rbsp( ) {; Descriptor. …. If( (!ref_pic_resampling_enabled_flag && pps_subpic_id_signalling_present_flag && subpic_treated_as_pic_flag[ i ] ) ¦¦ (ref_pic_resampling_enabled_flag && !subpics_present_flag ) ) {. for( i = 0; i <= sps_num_subpics_minus1; i++ ) {. ph_scaling_window_flag[ i ]; u(1). if( ph_scaling_window_flag[ i ] ) {. ph_scaling_win_left_offset[ i ]; ue(v). ph_scaling_win_right_offset[ i ]; ue(v). ph_scaling_win_top_offset[ i ]; ue(v). ph_scaling_win_bottom_offset[ i ]; ue(v). }. }. }. …. Section 7.4.3.6 in VVC Draft 7 version 14: ph_scaling_window_flag[ i ] equal to 1 specifies that the scaling window offset parameters for the i-th subpicture are present in the PH. ph_scaling_window_flag[ i ] equal to 0 specifies that the scaling window offset parameters for the i-th subpicture are not present in the PH. If not present, the value of ph_scaling_window_flag[ i ] shall be inferred to be 0. ph_scaling_win_left_offset[ i ], ph_scaling_win_right_offset[ i ], ph_scaling_win_top_offset[ i ], and ph_scaling_win_bottom_offset[ i ] specify the offsets, in units of luma samples, that are applied to the picture size of the i-th subpicture for scaling ratio calculation. When ph_scaling_window_flag[ i ] is equal to 0, the values of ph_scaling_win_left_offset[ i ], ph_scaling_win_right_offset[ i ], ph_scaling_win_top_offset[ i ], and ph_scaling_win_bottom_offset[ i ] are inferred to be equal to 0. The value of ph_scaling_win_left_offset[ i ] + ph_scaling_win_right_offset[ i ] shall be less than subpic_width_in_luma_samples[ i ], and the value of ph_scaling_win_top_offset[ i ] + ph_scaling_win_bottom_offset[ i ] shall be less than subpic_height_in_luma_samples[ i ]. The variables scaling_win_left_offset[ i ], scaling_win_right_offset[ i ], scaling_win_top_offset[ i ], and scaling_win_bottom_offset[ i ] are derived as follows: scaling_win_left_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_left_offset[ i ] : pps_scaling_win_left_offset[ i ] scaling_win_right_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_right_offset[ i ] : pps_scaling_win_ right_offset[ i ] scaling_win_top_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_ top_offset[ i ] : pps_scaling_win_ top_offset[ i ] scaling_win_bottom_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_bottom_offset[ i ] : pps_scaling_win_ bottom_offset[ i ] The variables PicOutputWidthL[ i ] and PicOutputHeightL[ i ] are derived as follows: PicOutputWidthL[ i ] = subpic_weight_in_luma_samples[ i ] − (scaling_win_right_offset[ i ] + scaling_win_left_offset[ i ] ) PicOutputHeightL[ i ] = subpic_height_in_luma_samples[ i ] − (scaling_win_bottom_offset[ i ] + scaling_win_top_offset[ i ] )

# Subpicture scaling ratios

The same derivation of the current design for the regular picture scaling ratios can be shared to derive the subpicture scaling ratios by simply adding the subpicture index to the original design. The proposed changes with yellow change marks compared to the current VVC specification are shown as follows:

RefSubpicIdList[ k ] is set equal to the subpicture ID list SubpicIdList[ k ] derived in the reference picture RefPicList[ i ][ j ], and RefSubPicIdx is derived to be such that RefSubpicIdList[ RefSubPicIdx ] is equal to slice_subpic_id of the current picture

fRefWidth is set equal to PicOutputWidthL[ RefSubPicIdx ] of the reference picture RefPicList[ i ][ j ] in luma samples
fRefHeight is set equal to PicOutputHeightL[ RefSubPicIdx ] of the reference picture RefPicList[ i ][ j ] in luma samples

RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL[ SubPicIdx ]  >>  1 ) ) / PicOutputWidthL[ SubPicIdx ]
RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL[ SubPicIdx ]  >>  1 ) ) / PicOutputHeightL[ SubPicIdx ]

Note that SubPicIdx is derived to be such that SubpicIdList[ SubPicIdx ] of the current picture is equal to slice_subpic_id of the current slice

# Constraints on the subpicture scaling ratios

The same issue in this section is studied in JVET-Q0331.

In current VVC specification, the range of the reference picture scaling ratios shall be in the range of 1/8 to 2, inclusive, and there are bitstream conformance to apply this range to the picture widths and heights between the reference and current pictures as follows:

Let refPicWidthInLumaSamples and refPicHeightInLumaSamples be the pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of a reference picture of a current picture referring to this PPS. There is a requirement of bitstream conformance that all of the following conditions are satisfied:

–	pic_width_in_luma_samples * 2 shall be greater than or equal to refPicWidthInLumaSamples.

–	pic_height_in_luma_samples * 2 shall be greater than or equal to refPicHeightInLumaSamples.

–	pic_width_in_luma_samples shall be less than or equal to refPicWidthInLumaSamples * 8.

–	pic_height_in_luma_samples shall be less than or equal to refPicHeightInLumaSamples * 8.

Because a subpicture scaling ratio is derived by the scaling window of a subpicture, i.e., PicOutputWidthL[ m ] and PicOutputHeightL[ n ], the scaling ratios could be outside the range if there is no restriction to PicOutputWidthL[ m ] and PicOutputHeightL[ m ]. Therefore, it is proposed to constrain the refPicOutputWidthL / PicOutputWidthL and refPicOutputHeightL / PicOutputHeightL to be in the range of 1/8 to 2, inclusive, as follows:

Let RefSubpicIdList[ i ] set equal to the subpicture ID list SubpicIdList[ i ] derived in the slice header referring to a reference picture of a current picture. RefSubPicIdx is derived to be such that RefSubpicIdList[ RefSubPicIdx ] is equal to slice_subpic_id of the current slice header referring to the current picture.

Let refPicOutputWidthL[ i ] and refPicOutputHeightL[ i ] are the PicOutputWidthL[ i ] and PicOutputHeightL[ i ] of the reference picture.

It is required that the bitstream conformance shall satisfy all of the following conditions:

–	PicOutputWidthL[ SubPicIdx ] * 2 shall be greater than or equal to refPicOutputWidthL[ RefSubPicIdx ].

–	PicOutputHeightL[ SubPicIdx ] * 2 shall be greater than or equal to refPicOutputHeightL[ RefSubPicIdx ].

–	PicOutputWidthL[ SubPicIdx ] shall be less than or equal to refPicOutputWidthL[ RefSubPicIdx ] * 8.

–	PicOutputHeightL[ SubPicIdx ] shall be less than or equal to refPicOutputHeightL[ RefSubPicIdx ] * 8.

Scaling

window

Scaling

window

Figure 4. The reference block pointed out by a motion vector from the current coding block

# Integer positions derivation for interpolation filtering

If a current subpicture is treated as a picture, the coding block in the current subpicture ID 1 should refer to the block in the reference subpicture ID 1 to get the same source content for prediction as shown in Figure 4. However, the current VVC specification only can support the coding block in the current subpicture ID 1 referring to the block in the reference subpicture ID 2.

The same issue in this section is studied in JVET-Q0157. This proposal would like to further add the subpicture indexes to the scaling window offsets. The proposed syntax change are as described in the following table with yellow change mark compared to JVET-Q0157:

The variables leftBoundaryOffset is set equal to scaling_win_left_offset[ SubPicIdx ], topBoundaryOffset is set equal to scaling_win_top_offset[ SubPicIdx ], refSubPicLeftBoundaryPos, and refSubPicTopBoundaryPos are set equal to 0. If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies leftBoundaryOffset += SubPicLeftBoundaryPos topBoundaryOffset += SubPicTopBoundaryPos RefSubPicIdx is derived to be such that SubpicIdList[ RefSubPicIdx ] of the reference picutre is equal to slice_subpic_id. refSubPicLeftBoundaryPos = subpic_ctu_top_left_x[ RefSubPicIdx ] * CtbSizeY refSubPicTopBoundaryPos = subpic_ctu_top_left_y[ RefSubPicIdx ] *CtbSizeY The variable fRefLeftOffset is set equal to (( scaling_win_left_offset[ RefSubPicIdx ] + refSubPicLeftBoundaryPos) << 10 ), where scaling_win_left_offset[ i ] is the scaling_win_left_offset[ i ] for the reference picture. The variable fRefTopOffset is set equal to (( scaling_win_top_offset[ RefSubPicIdx ] + refSubPicTopBoundaryPos) << 10 ), where scaling_win_top_offset[ i ] is the scaling_win_top_offset[ i ] for the reference picture. … - If cIdx is equal to 0, the following applies: - For each luma sample location ( xL = 0..sbWidth − 1 + brdExtSize, yL = 0..sbHeight − 1 + brdExtSize ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows: - Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows: refxSbL = ( ( ( xSb − leftBoundaryOffset ) << 4 ) + refMvLX[ 0 ] ) * scalingRatio[ 0 ] (937) refxL = ( ( Sign( refxSbL ) * ( ( Abs( refxSbL ) + 128 ) >> 8 ) + xL * ( ( scalingRatio[ 0 ] + 8 ) >> 4 ) ) + fRefLeftOffset + 32 ) >> 6 (938) refySbL = ( ( ( ySb − topBoundaryOffset ) << 4 ) + refMvLX[ 1 ] ) * scalingRatio[ 1 ] (939) refyL = ( ( Sign( refySbL ) * ( ( Abs( refySbL ) + 128 ) >> 8 ) + yL * ( ( scalingRatio[ 1 ] + 8 ) >> 4 ) ) + fRefTopOffset + 32 ) >> 6 (940)

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).