| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0334 |
| --- | --- |

| Title: | AhG8/AhG12: On the reference picture resampling for the subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 845 1779 yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal would like to investigate the use case when both RPR and the subpicture are enabled at the same time. The proposals include:

- The proposed design requires to maintain two types of the picture scaling processes: one uses the scaling window and the scaling ratio that are currently in VVC specification for the regular pictures, and the other one uses the subpicture scaling window and the subpicture scaling ratio for the subpictures when the subpicture is treated as a picture.

- The subpicture scaling window for the i-th subpicture is enabled by a scaling window flag. If enabled, a scaling window is cropped from the i-th subpicture.

  - The parameters of subpicture scaling window are signaled in both PPS and PH. The scaling window for the i-th subpicture derived from PPS can be overridden by the scaling window derived from PH.

- The subpicture scaling ratio is derived from the subpicture scaling windows of the reference subpicture and the current subpicture, i.e., refPicOutputWidthL[ RefSubPicIdx ] / PicOutputWidthL[ SubPicIdx ] and refPicOutputHeightL[ RefSubPicIdx ] / PicOutputHeightL[ SubPicIdx ]

This proposal develops the RPR for subpictures with the proposal in JVET-Q0xxx, in which allows the reference subpicture scaling process for a current subpicture to scale up or scale down a reference subpicture using the same subpicture ID. The specification changes of this proposal with JVET-Q0333 and without JVET-Q0333 are both appended with this contribution.

Figure 1. Adaptive streaming in an open GOP structure (extracted from JVET-M0259)

# Problem Statement

The reference picture resampling (RPR) was adopted into VVC standard to support an adaptive streaming in an open group of picture (GOP) prediction structure (starting with CRA picture) as shown in Figure 1. Another subpicture functionality was also adopted into VVC standard to support the extraction and merging of subpictures in the use cases of 360 streaming, video conferencing, etc. The current VVC standard only enables RPR in a regular picture. If the subpicture functionality is working, the RPR functionality is disabled. This proposal would like to investigate the use case when both RPR and the subpicture are enabled at the same time. This allows an adaptive subpicture streaming in an open GOP prediction structure, and shares the same benefits as those of RPR in the regular picture use cases that get a better compression performance and better subjectively visual quality compared to a traditionally closed GOP prediction structure. Figure 2 describes the cases of RPR and subpicture enabled at the same time that this proposal investigates.

1

2

3

2

1

3

Reference picture

Current picture

Figure 2. RPR and subpicture are both enabled at the same time

This proposal develops the RPR for subpictures with the proposal in JVET-Q0333, in which allows the reference subpicture scaling process for a current subpicture to scale up or scale down a reference subpicture using the same subpicture ID as shown in Figure 3. The proposed design requires to maintain two categories of scaling processes: one uses the scaling window and the scaling ratio for the regular pictures, and another uses the subpicture scaling window and the subpicture scaling ratio for the subpictures. The specification changes with JVET-Q0333 and without JVET-Q0333 are appended with this contribution.

Figure 3. The subpicture scaling process to scale up or down the current subpicture to a reference subpicture using the same subpicture ID

# Subpicture scaling windows

Current picture

Reference picture

Reference scaling window

Current scaling window

Figure 4. The scaling windows for the regular pictures

The scaling window is used to avoid the location misalignment in the reference picture scaling process when the reference picture size and the current picture size are different. It is proposed to derive two types of scaling windows: one is the scaling window currently in VVC specification for the regular picture as shown in Figure 4, and another is subpicture scaling windows for the subpictures when the current subpicture is treated as a picture as shown in Figure 5.

PicOutputWidthL[ SubPicIdx ]

Current scaling

window

Reference scaling

window

fRefWidth

fRefHeight

PicOutputHeightL[ SubPicIdx ]

Figure 5. The subpicture scaling window with fRefWidth and fRefHeight for the reference subpicture 1, and the subpicture scaling window with PicOutputWidthL[ 1 ] and PicOutputHeightL[ 1 ] for the current subpicture. Note that RefSubPicIdx and SubPicIdx are subpicture layout indexes, and are respectively equal to 0 and 1 for subpicture using ID = 1

The modifications on top of the current VVC specification for the proposed subpicture scaling windows are as follows:

- Each subpicture signals a scaling window flag, e.g. pps_scaling_window_flag[ i ]. If the i-th flag pps_scaling_window_flag[ i ] is equal to 1, the scaling window offsets are signaled for the i-th subpicture. The scaling window for the i-th subpicture are cropped from the i-th subpicture as shown in Figure 5.

- The number of subpictures in the CLVS shall be first signaled to loop a group of the scaling window offsets.

- Subpicture scaling window parameters are signaled in both PPS and PH. The scaling window for the i-th subpicture derived from PPS can be overridden by the scaling window derived from PH.

- The value of pps_scaling_window_flag[ i ] shall be inferred to be 0 to avoid unexpected activation of the picture scaling process if at least one of the following conditions is true

  - subpicture functionality is enabled and the i-th subpicture is not treated as a picture

  - RPR functionality is disabled and subpicture ID list is not reordered in the CLVS

  - Subpicture functionality is disabled

The proposed syntax change in VVC Draft 7 version 14 are as described in the following table with yellow change mark:

Section 7.3.2.4 in VVC Draft 7 version 14: pic_parameter_set_rbsp( ) {; Descriptor. …. pic_width_in_luma_samples; ue(v). pic_height_in_luma_samples; ue(v). …. scaling_window_flag; u(1). if( scaling_window_flag ) {. scaling_win_left_offset; ue(v). scaling_win_right_offset; ue(v). scaling_win_top_offset; ue(v). scaling_win_bottom_offset; ue(v). }. pps_num_subpics_minus1; ue(v). for( i = 0; i <= pps_num_subpics_minus1; i++ ) {. pps_scaling_window_flag[ i ]; u(1). if( pps_scaling_window_flag[ i ] ) {. pps_scaling_win_left_offset[ i ]; ue(v). pps_scaling_win_right_offset[ i ]; ue(v). pps_scaling_win_top_offset[ i ]; ue(v). pps_scaling_win_bottom_offset[ i ]; ue(v). }. }. …. pps_subpic_id_signalling_present_flag; u(1). if( pps_subpics_id_signalling_present_flag ) {. pps_num_subpics_minus1; ue(v). pps_subpic_id_len_minus1; ue(v). for( i = 0; i <= pps_num_subpic_minus1; i++ ). pps_subpic_id[ i ]; u(v). }. …. Section 7.4.3.4 in VVC Draft 7 version 14: pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples. When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples. pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples. When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples. scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation. When scaling_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0. The value of scaling_win_left_offset + scaling_win_right_offset shall be less than pic_width_in_luma_samples, and the value of scaling_win_top_offset + scaling_win_bottom_offset shall be less than pic_height_in_luma_samples. The variables PicOutputWidthL and PicOutputHeightL are derived as follows: PicOutputWidthL = pic_width_in_luma_samples − (74) ( scaling_win_right_offset + scaling_win_left_offset ) PicOutputHeightL = pic_height_in_luma_samples − (75) ( scaling_win_bottom_offset + scaling_win_top_offset ) pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS. It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to sps_num_subpics_minus1. pps_scaling_window_flag[ i ] equal to 1 specifies that the scaling window offset parameters for the i-th subpicture are present in the PPS. pps_scaling_window_flag[ i ] equal to 0 specifies that the scaling window offset parameters are not present for the i-th subpicture in the PPS. The value of pps_scaling_window_flag[ i ] shall be inferred to be 0 if one of the following conditions is true: - subpics_present_flag is equal to 0 - ref_pic_resampling_enabled_flag is equal to 0 and sps_subpic_id_present_flag is equal to 0 - ref_pic_resampling_enabled_flag is equal to 0 and sps_subpic_id_signalling_present_flag is equal to 1 - subpics_present_flag is equal to 1 and subpic_treated_as_pic_flag[ i ] is equal to 0 pps_scaling_win_left_offset[ i ], pps_scaling_win_right_offset[ i ], pps_scaling_win_top_offset[ i ], and pps_scaling_win_bottom_offset[ i ] specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation of the i-th subpicture. When pps_scaling_window_flag[ i ] is equal to 0, the values of pps_scaling_win_left_offset[ i ], pps_scaling_win_right_offset[ i ], pps_scaling_win_top_offset[ i ], and pps_scaling_win_bottom_offset[ i ] are inferred to be equal to 0. The width and height of the i-th subpicture is derived as follows: subpic_width_in_luma_samples[ i ] = Min( pic_width_in_luma_samples, ( subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1 ) * CtbSizeY ) – subpic_ctu_top_left_x[ i ] * CtbSizeY subpic_height_in_luma_samples[ i ] = Min( pic_height_in_luma_samples, ( subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) * CtbSizeY ) – subpic_ctu_top_left_y[ i ] * CtbSizeY The value of pps_scaling_win_left_offset[ i ] + pps_scaling_win_right_offset[ i ] shall be less than subpic_width_in_luma_samples[ i ], and the value of pps_scaling_win_top_offset[ i ] + pps_scaling_win_bottom_offset[ i ] shall be less than subpic_height_in_luma_samples[ i ]. Section 7.3.2.6 in VVC Draft 7 version 14: Picture_header_rbsp( ) {; Descriptor. …. If( (!ref_pic_resampling_enabled_flag && pps_subpic_id_signalling_present_flag && subpic_treated_as_pic_flag[ i ] ) ¦¦ (ref_pic_resampling_enabled_flag && subpics_present_flag && subpic_treated_as_pic_flag[ i ]) ) {. for( i = 0; i <= sps_num_subpics_minus1; i++ ) {. ph_scaling_window_flag[ i ]; u(1). if( ph_scaling_window_flag[ i ] ) {. ph_scaling_win_left_offset[ i ]; ue(v). ph_scaling_win_right_offset[ i ]; ue(v). ph_scaling_win_top_offset[ i ]; ue(v). ph_scaling_win_bottom_offset[ i ]; ue(v). }. }. }. …. Section 7.4.3.6 in VVC Draft 7 version 14: ph_scaling_window_flag[ i ] equal to 1 specifies that the scaling window offset parameters for the i-th subpicture are present in the PH. ph_scaling_window_flag[ i ] equal to 0 specifies that the scaling window offset parameters for the i-th subpicture are not present in the PH. If not present, the value of ph_scaling_window_flag[ i ] shall be inferred to be 0. ph_scaling_win_left_offset[ i ], ph_scaling_win_right_offset[ i ], ph_scaling_win_top_offset[ i ], and ph_scaling_win_bottom_offset[ i ] specify the offsets, in units of luma samples, that are applied to the picture size of the i-th subpicture for scaling ratio calculation. When ph_scaling_window_flag[ i ] is equal to 0, the values of ph_scaling_win_left_offset[ i ], ph_scaling_win_right_offset[ i ], ph_scaling_win_top_offset[ i ], and ph_scaling_win_bottom_offset[ i ] are inferred to be equal to 0. The value of ph_scaling_win_left_offset[ i ] + ph_scaling_win_right_offset[ i ] shall be less than subpic_width_in_luma_samples[ i ], and the value of ph_scaling_win_top_offset[ i ] + ph_scaling_win_bottom_offset[ i ] shall be less than subpic_height_in_luma_samples[ i ]. The variables scaling_win_left_offset[ i ], scaling_win_right_offset[ i ], scaling_win_top_offset[ i ], and scaling_win_bottom_offset[ i ] are derived as follows: scaling_win_left_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_left_offset[ i ] : pps_scaling_win_left_offset[ i ] scaling_win_right_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_right_offset[ i ] : pps_scaling_win_ right_offset[ i ] scaling_win_top_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_ top_offset[ i ] : pps_scaling_win_ top_offset[ i ] scaling_win_bottom_offset[ i ] = ph_scaling_window_flag[ i ] ? ph_scaling_win_bottom_offset[ i ] : pps_scaling_win_ bottom_offset[ i ] The variables PicOutputWidthL[ i ] and PicOutputHeightL[ i ] are derived as follows: PicOutputWidthL[ i ] = subpic_weight_in_luma_samples[ i ] − (scaling_win_right_offset[ i ] + scaling_win_left_offset[ i ] ) PicOutputHeightL[ i ] = subpic_height_in_luma_samples[ i ] − (scaling_win_bottom_offset[ i ] + scaling_win_top_offset[ i ] )

# Subpicture scaling ratios

It is proposed to maintain two separate scaling ratios, one is the original scaling ratio for the regular pictures as follows:

fRefWidth is set equal to PicOutputWidthL of the reference picture RefPicList[ i ][ j ] in luma samples
fRefHeight is set equal to PicOutputHeightL of the reference picture RefPicList[ i ][ j ] in luma samples
RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL  >>  1 ) ) / PicOutputWidthL
RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL  >>  1 ) ) / PicOutputHeightL

and another is the subpicture scaling ratio for the subpictures when the current subpicture is treated as a picture:

fRefWidth and fRefHeight are defined as PicOutputWidthL[ RefSubPicIdx ] and PicOutputHeightL[ RefSubPicIdx ] of the reference picture RefPicList[ i ][ j ] in luma samples, respectively

RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL[ SubPicIdx ]  >>  1 ) ) / PicOutputWidthL[ SubPicIdx ]
RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL[ SubPicIdx ]  >>  1 ) ) / PicOutputHeightL[ SubPicIdx ]

- SubPicIdx is derived to be such that SubpicIdList[ SubPicIdx ] of the current picture is equal to slice_subpic_id of the current slice.

- RefSubPicIdx is derived to be such that SubpicIdList[ RefSubPicIdx ] of the reference picture RefPicList[ i ][ j ] is equal to slice_subpic_id of the current slice.

# Constraints on the subpicture scaling ratios

The same issue in this section is studied in JVET-Q0331 and JVET-Q0333.

In current VVC specification, the range of the picture scaling ratios shall be in the range of 1/8 to 2, inclusive, and there are bitstream conformance to apply this range to the picture widths and heights between the reference and current pictures as follows:

Let refPicWidthInLumaSamples and refPicHeightInLumaSamples be the pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of a reference picture of a current picture referring to this PPS. There is a requirement of bitstream conformance that all of the following conditions are satisfied:

–	pic_width_in_luma_samples * 2 shall be greater than or equal to refPicWidthInLumaSamples.

–	pic_height_in_luma_samples * 2 shall be greater than or equal to refPicHeightInLumaSamples.

–	pic_width_in_luma_samples shall be less than or equal to refPicWidthInLumaSamples * 8.

–	pic_height_in_luma_samples shall be less than or equal to refPicHeightInLumaSamples * 8.

Because the picture scaling ratios are derived by the scaling window widths and heights, i.e., PicOutputWidthL and PicOutputHeightL, the scaling ratios could be outside the range if there is no restriction to PicOutputWidthL and PicOutputHeightL. Therefore, it is proposed to constrain the refPicOutputWidthL / PicOutputWidthL and refPicOutputHeightL / PicOutputHeightL to be in the range of 1/8 to 2, inclusive, as follows:

Let refPicOutputWidthL and refPicOutputHeightL be the PicOutputWidthL  and PicOutputHeightL, respectively, of a reference picture of a current picture referring to this PPS. There is a requirement of bitstream conformance that all of the following conditions are satisfied:

–	PicOutputWidthL * 2 shall be greater than or equal to refPicOutputWidthL .

–	PicOutputHeightL * 2 shall be greater than or equal to refPicOutputHeightL.

–	PicOutputWidthL shall be less than or equal to refPicOutputWidthL  * 8.

–	PicOutputHeightL shall be less than or equal to refPicOutputHeightL * 8.

There is the same issue that the subpicture scaling ratios could be outside the range if there is no restriction to the subpicture scaling window, i.e., PicOutputWidthL[ m ] and PicOutputHeightL[ m ]. Therefore, it is proposed to constrain the subpicture scaling ratio, i.e., refPicOutputWidthL[ RefSubPicIdx ] / PicOutputWidthL[ SubPicIdx ] and refPicOutputHeightL[ RefSubPicIdx ] / PicOutputHeightL[ SubPicIdx ], to be in the range of 1/8 to 2, inclusive:

Let RefSubpicIdList[ i ] set equal to the subpicture ID list SubpicIdList[ i ] derived in the slice header referring to a reference picture of a current picture.

RefSubPicIdx is derived to be such that RefSubpicIdList[ RefSubPicIdx ] is equal to slice_subpic_id of the current slice header referring to the current picture.

Let refPicOutputWidthL[ i ] and refPicOutputHeightL[ i ] are the PicOutputWidthL[ i ] and PicOutputHeightL[ i ] of the reference picture.

It is required that the bitstream conformance shall satisfy all of the following conditions:

–	PicOutputWidthL[ SubPicIdx ] * 2 shall be greater than or equal to refPicOutputWidthL[ RefSubPicIdx ].

–	PicOutputHeightL[ SubPicIdx ] * 2 shall be greater than or equal to refPicOutputHeightL[ RefSubPicIdx ].

–	PicOutputWidthL[ SubPicIdx ] shall be less than or equal to refPicOutputWidthL[ RefSubPicIdx ] * 8.

–	PicOutputHeightL[ SubPicIdx ] shall be less than or equal to refPicOutputHeightL[ RefSubPicIdx ] * 8.

Scaling

window

Scaling

window

Figure 6. The reference block pointed out by a motion vector from the current coding block

# Integer positions derivation for interpolation filtering

The same issue is studied in JVET-Q0157 and JVET-Q0333.

If a current subpicture is treated as a picture, the coding block in the current subpicture ID 1 should refer to the block in the reference subpicture ID 1 to get the same source content for prediction as shown in Figure 6. However, the current VVC specification only can support the coding block in the current subpicture ID 1 referring to the block in the reference subpicture ID 2.

The same solution provided in JVET-Q0333 should be applied directly to the proposed design. The proposed syntax change are as described in the following table with yellow change mark compared to JVET-Q0157:

The variables leftBoundaryOffset is set equal to scaling_win_left_offset[ SubPicIdx ], topBoundaryOffset is set equal to scaling_win_top_offset[ SubPicIdx ], refSubPicLeftBoundaryPos, and refSubPicTopBoundaryPos are set equal to 0. If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies leftBoundaryOffset += SubPicLeftBoundaryPos topBoundaryOffset += SubPicTopBoundaryPos RefSubPicIdx is derived to be such that SubpicIdList[ RefSubPicIdx ] of the reference picutre is equal to slice_subpic_id. refSubPicLeftBoundaryPos = subpic_ctu_top_left_x[ RefSubPicIdx ] * CtbSizeY refSubPicTopBoundaryPos = subpic_ctu_top_left_y[ RefSubPicIdx ] *CtbSizeY The variable fRefLeftOffset is set equal to (( scaling_win_left_offset[ RefSubPicIdx ] + refSubPicLeftBoundaryPos) << 10 ), where scaling_win_left_offset[ i ] is the scaling_win_left_offset[ i ] for the reference picture. The variable fRefTopOffset is set equal to (( scaling_win_top_offset[ RefSubPicIdx ] + refSubPicTopBoundaryPos) << 10 ), where scaling_win_top_offset[ i ] is the scaling_win_top_offset[ i ] for the reference picture. … - If cIdx is equal to 0, the following applies: - For each luma sample location ( xL = 0..sbWidth − 1 + brdExtSize, yL = 0..sbHeight − 1 + brdExtSize ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows: - Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows: refxSbL = ( ( ( xSb − leftBoundaryOffset ) << 4 ) + refMvLX[ 0 ] ) * scalingRatio[ 0 ] (937) refxL = ( ( Sign( refxSbL ) * ( ( Abs( refxSbL ) + 128 ) >> 8 ) + xL * ( ( scalingRatio[ 0 ] + 8 ) >> 4 ) ) + fRefLeftOffset + 32 ) >> 6 (938) refySbL = ( ( ( ySb − topBoundaryOffset ) << 4 ) + refMvLX[ 1 ] ) * scalingRatio[ 1 ] (939) refyL = ( ( Sign( refySbL ) * ( ( Abs( refySbL ) + 128 ) >> 8 ) + yL * ( ( scalingRatio[ 1 ] + 8 ) >> 4 ) ) + fRefTopOffset + 32 ) >> 6 (940)

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).