| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0337 |
| --- | --- |

| Title: | Non-CE4: On weight values of the chroma-component for TPM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current TPM mode, edge between two blocks is blended with derived weight values. The weight values of two direction are symmetric for luma-component but not for chroma-component. Specifically, for chroma-component, samples on edge may have different weight values according to the given direction. In this contribution, in order to align the chroma weight values for two direction to use the equal weight value for samples on edge, weight values of the chroma-component are derived with proposed shifted-sub-sampling method. The experimental results reportedly show 0.01%, -0.02% BD-rate changes with no encoding and decoding run-time changes compared to VTM7.0 as anchor in RA, LB configurations.

Introduction

In current VVC draft, TPM mode has two directions, which are indicated by triangleDir. In TPM mode, edge between two blocks is blended with weight values and the weight values of two direction are symmetric shown in Figure 1. Figure 1(a), Figure 1(b) and Figure 1(c) specifically show the weight values of the luma-component for 8x8, 16x8, and 8x16 block as examples according to the triangle directions, respectively.

Figure 1 weight values of the luma-component for each block and for each direction

For luma-component, samples on edge are blended with equal weight values but, it isn’t for chroma-component when triangle direction is equal to 1. Also, the blending weights of each direction are not symmetric because the weight values of the chroma samples are sub-sampled from those of luma-component. Figure 2(a), Figure 2(b) and Figure 2(c) show the weight values of the chroma-component for 4x4, 8x4, and 4x8 block as examples according to the triangle direction, respectively.

Figure 2 weight values of the chroma-component for each block and for each direction

Proposed method

In order to align the chroma weight values for two direction and to use the equal weight value for samples on edge, weight values of the chroma-component are derived with proposed shifted-sub-sampling method. Figure 3 shows the example of the shifted-sub-sampling when triangle direction is equal to 1. This x-axis shift is affected to 4:2:0 and 4:2:2 chroma format.

Figure 3 Example of the shifted-sub-sampling

Proposed spec changes

In the VVC WD 7.0 [2], the newly added texts are highlighted and removed texts are marked as stricken-red-font text.

- 8.5.7.2 Weighted sample prediction process for triangle merge mode

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbWY, nCbHY and nCbR is derived as follows:

nCbWY = ( cIdx = = 0 )  ?  nCbW  :  nCbW * SubWidthC	(1027)

nCbHY = ( cIdx = = 0 )  ?  nCbH  :  nCbH * SubHeightC	(1028)

nCbR = ( nCbWY > nCbHY )  ?  ( nCbWY / nCbHY )  :  ( nCbHY / nCbWY )	(1029)

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − BitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

Depending on the values of triangleDir and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variables offset, xL and yL are derived as follows:

offset = (triangleDir == 0 || subWidthC == 1) ? 0 : 1

xL = ( cIdx = = 0 ) ? x : x * SubWidthC + offset

yL = ( cIdx = = 0 ) ? y : y * SubHeightC

–	The variable wValue specifying the weight of the prediction sample is derived as follows:

- If triangleDir is equal to 0, the following applies:

wValue = ( nCbWY > nCbHY ) ? ( Clip3( 0, 8, ( xL / nCbR − yL ) + 4 ) )	(1030)
	: ( Clip3( 0, 8, ( xL − yL / nCbR ) + 4 ) )

- Otherwise (triangleDir is equal to 1), the following applies:

wValue = ( nCbWY > nCbHY ) ? ( Clip3( 0, 8, ( nCbHY − 1 − xL / nCbR − yL ) + 4 ) ) 	(1031)
		( Clip3( 0, 8, ( nCbWY − 1 − xL − yL / nCbR ) + 4 ) )

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(1032)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

Experimental results

The proposed method has been implemented on top of the VTM7.0 with default setting and tested using JVET SDR common test condition [1]. The experimental results of the proposed method reportedly show 0.01%, -0.02% BD-rate changes with no encoding and decoding run-time changes compared to VTM7.0 as anchor in RA, LB configurations.

|  | Over VTM 7.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.02% | 100% | 101% |  |  |  |  |  |
| Class A2 | -0.02% | -0.05% | 0.01% | 100% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.06% | 0.05% | 100% | 100% | -0.01% | -0.12% | 0.12% | 96% | 99% |
| Class C | 0.03% | 0.00% | 0.07% | 100% | 100% | -0.04% | -0.17% | 0.01% | 97% | 93% |
| Class E |  |  |  |  |  | -0.01% | 0.17% | -0.57% | 96% | 96% |
| Overall | 0.01% | 0.01% | 0.03% | 100% | 100% | -0.02% | -0.06% | -0.09% | 96% | 96% |
| Class D | -0.01% | -0.01% | -0.05% | 100% | 100% | -0.03% | 0.55% | -0.84% | 97% | 96% |
| Class F | -0.02% | 0.01% | -0.02% | 100% | 101% | 0.03% | -0.46% | 0.27% | 96% | 96% |

Conclusion

In this contribution, in order to align the chroma weight values for two direction and to use the equal weight value for samples on edge, weight values of the chroma-component are derived with shifted-sub-sampling method. The experimental results reportedly show negligible changes in BD-rate and encoding/decoding run-time. It is recommended to consider this method in the next VTM.

Reference

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- B. Bross, et al., “Versatile Video Coding (Draft 7)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P2001(JVET-P2001-vE), 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).