| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0338-r1 |
| --- | --- |

| Title: | CE4-related: Harmonized conditions for CIIP and GEO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the previous meeting, GEO mode was proposed and common base code for CE was provided. In the common base, the several conditions to apply the GEO mode were introduced and additional block size restriction was proposed in CE4-2.1. These constraints contributes to the number of the parsing conditions for CIIP and GEO becomes larger. In this contribution, simplified parsing conditions are proposed by aligning with conditions to the regular merge mode, CIIP mode and GEO mode. The experimental results reportedly show 0.02%, 0.06% BD-rate changes with no encoding and decoding run-time changes compared to VTM7.0 with CE4-2.1 as anchor in RA, LB configurations. According to the adoption of the 64x8 and 8x64 block size reduction, additional restriction is tested.

Proposed method

In the common base of the GEO [1], block size restriction was applied. From this restriction, GEO mode is disabled when block width or height is less than 8. Also, in the CE4-2.1 [2], additional block size restriction of GEO was proposed. From the CE4-2.1, GEO mode is disabled when the block width or height is greater than or equal to 128. Also, GEO mode is disallowed when the block size is 64x8 or 8x64.

The GEO mode is decided when both regular merge mode and CIIP are disabled. But, different constrained block size on each mode makes the parsing condition complicated. In this proposal, the block size constraint for CIIP and GEO on parsing conditions are unified. The below table shows the modified conditions. The modifications of the CE4-2.1 and proposed method are shown as green and yellow color highlight, respectively.

Table 1 Current conditions and the proposed conditions.

|  | Current | Proposed Method 1 | Proposed Method 2 |
| --- | --- | --- | --- |
| CIIP | ( cbWidth * cbHeight ) >= 64 cbWidth < 128 && cbHeight < 128 | cbWidth >= 8 && cbHeight >= 8 cbWidth < 128 && cbHeight < 128 | cbWidth >= 8 && cbHeight >= 8 cbWidth + cbHeight != 72 cbWidth < 128 && cbHeight < 128 |
| GEO | cbWidth >= 8 && cbHeight >= 8 | cbWidth >= 8 && cbHeight >= 8 cbWidth < 128 && cbHeight < 128 | cbWidth >= 8 && cbHeight >= 8 cbWidth + cbHeight != 72 cbWidth < 128 && cbHeight < 128 |

In the proposed methods, 4xN, Nx4 (N>8) blocks are additionally restricted for CIIP. Also, 64x 8 and 8x64 blocks are restricted for CIIP in the proposed method 2. Since the conditions of the CIIP are modified, regular merge flag is inferred to be equal to 1 when block width or height is smaller than 8 or larger than 64. Table 2 shows the changes for each block. The modifications of the CE4-2.1 and proposed method are shown as green and yellow color highlight, respectively.

Table 2 Changes for each block size.

| Block Size |  | CIIP | GEO |
| --- | --- | --- | --- |
| width | Height |  |  |
| 4 | 8 | x | x |
| 8 | 4 | x | x |
| 8 | 8 | o | o |
| 4 | 16 | o -> x | x |
| 16 | 4 | o -> x | x |
| … | … | … | … |
| 16 | 16 | o | o |
| … | … | … | … |
| 4 | 32 | o -> x | o |
| 32 | 4 | o -> x | o |
| … | … | … | … |
| 64 | 128 | x | o -> x |
| 128 | 64 | x | o -> x |
| 128 | 128 | x | o -> x |

From the common base spec text [1], the newly added texts are highlighted and removed texts are marked as stricken-red-font text. In the proposed methods, in the worst case, one multiplication is removed and 8 “AND” and 9 “COMPARISON” operations are reduced to 3 “AND” and 4 “COMPARISON” operation, respectively.

| The parsing condition of the “regular_merge_flag” | if( ( cbWidth * cbHeight ) >= 64 ( cbWidth >= 8 && cbHeight >= 8 && cbWidth < 128 && cbHeight < 128 && cbWidth + cbHeight != 72) && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128) ¦ ¦ (sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && cbWidth>=8 && cbHeight>=8 && cbWidth + cbHeight != 72 && slice_type = = B ) ) ) |
| --- | --- |
| The parsing condition of the “ciip_flag” | if( sps_ciip_enabled_flag && sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth >= 8 && cbHeight >= 8 && cbWidth + cbHeight != 72 && cbWidth < 128 && cbHeight < 128 ) |

Experimental results

The proposed method has been implemented on top of the CE4-2.1. The experimental results reportedly show 0.02%, 0.06% BD-rate changes with no encoding and decoding run-time changes compared to CE4-2.1 as anchor in RA(LB) configurations. Also, The proposed method has been implemented on top of the CE4-2.1 and JVET-Q0309, which disallow the 64x8 and 8x64 for GEO. The experimental results reportedly show 0.0x%, 0.0x% BD-rate changes with no encoding and decoding run-time changes compared to CE4-2.1 + JVET-Q0309 as anchor in RA(LB) configurations.

|  |  | Over VTM7.0 with CE4-2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access Main10 |  |  |  |  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.00% | 100% | 99% |  |  |  |  |  |
| Class A2 | 0.03% | 0.04% | 0.02% | 100% | 99% |  |  |  |  |  |
| Class B | 0.00% | -0.04% | 0.04% | 100% | 98% | 0.04% | -0.19% | 0.02% | 99% | 99% |
| Class C | 0.07% | 0.02% | 0.00% | 100% | 99% | 0.08% | -0.09% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  | 0.08% | 0.40% | 0.21% | 100% | 102% |
| Overall |  | 0.02% |  | -0.01% |  | 0.02% |  | 100% |  | 99% |  | 0.06% |  | -0.01% |  | 0.06% |  | 100% |  | 100% |  |
| Class D |  | 0.03% |  | -0.01% |  | 0.14% |  | 99% |  | 99% |  | 0.03% |  | 0.71% |  | 0.06% |  | 99% |  | 96% |  |
| Class F |  | 0.02% |  | 0.00% |  | -0.02% |  | 100% |  | 99% |  | 0.03% |  | -0.60% |  | -0.54% |  | 100% |  | 98% |  |

|  |  | Over VTM7.0 with CE4-2.1 + JVET-Q0309 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access Main10 |  |  |  |  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.05% | 0.11% | 100% | 100% |  |  |  |  |  |
| Class C | 0.07% | 0.03% | 0.01% | 99% | 100% | 0.05% | 0.08% | 0.15% | 99% | 100% |
| Class E |  |  |  |  |  | -0.04% | 0.04% | -0.11% | 99% | 99% |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D |  | 0.03% |  | 0.05% |  | 0.11% |  | 100% |  | 101% |  | 0.09% |  | -0.26% |  | -0.37% |  | 99% |  | 100% |  |
| Class F |  | 0.07% |  | 0.02% |  | 0.06% |  | 100% |  | 101% |  |  |  |  |  |  |  |  |  |  |  |

Proposed text changes

In the common base spec text [1], the newly added texts are highlighted and removed texts are marked as stricken-red-font text.

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) { |  |
| if( MaxNumIbcMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( ( cbWidth * cbHeight ) >= 64 ( cbWidth >= 8 && cbHeight >= 8 && cbWidth + cbHeight != 72 && cbWidth < 128 && cbHeight < 128 ) && ( (sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth < 128 && cbHeight < 128 ) ¦ ¦ ( sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && cbWidth>=8 && cbHeight >-8 && cbWidth + cbHeight != 72 && slice_type = = B ) ) ) |  |
| regular_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( regular_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( sps_mmvd_enabled_flag ) |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_merge_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| mmvd_cand_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_ciip_enabled_flag && sps_geo_enabled_flag && MaxNumGeoMergeCand > 1 && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth >= 8 && cbHeight >= 8 && && cbWidth + cbHeight != 72 && cbWidth < 128 && cbHeight < 128 ) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumGeoMergeCand > 1 ) { |  |
| merge_geo_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

ciip_flag[ x0 ][ y0 ] specifies whether the combined inter-picture merge and intra-picture prediction is applied for the current coding unit. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When ciip_flag[ x0 ][ y0 ] is not present, it is inferred as follows:

- If all the following conditions are true, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- sps_ciip_enabled_flag is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- cu_skip_flag[ x0 ][ y0 ] is equal to 0.

- cbWidth is less than 128.

- cbHeight is less than 128.

- cbWidth * cbHeight is greater than or equal to 64.

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbWidth + cbHeight is not 72

- Otherwise, ciip_flag[ x0 ][ y0 ] is inferred to be equal to 0.

When ciip_flag[ x0 ][ y0 ] is equal to 1, the variable IntraPredModeY[ x ][ y ] with x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1 is set to be equal to INTRA_PLANAR.

The variable MergeGeoFlag[ x0 ][ y0 ], which specifies whether triangular shape based motion compensation is used to generate the prediction samples of the current coding unit, when decoding a B slice, is derived as follows:

- If all the following conditions are true, MergeGeoFlag[ x0 ][ y0 ] is set equal to 1:

- sps_geo_enabled_flag is equal to 1.

- slice_type is equal to B.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- MaxNumGeoMergeCand is greater than or equal to 2.

- cbWidth is less than 128.

- cbHeight is less than 128.

- cbWidth * cbHeight is greater than or equal to 64.

- cbWidth is greater than or equal to 8.

- cbHeight is greater than or equal to 8.

- cbWidth + cbHeight is not 72

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.o

- ciip_flag[ x0 ][ y0 ] is equal to 0.

- Otherwise, MergeGeoFlag[ x0 ][ y0 ] is set equal to 0.

- 8.5.6 Decoding process for inter blocks

- 8.5.6.1 General

…….

When ciip_flag[ xCb ][ yCb ] is equal to 1, the array predSamples of prediction samples is modified as follows:

- If cIdx is equal to 0, the following applies:

- The general intra sample prediction process as specified in clause 8.4.5.2.5 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the intra prediction mode predModeIntra set equal to INTRA_PLANAR, the transform block width nTbW and height nTbH set equal to cbWidth and cbHeight, the coding block width nCbW and height nCbH set equal to cbWidth and cbHeight, and the variable cIdx as inputs, and the output is assigned to the (cbWidth)x(cbHeight) array predSamplesIntraL.

- The weighted sample prediction process for combined merge and intra prediction as specified in clause 8.5.6.7 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the coding block width cbWidth, the coding block height cbHeight, the sample arrays predSamplesInter and predSamplesIntra set equal to predSamples and predSamplesIntraL, respectively, and the colour component index cIdx as inputs, and the output is assigend to the (cbWidth)x(cbHeight) array predSamples.

- Otherwise, if cIdx is equal to 1 and cbWidth / SubWidthC is greater than or equal to 4, the following applies:

- The general intra sample prediction process as specified in clause 8.4.5.2.5 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb / SubWidthC , yCb / SubHeightC ), the intra prediction mode predModeIntra set equal to INTRA_PLANAR, the transform block width nTbW and height nTbH set equal to cbWidth / SubWidthC  and cbHeight / SubHeightC, the coding block width nCbW and height nCbH set equal to cbWidth / SubWidthC  and cbHeight / SubHeightC, and the variable cIdx as inputs, and the output is assigned to the (cbWidth / SubWidthC )x(cbHeight / SubHeightC) array predSamplesIntraCb.

- The weighted sample prediction process for combined merge and intra prediction as specified in clause 8.5.6.7 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the coding block width cbWidth / SubWidthC , the coding block height cbHeight / SubHeightC, the sample arrays predSamplesInter and predSamplesIntra set equal to predSamplesCb and predSamplesIntraCb, respectively, and the colour component index cIdx as inputs, and the output is assigend to the (cbWidth / SubWidthC )x(cbHeight / SubHeightC) array predSamples.

- Otherwise, if cIdx is equal to 2 and cbWidth / SubWidthC is greater than or equal to 4, the following applies:

- The general intra sample prediction process as specified in clause 8.4.5.2.5 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb / SubWidthC , yCb / SubHeightC ), the intra prediction mode predModeIntra set equal to INTRA_PLANAR, the transform block width nTbW and height nTbH set equal to cbWidth / SubWidthC  and cbHeight / SubHeightC, the coding block width nCbW and height nCbH set equal to cbWidth / SubWidthC  and cbHeight / SubHeightC, and the variable cIdx as inputs, and the output is assigned to the (cbWidth / SubWidthC )x(cbHeight / SubHeightC) array predSamplesIntraCr.

- The weighted sample prediction process for combined merge and intra prediction as specified in clause 8.5.6.7 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xCb, yCb ), the coding block width cbWidth / SubWidthC , the coding block height cbHeight / SubHeightC, the sample arrays predSamplesInter and predSamplesIntra set equal to predSamplesCr and predSamplesIntraCr, respectively, and the colour component index cIdx as inputs, and the output is assigend to the (cbWidth / SubWidthC )x(cbHeight / SubHeightC) array predSamples.

Conclusion

In this contribution, harmonized conditions for CIIP and GEO mode are proposed. By using the proposed restrictions, parsing condition for regular merge, CIIP and GEO flag are simplified. The experimental results reportedly show negligible changes in BD-rate and encoding/decoding run-time. It is recommended to consider this method in the next VTM.

Reference

- Han Gao, et al., “CE4 Common Base: Geometric inter prediction”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q0079, 17th Meeting: Brussels, BE, 7–17 January 2020.

- Z. Deng, et al., “CE4-2: Block size restriction of GEO”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q0077, 17th Meeting: Brussels, BE, 7–17 January 2020.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).