| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 Jan. 2020 | Document: JVET-Q0344-v3 |
| --- | --- |

| Title: | AHG6/AHG9: Signalling wrap-around for subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ya-Hsuan Lee, Jian-Liang Lin, Ying-Jui Chen, Chi-Cheng Ju (MediaTek) No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 { louise.lee, jl.lin, yr.chen, cc.ju}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to signal wrap-around motion compensation syntax for each subpicture. It is possible that a subpicture is a 360 projected picture (e.g. ERP) where the content on the vertical or horizontal boundaries is continuous. By enabling the wrap-around motion compensation for the subpicture, the objective and subjective quality may be improved.

Method 2 was added in version 3.

Introduction

In the 12th JVET meeting in Macao, the horizontal wrap-around motion compensation [1] was adopted. The wrap-around motion compensation can effectively reduce the seam artifacts in reconstructed viewports of 360 videos that encompass the left and right boundaries. On the other hand, subpicture support was also discussed and adopted in the recent meetings. One of the main use cases of the subpictures is the viewport-dependent 360 video as shown in Figure 1. Since a subpicture may be a 360 projected picture (e.g. ERP), the wrap-around motion compensation can be applied to improve the objective and subjective quality of the subpicture if the content on the vertical or horizontal boundaries is continuous.

Figure 1. An example of a viewport-dependent 360 picture including two subpictures with the ERP format

Proposed syntax and semantics

Method 1: signal in each subpicture

For each subpicture, a flag, subpic_ref_wraparound_enabled_flag[ i ], is added to indicate whether wrap-around motion compensation is applied for the i-th subpicture when subpic_treated_as_pic_flag[ i ] is equal to 1. If it is enabled, subpic_ref_wraparound_horizontal_flag[ i ] and subpic_ref_wrapaournd_offset_minus1[ i ] should be signaled to specify the direction and offset of the wrap-around motion compensation for the i-th subpicture, respectively.

The proposed syntax elements are shown highlighted compared to VVC Draft 7 [2].

Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| if( subpic_treated_as_pic_flag[ i ] ) { |  |
| subpic_ref_wraparound_enabled_flag[ i ] | u(1) |
| if( subpic_ref_wraparound_enabled_flag[ i ] ) { |  |
| subpic_ref_wraparound_horizontal_flag[ i ] | u(1) |
| subpic_ref_wraparound_offset_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |

7.4.3.3 Sequence parameter set RBSP semantics

…

subpic_ref_wraparound_enabled_flag[ i ] equal to 1 specifies that wrap-around motion compensation is applied in inter prediction for the i-th subpicture of each coded picture in the CLVS. subpic_ref_wraparound_enabled_flag[ i ] equal to 0 specifies that wrap-around motion compensation is not applied for the i-th subpicture of each coded picture in the CLVS.

subpic_ref_wraparound_horizontal_flag[ i ] equal to 1 specifies that wrap-around motion compensation is horizontally applied for the i-th subpicture of each coded picture in the CLVS. subpic_ref_wraparound_horizaontal_flag[ i ] equal to 0 specifies that wrap-around motion compensation is vertically applied for the i-th subpicture of each coded picture in the CLVS.

When the value of ( CtbSizeY / MinCbSizeY + 1) is greater than ( ( subpic_width_minus1[ i ] + 1 ) * CtbSizeY / MinCbSizeY − 1 ), the value of subpic_ref_wraparound_horizontal_flag[ i ] shall not be equal to 1.

When the value of ( CtbSizeY / MinCbSizeY + 1) is greater than ( ( subpic_height_minus1[ i ] + 1 ) * CtbSizeY / MinCbSizeY − 1 ), the value of subpic_ref_wraparound_horizontal_flag[ i ] shall not be equal to 0.

subpic_ref_wraparound_offset_minus1[ i ] plus 1 specifies the offset used for computing the wrap-around position in units of MinCbSizeY luma samples for the i-th subpicture of each coded picture in the CLVS. When subpic_ref_wraparound_horizontal_flag[ i ] is equal to 1, the value of subpic_ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( ( subpic_width_minus1[ i ] + 1 ) * CtbSizeY / MinCbSizeY ) − 1, inclusive. When subpic_ref_wraparound_horizontal_flag[ i ] is equal to 0, the value of subpic_ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( ( subpic_height_minus1[ i ] + 1 ) * CtbSizeY / MinCbSizeY ) − 1, inclusive.

…

        - Luma sample bilinear interpolation process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY, SubPicRightBoundaryPos + 1, ( xIntL + i ) ) : xIntL + i )		(642)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && !subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY, SubPicBotBoundaryPos + 1, ( yIntL + i ) ) : yIntL + i )		(643)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(644)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(645)

…

8.5.6.3.2  Luma sample interpolation filtering process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY, SubPicRightBoundaryPos + 1, xIntL + i − 3 ) : xIntL + i − 3 )		(955)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && !subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY, SubPicBotBoundaryPos + 1, yIntL + i − 3 ) : yIntL + i − 3 )		(956)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(957) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(958)

…

8.5.6.3.3  Luma integer sample fetching process

…

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY, SubPicRightBoundaryPos + 1, xIntL ) : xIntL )		(966)

yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && !subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY, SubPicBotBoundaryPos + 1, yIntL ) : yIntL )		(967)

–	Otherwise, the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(968)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(969)

…

8.5.6.3.4  Chroma sample interpolation process

…

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY ) / SubWidthC, SubPicRightBoundaryPos / SubWidthC + 1, xIntC + i − 1 ) : xIntC + i - 1 xIntL + i )	(971)

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, subpic_ref_wraparound_enabled_flag[ SubPicIdx ] && !subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( ( subpic_ref_wraparound_offset_minus1[ SubPicIdx ] + 1 ) * MinCbSizeY ) / SubHeightC, SubPicBotBoundaryPos / SubHeightC + 1, yIntC + i − 1 ) : yIntC + i - 1 yIntL + i )		(972)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(973)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(974)

…

Method2: signal in subpictures with constraints

In another method, wrap-around motion compensation for a subpicture is enabled only when the width of the subpicture is equal to the width of the picture, or the height of the subpicture is equal to the height of the picture. A syntax element, subpic_ref_wraparound_horizontal_flag[ i ] is added to indicate the direction for wrap around.

The proposed syntax elements are shown highlighted compared to VVC Draft 7 [2].

Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| if( subpic_treated_as_pic_flag[ i ] && sps_ref_wraparound_enabled_flag ) { |  |
| subpic_ref_wraparound_enabled_flag[ i ] | u(1) |
| if( subpic_ref_wraparound_enabled_flag[ i ] ) |  |
| subpic_ref_wraparound_horizontal_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |

7.4.3.3 Sequence parameter set RBSP semantics

…

subpic_ref_wraparound_enabled_flag[ i ] equal to 1 specifies that wrap-around motion compensation is applied in inter prediction to the i-th subpicture. subpic_ref_wraparound_enabled_flag[ i ] equal to 0 specifies that wrap-around motion compensation is not applied to the i-th subpicture. When not present, the value of subpic_ref_wraparound_enabled_flag[ i ] shall be equal to 0.

subpic_ref_wraparound_horizontal_flag[ i ] equal to 1 specifies that wrap-around motion compensation is horizontally applied to the i-th subpicture. subpic_ref_wraparound_horizaontal_flag[ i ] equal to 0 specifies that wrap-around motion compensation is vertically applied to the i-th subpicture.

It is a requirement of bitstream conformance that subpic_ref_wraparound_horizontal_flag[ i ] shall not be equal to 1 when subpic_ref_wraparound_enabled_flag[ i ] is equal to 1 and the value of subpic_width_minus1[ i ] is not equal to PicWidthInCtbsY – 1.

It is a requirement of bitstream conformance that subpic_ref_wraparound_horizontal_flag[ i ] shall not be equal to 0 when subpic_ref_wraparound_enabled_flag[ i ] is equal to 1 and the value of subpic_height_minus1[ i ] is not equal to PicHeightInCtbsY – 1.

…

        - Luma sample bilinear interpolation process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

–	If subpic_ref_wraparound_enabled_flag is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, SubPicRightBoundaryPos + 1, ( xIntL + i ) ) : xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, !subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, SubPicBotBoundaryPos + 1, ( yIntL + i ) ) : yIntL + i )

–	Otherwise (subpic_ref_wraparound_enabled_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i )	(642)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )	(643)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(644)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(645)

…

8.5.6.3.2  Luma sample interpolation filtering process

…

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

–	If subpic_ref_wraparound_enabled_flag is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( sps_ref_wraparound_offset_minus1+ 1 ) * MinCbSizeY, SubPicRightBoundaryPos + 1, xIntL + i − 3 ) : xIntL + i − 3 )

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, !subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, SubPicBotBoundaryPos + 1, yIntL + i − 3 ) : yIntL + i − 3 )

–	Otherwise (subpic_ref_wraparound_enabled_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i − 3 )	(955)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 )	(956)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(957) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(958)

…

8.5.6.3.3  Luma integer sample fetching process

…

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

–	If subpic_ref_wraparound_enabled_flag is equal to 1, the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, SubPicRightBoundaryPos + 1, xIntL ) : xIntL )

yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, 
!subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( subpic_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, SubPicBotBoundaryPos + 1, yIntL ) : yIntL )

–	Otherwise (subpic_ref_wraparound_enabled_flag[ SubPicIdx ] is equal to 0), the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL )	(966)

yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL )	(967)

–	Otherwise, the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(968)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(969)

…

8.5.6.3.4  Chroma sample interpolation process

…

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

–	If subpic_ref_wraparound_enabled_flag is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( xOffset, SubPicRightBoundaryPos / SubWidthC + 1, xIntC + i − 1 ) : xIntC + i - 1 xIntL + i )

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, !subpic_ref_wraparound_horizontal_flag[ SubPicIdx ] ? ClipH( ( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubHeightC, SubPicBotBoundaryPos / SubHeightC + 1, yIntC + i − 1 ) : yIntC + i - 1 yIntL + i )

–	Otherwise (subpic_ref_wraparound_enabled_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(973)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(974)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(973)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(974)

…

- Conclusion

In this contribution, it is proposed to signal syntax for enabling wrap-around motion compensation in each subpicture. With this functionality, a subpicture with continuous boundary content may achieve better coding efficiency.

References

- P. Hanhart, Y. He, Y. Ye, “CE13: PERP with horizontal geometry padding of reference pictures (Test 3.3)”, Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L0231, Oct. 2018, Macau, CN.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, Oct. 2019, Geneva, CH.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).